3GPP TSG-RAN WG2 Meeting #131bis	R2-2507343

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on RLC open issues

Document for:	Discussion, Decision

Introduction

This paper presents our views on the open issues related to RLC.

Discussion

  - RLC-V01

In Rel-19, when t-RxDiscard expires, the AM RLC entity stops waiting for the obsolete RLC SDUs, and these RLC SDUs will be positively acknowledged in the STATUS report. Some companies suggest the receiving RLC entity should indicate the SN of these obsolete RLC SDUs to the receiving PDCP entity to prevent unnecessary delays in packet delivery due to t-Reordering.

While we understand the motivation behind the proposals, we see it as an optimization and have concerns about the feasibility of the receiving PDCP entity identifying a valid PDCP SN value correctly. The receiving side does not have insight into how the PDCP SN and RLC SN are related, especially in the scenarios outlined below.

- The transmitting RLC entity may need to assign an RLC SN to a PDCP control PDU that lacks a PDCP SN. As a result, a packet with a PDCP SN of X may be assigned an RLC SN of Y, where X differs from Y.

- When the PDCP SDU is discarded, a PDCP SN gap may be created, while the RLC SN gap may not appear correspondingly. As a result, a packet with a PDCP SN of X may be assigned an RLC SN of Y, where X differs from Y.

In TS 38.323 Clause 5.4 If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE 2: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard. In TS 38.322 Clause 5.4 When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

- If a PDCP entity is associated with multiple RLC entities, it raises concerns about how the receiving side acknowledges the relationship between the PDCP SN and the RLC SN when packets are transmitted through different RLC entities.

Therefore, we prefer to maintain the specification as it is. Specifically, the receiving RLC entity should not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

- [RLC-V01] The receiving side cannot determine how the PDCP SN and RLC SN are associated, particularly when a PDCP control PDU exists, a PDCP SN gap occurs, or PDCP SDUs are transmitted through multiple RLCs.

- [RLC-V01] The receiving RLC entity does not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires (No spec change).

  - RLC-X01

During the post-meeting discussion, some companies expressed their concerns about the removal of delay reporting and non-delay reporting data concepts from RLC.

In last meeting, we have agreed: - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed.

We do not see the necessity for RLC to be aware of non-delay reporting data and delay reporting data, for the following reasons,

- All SDUs in RLC originate from PDCP, so PDCP is aware of the status of all packets and can inform RLC which SDUs should be included in the data volume calculation relevant to specific thresholds.

- The handling of SDUs within the RLC layer should follow the same order as in the PDCP layer, as both layers typically share the same buffer in the development process.

- [RLC-X01] RAN2 confirms to remove the definition of delay reporting and non-delay reporting data from RLC (No spec change).

Conclusion

We have the following observations:

Observation 1	[RLC-V01] The receiving side cannot determine how the PDCP SN and RLC SN are associated, particularly when a PDCP control PDU exists, a PDCP SN gap occurs, or PDCP SDUs are transmitted through multiple RLCs.

We have the following proposals:

Proposal 1	[RLC-V01] The receiving RLC entity does not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires (No spec change).

Proposal 2	[RLC-X01] RAN2 confirms to remove the definition of delay reporting and non-delay reporting data from RLC (No spec change).

Reference

[1] RP-250107 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)