3GPP TSG-RAN WG2 #131bis	R2-2507373

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.3.2	Control plane

Title:		Overview of 6G Control Plane aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#109, the revised SID includes many aspects as RAN2 objectives [1]. In this contribution, we discuss overview of the Control Plane (CP) aspects and provide our views.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# 2. Discussion

2.1	Overview of control plane aspects

There are multiple aspects to be studied from CP perspective including:

- Initial access procedure

- RRC connection management

- Paging framework

- Network energy saving and UE power saving

All these are not independent, but tightly related each other. For example, the initial access procedure involves the transmission of the essential information and signals required for cell access, which is largely related to the network energy saving from the lessons in 5G (e.g. NES). Another essential aspect is the RRC connection management including the study on RRC states to be supported. The network energy saving and the UE power saving are very important aspects regardless of the RRC states. Then for RRC_IDLE state, an efficient paging framework is required from Day1.

Proposal 1: RAN2 study multiple CP aspects jointly to achieve system-wide optimization.

2.2	Initial access procedure

This is the most important aspect involving the transmission/reception of the essential information and signal for the cell access. L1 (physical layer) signaling like synchronization signal (SS), random access preamble in the 5G NR is the RAN1 scope, while it would be good to consider it in RAN2 from higher layer perspective, e.g. SSB, RACH procedure on top of the RAN1 progress.

In the NR, the SSB and the essential SI (MIB, SIB1) are transmitted periodically in PCell. The other SI can be transmitted by On-demand. By introducing the On-demand SI transmission, the signaling overhead and thus network energy saving can be saved. This could be reused for 6G basically. Furthermore, On-demand SIB1 transmission for Idle/Inactive in a capacity cell (i.e. its coverage is fully covered by other cell (Cell A)) is introduced for network energy saving purpose.

For the NR, there were some more aggressive proposals as the WI scope for NES, although those were not selected as the objectives. For example, On-demand SIB1 in PCell as stand-alone (e.g. coverage) cell. This is also interesting, but RAN2 should consider higher level aspects at first. For example, RAN2 study which information should be included in SIB1, or whether SIB1 design should be kept from NR.

Proposal 2: RAN2 study initial access procedure by using that in the NR as starting point.

There is one different aspect related to the initial access. In the NR, the network can provide SCell only cell without SIB1 transmission. Further, there is the concept of SSB-less SCell. These implies that the network can configure some cells (e.g. Group A) for cell camping and some other cells (e.g. Group B) for data transmission purpose only. For examples, a UE in RRC_IDLE can camp on a cell #1 of the Group A. The UE can perform the initial access procedure on the cell #1. While the UE in RRC_CONNECTED, the UE may use a cell #2 of the Group B as the SCell of the carrier aggregation. Also, it is up to network whether the UE should stay in this cell #1 or move to the cell #2 as the PCell, unless the cell #2 is SSB-less cell.

For 6G, it would be good to study this kind of cell (or carrier) type differentiation a bit more generic, as the fundamental design of a cell or a carrier is under study from scratch. For example, one type of cells may be mainly used for the Idle mobility purpose, while the other type of cells may be used only for the Connected purpose including the data transmission and the mobility. With this, the essential signal and/or information on each type of the cell or carrier can be optimized to fit (main) purpose of each type, respectively. However, there should be comparison of pros and cons, e.g. a drawback may be that many UEs may camp on the same cells, while the paging transmission can be limited to those cells.

Proposal 3: RAN2 study possibility of the cell/carrier type differentiation to optimize the cell/carrier design to fit its purpose(s).

2.3	RRC connection management

In the NR, three RRC states are defined, i.e. RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED state. The motivation of introducing RRC_INACTIVE state is to reduce access latency with reduced UE power consumption. However, in fact, as mentioned in [2] [3], RRC_INACTIVE state is not attractive and extensively used in the commercial network. The main reason is that the reduced access latency is limited compared to RRC_IDLE state. We can see that random access latency is the same for RRC_IDLE and RRC_INACTIVE. The main benefit in RRC_INACTIVE is that the data transmission can be started upon RRC resume completion. The power consumption of RRC_INACTIVE state is almost or even more compared to RRC_IDLE state.

Besides, during RRC_INACTIVE state, the UE may perform positioning and small data transmission (SDT). In this case, UE specific parameters need to be configured by RRC release message, such as configured grant configuration, dedicated PDCCH, PDSCH and PUSCH configuration, which is very similar to RRC_CONNECTED state. From the network perspective, keeping many RRC_INACTIVE UEs would result in storing too many information in the memory, which is not so easy in some deployment. And three state maintenance can also make the system more complex.

Based on the analysis above, RAN2 to study whether it is feasible to simplify RRC state. For example, only keep two RRC states (IDLE and CONNECTED), or re-design the RRC_IDLE.

Proposal 4: RAN2 to study RRC state(s) to be supported from Day1 (e.g. whether it is feasible to maintain only two RRC states: RRC_IDLE and CONNECTED without RRC_INACTIVE).

2.4	Paging framework

The paging framework is one of essential system design. Usually, the paging messages can be distributed by using the paging frame (PF) and paging occasion (PO). With sparse PO, the paging overhead can be reduced, while the paging latency and capacity tend to be smaller. There should be reasonable and flexible balances between the paging latency/capacity and the overhead.

On top of that, in Rel-19 NR NES, paging adaptation is introduced in time domain by extending the PF interval (i.e., N) and increasing the number of POs within a PF (i.e., Ns) to compensate for the reduced number of PFs. This allows the network to have longer consecutive sleep periods, thereby reducing power consumption. To ensure backward compatibility with legacy UEs, a dedicated paging configuration, which is signalled separately in the system information, is introduced for UEs supporting paging adaptation.

Unfortunately, Rel-19 paging adaptation can only be based on semi-static configuration and is not applicable to legacy UEs, which limits its effectiveness and reduces flexibility in updating paging parameters. In 6G, paging adaptation may be studied from Day1 without such constraint, and more dynamic approaches—such as L1-based mechanisms—can be considered to enable faster and more efficient paging adaptation by considering the paging load.

Proposal 5: RAN2 to study paging adaptation from 6G Day1, and more dynamic approaches can be considered to enable faster and more efficient paging adaptation.

There is an inherent association between SSBs and paging monitoring occasions. Specifically, each PDCCH monitoring occasion for paging (PMO) is associated with an SSB for beam determination. Furthermore, prior to performing paging monitoring, the UE shall acquire 2 or 3 SSBs to achieve finer synchronization. This implies that any change in the SSB periodicity or SSB pattern may impact the paging monitoring. For example, if the SSB periodicity is extended to 160ms, keeping a dense paging cycle with a periodicity smaller than the SSB periodicity may not be optimal. Therefore, to achieve a more energy-efficient system, 6G could study the joint design or adaptation of common channels and signals, such as coordination between SSB and paging. Specially, extending the SSB periodicity would lead to a corresponding extension of the paging cycle.

Basically the common channel design (e.g. SSB) is led by RAN1, so RAN2 should collaborate with RAN1 for system-wide optimization for the paging.

Proposal 6: RAN2 to study joint design or adaptation of common channels and signals (e.g., between SSB and paging) by collaborating with RAN1.

2.4 UE power saving

The power consumption performance directly determines the availability, user experience, and ecological scalability of communication services, and serves as a core pillar supporting the efficient operation of modern communication systems. Whether it is daily calls, mobile office work, HD video/streaming, or IoT data collection, excessive power consumption causing device out of power frequently may interrupt the service process, therefore UE power saving technology is promising to be deployed in every release.

In NR, multiple power saving technology is introduced from RAN2 perspective:

In Rel-15, discontinuous reception (DRX) is introduced to let UE enter sleep mode if there is no data transmission and BWP adaptation is used to dynamically switch the BWP considering data traffic.

In Rel-16, DCP is introduced to further indicate whether the UE should start C-DRX active timer or not. SCell dormancy is to reduce PDCCH monitoring when no data is transmitted in the SCell. And RRM measurement relaxation enables RRC_IDLE/INACTIVE UE to perform relaxed measurement. Furthermore, multiple UE preferences for power saving is included in UAI.

In Rel-17, to further reduce the power consumption of paging monitoring, PEI mechanism is introduced to indicate whether the UE should monitor paging or not. TRS can be configured during RRC_IDLE/INACTIVE for precise synchronization to make the synchronization duration longer. RLM/BFD relaxation is also introduced by UAI mechanism.

In Rel-19, LP-WUS mechanism is introduced by utilizing a separate UE device (which is a low power consumption device) to replace paging monitoring during RRC_IDLE/INACTIVE and PDCCH monitoring during RRC_CONNECTED for more power saving gain, meanwhile, further RRM measurement relaxation is enabled by using LP-SS measurement.

Looking at the NR UE power saving technologies, there are two types of power saving, i.e. one is efficient ways of balancing between the communication performance (e.g. data transmission, paging latency) and the power saving gain, the other is efficient ways of balancing between the mobility performance (e.g. cell reselection, beam management, handover) and the power saving gain. Both are very important for 6G from Day1.

When it comes to 6G, we believe that it is necessary to study some new power saving technologies considering that in 6G, more and more high-level performance and power consuming applications will be introduced. Generally speaking, the main power consumption is from UE active time for signalling/data monitoring and measurement, therefore, NR DRX-like mechanism (including I-DRX/C-DRX) can be as baseline to make UE stay in “sleep mode” as much as possible and try to further reduce measurement. Furthermore, wake-up function is also useful, e.g., Rel-19 LP-WUS like mechanism by using a separate device at the UE side can also be considered. For 6G, enhanced low power (LP) radio can be utilized for multiple power saving related features.

Note that all of these challenges should be applied to all RRC states basically. However, it would be better to focus on each RRC state separately at first, rather than reusing or limiting to common design among different RRC states unnecessarily, although it would be good to see a certain common design among RRC states in the end.

Proposal 7: RAN2 study the UE power saving for 6G Day1 to achieve good trade-off between the power saving gain and the communication performance (e.g. data transmission, paging latency) as well as mobility performance (e.g. cell reselection, beam management, handover).

# 3. Conclusion

In this contribution we discussed the overview of the CP aspects and made the following proposals.

Overview of control plane aspects

Proposal 1: RAN2 study multiple CP aspects jointly to achieve system-wide optimization.

Initial access procedure

Proposal 2: RAN2 study initial access procedure by using that in the NR as starting point.

Proposal 3: RAN2 study possibility of the cell/carrier type differentiation to optimize the cell/carrier design to fit its purpose(s).

RRC connection management

Proposal 4: RAN2 to study RRC state(s) to be supported from Day1 (e.g. whether it is feasible to maintain only two RRC states: RRC_IDLE and CONNECTED without RRC_INACTIVE).

Paging framework

Proposal 5: RAN2 to study paging adaptation from 6G Day1, and more dynamic approaches can be considered to enable faster and more efficient paging adaptation.

Proposal 6: RAN2 to study joint design or adaptation of common channels and signals (e.g., between SSB and paging) by collaborating with RAN1.

UE power saving

Proposal 7: RAN2 study the UE power saving for 6G Day1 to achieve good trade-off between the power saving gain and the communication performance (e.g. data transmission, paging latency) as well as mobility performance (e.g. cell reselection, beam management, handover).

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] 6GWS-250144, CMCC’s views on 6G RAN study CMCC,

[3] RP-252138, Consideration on UE management Huawei