3GPP TSG-RAN WG2 Meeting #131 bis	R2-2507376

Prague, Czech Republic, 13 - 17 October 2025

Agenda Item:	 8.12.3

Source:	 Huawei, HiSilicon

Title:	 RRC issues for MIMO

Document for:  Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, significant progress on asymmetric DL/UL mTRP study has been made in previous meetings. However, there is still one remaining issue on this topic in the post email discussion “[Post130][219][MIMO_Ph5] MAC open issues for MIMO”[1] as follows

- Other issue: Discuss whether to support Rel-19 sDCI mTRP 2TA with PL offset for Rel-18 LTM cell switch. If it is supported, how to indicate the up-to-date PL offset for LTM cell switch.

For the above issue, in the Rel-19 TEI meeting, it has been agreed that:

Agreements

- Discussion is postponed and should take place in MlMO first. In October during ASN.1 review cross-Wl discussion, we may discuss whether the feature can be extended to LTM.

Therefore, in this contribution, we present analysis on whether to support the combination of the above two features, and provide our study on the corresponding RRC impacts.

# 2	Discussion

## 2.1	[H400][Z408] Where to configure UE-initiated CSI reports

UE-initiated CSI reports are configured using CSI-ReportConfig, which is included in CSI-MeasConfig, which is in ServingCellConfig. CSI-ReportConfig is defined as:

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). The IE CSI-ReportConfig is also used to configure UE initiated CSI reporting. See TS 38.214 [19], clause 5.2.1.

As can be observed:

-	for periodic or semi-persistent reports on PUCCH, the report is to be configured on the cell on which it is transmitted;

-	for semi-persistent or aperiodic reports on PUSCH, the report is to be configured on the cell on which the DCI triggering the report is received;

-	for UE-initiated CSI reports, it is not specified on which cell to configure the report.

In TS 38.214 v19.1.0, the transmission is specified separately for the 3 events defined so far:

5.2.1.5.4.1a	UE Initiated CSI reporting for event 2

[...]

The UE sends the CSI report

-	on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or

-	on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

5.2.1.5.4.1b	UE Initiated CSI reporting for event 2

[...]

The UE sends the CSI report

-	on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or

-	on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

5.2.1.5.4.1c	UE Initiated CSI reporting for event 7

[...]

The UE sends the CSI report

-	on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or

-	on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

Accordingly:

-	for mode A reports, it is like for aperiodic or semi-persistent on PUSCH;

-	for mode B reports, it is like periodic or semi-persistent reports on PUCCH.

Therefore, we suggest updating the description of the CSI-ReportConfig IE as follows:

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure either:

-	a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included; or

-	a UE-initiated report sent on PUSCH on the cell in which the CSI-ReportConfig is included; or

-	a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI); or

- 	a UE-initiated report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

Proposal 1: Update the description of the CSI-ReportConfig IE according to the TP in 2.1.

The UE-initiated CSI reports are configured with the following structure:

CSI-ReportUE-IBR-r19 ::=               SEQUENCE {

eventTypeUE-IBR-r19                    CHOICE {

-- [contents omitted]

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                NULL,

modeB-r19                                SEQUENCE {

pusch-ResourceOfModeB-r19                SEQUENCE {

configuredGrantConfigIndex-r19           ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                            BWP-Id,

servCellIndex-r19                        ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED {symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512}

}

},

-- [contents omitted]

}

As observed in the RIL Z408, the field highlighted in yellow seems to have no use.

Proposal 2: Remove servCellIndex-r19 in modeB-r19 in CSI-ReportUE-IBR-r19.

## 2.2	[H401] Resources for transmission

There have been discussions on what the UE behaviour should be if a UE-initiated report and in the active BWP:

-	there are no resources to transmit the UEIRI; or

-	there are no resources to transmit the UE-initiated report.

While this is not an RRC discussion, this raises attention to TS 38.331 v19.0.0, according to which the PUCCH resources to transmit the UEIRI are in a single BWP, and the configured grant to transmit the UE-initiated report (in the so called "mode B") is in a single BWP. This is in contrast to Rel-15 reporting on PUCCH, for which resources can be allocated on any BWP.

If resources can only be configured on a single BWP for UEIRI and mode-B reporting, the scenario where the UE is supposed to initiate a CSI report while there are no resources in the active BWP to transmit the UEIRI or the mode-B report is rather likely to occur, so the expected UE behaviour in such a scenario should be specified.

Proposal 3: Ask RAN1:

-	whether RAN1 will capture the UE behaviour for the case where a report is triggered while there are no resources for UEIRI transmission or mode-B transmission in the active BWP;

-	whether there is a particular reason why the resources for UEIRI transmission and for mode-B transmission on PUSCH are in a single BWP (unlike Rel-15 CSI reporting on PUCCH), which could make the above scenario difficult to avoid.

## 2.3	[H303] Combination with LTM

RAN2 agreement in RAN2#130

- Regarding Rel-19 sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement.

In the post email discussion “[Post130][219][MIMO_Ph5] MAC open issues for MIMO”[1] , it has already been agreed that, sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no specification impact. However, according to the previous RAN2 agreements in RAN2#130, when PL offset is not configured in joint/UL TCI state for asymmetric DL sTRP/UL mTRP, the UE should be configured with SSB-MTC-additionalPCI, which means DL-UL TRP and UL-only TRP belong to different cells, as shown in Figure 1.

Figure 1. R19 sDCI mTRP 2TA (with/without a PL offset)

Such inter-cell mTRP case is just one case for asymmetric DL sTRP/UL mTRP, the other common case is that UE is configured with a PL offset and both DL-UL TRP and UL-only TRP belong to the same cell. In such intra-cell mTRP case, the UL-only TRP does not send SSB and the PL offset objectively exists. In order to ensure a reasonable power of first PUSCH transmission in this scenario, it is necessary to provide the UE with a PL offset.

In the LTM cell switch procedure, the UE LTM cell switch command MAC CE indicates the candidate TCI state (one joint TCI state, or a pair of on DL TCI state and one UL TCI state) that the UE shall apply from then on. Candidate-TCI-State is defined as:

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Cond Joint

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                                                  OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                  Uplink-powerControlId-r17                                           OPTIONAL,   -- Cond Joint2

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

This means that the PL offset cannot be provided according to current signalling.

There could be two ways to allow this:

-	add the PL offset in Candidate-TCI-State and Candidate-UL-TCI-State

-	indicate the PL offset in the LTM cell switch MAC CE.

In principle, it may be beneficial to dynamically adjust the PL offset, but for inter-DU/CU cases, it may require additional network coordination. If the PL offset is stored in Candidate-TCI-State and set to such a value that the initial uplink transmission power is high enough, the initial uplink transmission can be received, even though the initial transmission power may be unnecessarily high.

Proposal 4: Confirm that, when the TA is already acquired, it is possible to perform an initial (RACH-less) uplink transmission to a cell configured with intra-cell asymmetric DL sTRP/UL mTRP using a pre-stored PL offset for the TCI state used.

Proposal 5: Leave it to the mobility enhancement session (whether) to specify the necessary signalling.

# 3	Conclusion

In this contribution, we have the following proposals:

Proposal 1: Update the description of the CSI-ReportConfig IE according to the TP in 2.1.

Proposal 2: Remove servCellIndex-r19 in modeB-r19 in CSI-ReportUE-IBR-r19.

Proposal 3: Ask RAN1:

-	whether RAN1 will capture the UE behaviour for the case where a report is triggered while there are no resources for UEIRI transmission or mode-B transmission in the active BWP;

-	whether there is a particular reason why the resources for UEIRI transmission and for mode-B transmission on PUSCH are in a single BWP (unlike Rel-15 CSI reporting on PUCCH), which could make the above scenario difficult to avoid.

Proposal 4: Confirm that, when the TA is already acquired, it is possible to perform an initial (RACH-less) uplink transmission to a cell configured with intra-cell asymmetric DL sTRP/UL mTRP using a pre-stored PL offset for the TCI state used.

Proposal 5: Leave it to the mobility enhancement session (whether) to specify the necessary signalling.

# 4	Reference

- [Post130][219][MIMO_Ph5] MAC open issues for MIMO