3GPP TSG-RAN WG2 #131bis	R2-2507400

# Prague, Czech Republic, 13th – 17th October 2025

Agenda Item:	7.0.2.22

Source:	Ericsson

Title:	Handling of inter-DU L2 reset for LTM

Document for:	Discussion

# 1	Introduction

In this contribution we address an LS received from RAN3 in R2-2506726 where it asked whether L2 reset is always performed in case of inter-DU scenarios.

# 2	Discussion

According to received LS in R2-2506726, RAN3 assumes that in case of inter-DU scenarios the L2 reset is always performed on both UE and network.

RAN3 has discussed the handling of L2 reset in the context of inter-DU LTM for Rel-18. The common understanding within RAN3 is that L2 reset is always performed when the UE switches between the DUs, i.e., inter-DU LTM

However, what the L2 reset was discussed in RAN2 during Rel-18, the understanding has been from the beginning that whether a L2 reset is performed on not is entirely controlled by the network, hence the standardization of the IDs which are used by the UE to determine whether a L2 reset is needed or not.

RAN2 has agreed that L2 reset for both inter-DU and intra-DU is network controlled.

On top of this, another motivation on why RAN2 understanding was that in inter-DU scenario the L2 reset may not always be needed is that network implementation may not require (or need) a L2 reset (e.g., in intra-vendor deployments). Therefore, there is no benefit in limiting such implementation as not performing L2 reset bring benefits in terms or latency.

Some network implementations (e.g., in case of intra-vendor deployments) may not require a L2 reset even in inter-DU scenarios.

According to this, would be good to clarify to RAN3 that RAN2 assumption is that whether L2 reset is performed for inter-DU scenario is according to network control. Therefore, it should not be assumed that in case of inter-DU scenarios the L2 reset is always performed. Thus, we propose:

RAN2 to reply to RAN3 that whether L2 reset is performed for inter-DU scenario is controlled by the network. Therefore, it should not be assumed that in case of inter-DU scenarios the L2 reset is always performed.

RAN2 to agree on the LS proposed in Annex

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has agreed that L2 reset for both inter-DU and intra-DU is network controlled.

Observation 2	Some network implementations (e.g., in case of intra-vendor deployments) may not require a L2 reset even in inter-DU scenarios.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to reply to RAN3 that whether L2 reset is performed for inter-DU scenario is controlled by the network. Therefore, it should not be assumed that in case of inter-DU scenarios the L2 reset is always performed.

Proposal 2	RAN2 to agree on the LS proposed in Annex

# Annex 1 – Reply LS to RAN3

3GPP TSG-RAN WG2 Meeting #131bis	R2-25xxxxx

Prague, Czech Republic, 13th – 17th August 2025

Title:	Reply LS on the handling of inter-DU L2 reset for LTM

Response to:	R2-2506726

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	Ericsson (to be RAN2)

To:	RAN3

Cc:

Contact Person:

Name:	Antonino Orsino

E-mail Address:	antonino.orsino@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN3 for the LS on the handling of inter-DU L2 reset for LTM. RAN2 has discussed the issue and would like to clarify that whether L2 reset is performed for inter-DU scenario is controlled by the network. Therefore, it should not be assumed that in case of inter-DU scenarios the L2 reset is always performed.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 kindly asks RAN3 to take the above information into account and update specifications, if needed.

3. Date of Next TSG-RAN2 Meetings:

3GPP TSG RAN WG2#132		17 - 21 	November 2025		Dallas, US

3GPP TSG-RAN WG2#133		9 - 13 		February 2026		Göteborg, Sweden