I 3GPP TSG-RAN WG2 Meeting #131bis	R2-2507425

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.3.3

Source:	Nokia

Title:	On AIML data transfer, interface protocols, framework

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses the fundamental components of AI/ML, including data transfer related to Objective 3e, and an extensible framework including lifecycle management (LCM) procedures related to Objective 8bi and data collection and data management related to Objective 8bii [cf. RP-251881].

To guide the study and discussions, it is important to clarify the technical meaning of certain terms related to the 6GR AI/ML air-interface discussions.

- “AI-native air-interface” or “AI-native radio” indicates the readiness of the specified solutions to support AI/ML air-interface use cases and functionalities at the UE-side and/or NW-side, including two-sided models. In this definition “AI-native” does not indicate e.g., the specified protocols are controlled/driven by AI/ML algorithms.

- “AI/ML framework” or “LCM framework” indicates primarily the set of management procedures for UE-side AI/ML use cases, including the required configuration, monitoring and data collection signaling procedures; the “AI/ML framework” shall also include the necessary management procedures for data collection for NW-side AL/ML use cases.

We discuss more details on requirements related to the “LCM framework” in Section 4 of our TDoc. For “AI-native air-interface”, we propose adopting the following key principles enabling AI-native air-interface, as part of the overall 6G AI/ML-Native System Framework:

- AI/ML should be considered a fundamental part of the 6G air-interface design from the first release, while ensuring high performance by enabling seamless fallback to non-AI/ML configurations.

- Establish RAN enablers with a comprehensive, future-proof design to minimize excessive specification enhancements in later releases, e.g., new use cases should be treated as modular plugins to the baseline, extending existing RRC functionalities, e.g., applicability reporting, rather than building new ones.

- Develop solutions with lean and scalable mechanisms for in-lab and in-field testing, validation, and monitoring.

Proposal 1: As a key principle, study the design of future-proof enablers for the modular addition of new use cases as they are introduced, extending existing RRC functionalities, e.g., applicability reporting.

# 2	AI/ML data collection and transfer design

This section discusses procedures related to the configuration of data collection and the transfer of that data, corresponding to Objectives 3e and 8b. We will discuss lessons learned from the 5G SI and WI on AI/ML for air interface and propose requirements to set the stage for the study.

The Release 19 5G Way

In 5G-A, ML use cases are primarily prediction-based: temporal, forecasting future events or states using historical data, such as Reference Signal Received Power (RSRP) or mobility events; and spatial, predicting the characteristics of reference signals (RS) based on the measurement of a subset of RS or a different set of RS. These predictions assist in decision-making (e.g., beam selection, scheduling or mobility decisions) but do not directly alter system behaviour or influence future predictions.

The nature of data collection in 5G-A is such that network-side (NW-side) data collection and UE-side data collection configure the UE with measurement resources, e.g., channel state information (CSI) reference signals (RS), and with a reporting configuration (for NW-side data collection) or a supressed reporting configuration (for UE-side data collection) defining the datatype to collect. As an example from the beam prediction use case, a data collection measurement could contain beam indices and beam L1-RSRPs. Broadly, this kind of data collection could be named “measurement report-based data collection”.

NW-side data collection was specified as an enhancement to MDT and gNB-triggered data collection wherein the UE logs measurement data in a buffer of at least 64KB. The UE can be configured to report that the buffer has reached a threshold and is required to report when the buffer is full. To retrieve the data from the buffer, the gNB issues UEInformationRequest and the UE responds with UEInformationResponse containing up to ~9KB of data (the maximum size of an RRC message). If more data is available in the buffer, the gNB needs to issue further UEInformationRequest messages until the buffer is empty. The NW need not wait for an indication that the buffer threshold has been reached or that the buffer has become full. Additionally, a new low priority SRB was created to transmit the UEInformationResponse messages.

Observation 1: The NW-side data collection solution from 5G-A has at least the following downsides: the size and volume of collected data is limited by CP capacity and the UE cannot opportunistically transmit measurements from its buffer as it must wait for a UEInformationRequest.

For UE-side data collection, the gNB can either directly configure the UE with RS for measurement or the gNB can provide the UE with data collection configuration candidates from which to select. If the UE indicates preferred data collection candidate configurations, the gNB can choose not to configure the UE or configure the UE with one or more measurement configurations. Because a data transfer mechanism was not designed for UE-side data collection, the reporting is supressed and left to proprietary means of transfer in Release 19, while SA2 is studying a standardized means for Release 20, but the measurement outputs, or ground truth, are the same.

Observation 2: The UE-side data collection solution from Release 19 5G-A has at least the following downsides: UE selection by the gNB is impossible since it does not know the vendor or model of UE and data transfer is not standardized and does not meet the requirements of controllability and visibility of data transfer.

Lastly, NW-side data collection and UE-side data collection are configured separately from one another. UE-side data collection is configured in CSI-ReportConfig, while NW-side data collection is configured in a new IE under CSI-MeasConfig, which replicates some fields from CSI-ReportConfig. The decision to separate the configurations may have been necessary as not to mix layer 1 CSI measurement reporting with layer 3 CSI measurement reporting for data collection. However, we find that it was less than ideal to define the measurement configurations for like data collection for NW-side and UE-side in different ways when the main difference was where the collected data is transmitted.

Observation 3: It was unideal to define the measurement configurations for like data collection for NW-side and UE-side in different ways when the main difference was where and how the collected data is transmitted.

The 6G Way

In the spirit of the key principles described in the previous section, we should endeavour to develop extensible solutions for data collection such that we do not need to redefine the measurement and reporting configurations for each type of data collection, including legacy, non-AI/ML, measurement reporting. It is reasonable to define different configuration IEs for different measurement types, however, depending on how the measurement framework is designed.

Taking 5G as an example, we think that the CSI-ReportConfig could have been enhanced to support NW-side data collection with logging by supressing measurement transmissions as in UE-side data collection, wherein the reporting type would instruct the UE what to do with the measurements, e.g., include them in the data collection buffer.

Therefore, we propose to strive to collocate measurement signal configuration, e.g., the configuration of CSI-RS resources, and the measurement quantity configuration, e.g., L1 RSRP and beam indices, among the different purposes of measurements, such as legacy, non-AI/ML, measurement reporting, AI/ML-based measurement reporting, performance reporting of AI/ML configurations, and NW-side and UE-side data collection. For example, a configuration used for NW-side monitoring of a beam prediction configuration could be one that occasionally reports the full set of beams while performing inference. The configuration to report the real L1 RSRP measurements for monitoring is similar enough to the configuration to report L1 RSRP measurements for use in NW-based beam selection that they should share the same configuration IEs.

Proposal 2: When designing the 6G measurement and reporting framework(s), the configuration of physical resources for measurements and of measurement types for non-AI/ML and AI/ML-based measurement reporting, AI/ML performance reporting, and NW-side and UE-side data collection should be common.

New type of data in 6G

In 5G-A use cases, the ground truth (or labels) for AI/ML training are the radio measurements, obtained from reference signals, which are compared to the predicted measurements of AI/ML model during training.

However, for 6G use cases such as AI receivers, the ground truth is obtained from the transmission itself, e.g., the transmitted sequences of bits are estimated by the AI-receiver and the ground truth is the actual transmitted bits. Hence, to train AI receivers, knowing what was transmitted is required so that it can be compared to reconstructed bits at the AI-receiver output. This is particularly important when the receiver is underperforming and cannot decode the signal but requires the transmitted sequence for finetuning. We refer to such known transmitted sequences as reference transmission (for example known transport block transmission).

Furthermore, the benefits and requirements for reference transmissions extend beyond AI receivers to other potential 6G AI/ML use cases that may require knowledge of transmitted content for training and optimization purposes. The 5G framework is not sufficient for such reference transmission.

Observation 4: 5G-A data collection framework is insufficient to support ground truth acquisition of new use cases, like AI receivers

# 3	AI/ML Extensible “LCM framework”

This section discusses requirements for extensible management procedures for UE-side ML (“LCM framework” in 5G-A) and its integration into the baseline 6G protocol stack. The goal is to start with requirements that ensure seamless configuration whether non-AI/ML-based or AI/ML-based, a robust fallback mechanism, and control on model transfer to the UE, corresponding to Objective 8b.

Release 19 5G Way

In 5G-A the only control mechanism for AI/ML-enabled configurations which was introduced is called applicability determination. The purpose of applicability determination is for the UE to evaluate whether it has an ML model to execute on an AI/ML-enabled configuration and that it has the resources to do so. The burden of this applicability determination rests solely on the UE, which evaluates its own conditions, the NW conditions and the configuration itself, and reports the result to the gNB. If the AI/ML-enabled configuration is applicable, then it can be applied.

Observation 5: 5G-A introduced applicability determination, wherein the UE evaluates whether it has the ability to execute on an AI/ML-enabled configuration based on its own conditions, NW conditions and the configuration itself, and reports the result to the gNB.

As an optimization, a second type of configuration consisting of “inference-related parameters”, or a subset of a full AI/ML-enabled configuration, was introduced. The procedure using inference-related parameters introduces an additional set of downlink and uplink messaging before a full AI/ML-enabled configuration is signalled to the UE. However, the optimization is minimal, particularly for the beam prediction use case for which this feature was designed, because the CSI-RS resources and most of the full configuration parameters are still required to be configured.

Observation 6: The optimization offered by providing the UE “inference-related parameters” for applicability determination is minimal because it requires an additional round of messages and because the CSI-RS resources and most of the full configuration parameters are still required to be configured.

The 6G Way

As stated in the key principles of Section 1, AI/ML should be considered a fundamental part of 6G design. Therefore, the configuration of an AI/ML-enabled feature should be as seamless as the configuration of any non-AI/ML-enabled feature in 5G. That is, we should study mechanisms to enable an AI/ML-enabled configuration to be immediately applicable, excluding the use of inference-related parameters. Additionally, a robust fallback mechanism must be designed such that switching between AI/ML and AI/ML and between AI/ML and non-AI/ML does not result in excess reconfiguration overhead. Lastly, we propose to design monitoring procedures used to aid the NW in reconfiguring the UE, informing the UE when performance is inadequate and triggering data collection for fine tuning.

Proposal 3: Study mechanisms to enable an AI/ML-enabled configuration to be immediately applicable, excluding the use of inference-related parameters.

Proposal 4: Study a fallback mechanism which minimizes reconfiguration overhead and fluid switching between AI/ML and AI/ML, and between AI/ML and non-AI/ML configurations.

Proposal 5: Study monitoring procedures used to aid the NW in reconfiguring the UE, informing the UE when performance is inadequate, and triggering data collection for offline training and fine tuning.

The testing and validation of the new 6GR AI/ML features (use cases) is another important aspect. In RAN4, for 5G-A Rel-19, the post-deployment testing and validation procedures have been already under discussion, even without considering use cases which make use of on-device online training/finetuning (continuous learning). In 6GR we expect the post-deployment testing and validation procedures to play a key role as part of the NW/UE mechanisms ensuring in-field stable and trusted performance of UE-side AI/ML features [R4-2513072]. These post-deployment testing and validation procedures imply at least enhancements to the RAN2 monitoring reporting configuration and procedures introduced in 5G-A Rel 19-20, in combination with the procedures for the different phases of the online training/finetuning, such as the exploration/tuning, discussed above.

Proposal 6: RAN2 studies should include management procedures for UE-side ML (configuration, monitoring, data collection) for in-field post-deployment validation procedures, designed in collaboration with RAN4, which would help ensure consistent performance in the network.

Lastly, we will discuss model or model related data transfer. There are two main aspects to consider when it comes to model transfer or the transfer of model-related data: which model or set of models is most useful to a UE for a given time period or scenario; and whether the performance gain due to model transfer significantly outweighs the cost of transferring a model. While it isn’t feasible for the 6gNB to determine whether a model will improve performance and by how much, the 6gNB will always make decisions about how to manage UL/DL radio resources. For example, by evaluating the amount of data in the UE’s DL and UL buffers and the size of the model, a determination can be made whether transferring a model or model-related data to the UE, or not, in a given scheduling period. To enable the NW with this type of control, RAN2 needs to study mechanisms which could be used by the NW to be aware (and/or have visibility) of model or model related data transfer.

Proposal 7: Study mechanisms for NW control of DL and UL model or model-related data transfer to a UE.

# 4	Conclusion

This document has made the following observations:

Observation 1: The NW-side data collection solution from 5G-A has at least the following downsides: the size and volume of collected data is limited by CP capacity and the UE cannot opportunistically transmit measurements from its buffer as it must wait for a UEInformationRequest.

Observation 2: The UE-side data collection solution from Release 19 5G-A has at least the following downsides: UE selection by the gNB is impossible since it does not know the vendor or model of UE and data transfer is not standardized and does not meet the requirements of controllability and visibility of data transfer.

Observation 3: It was unideal to define the measurement configurations for like data collection for NW-side and UE-side in different ways when the main difference was where and how the collected data is transmitted.

Observation 4: 5G-A data collection framework is insufficient to support ground truth acquisition of new use cases, like AI receivers

Observation 5: 5G-A introduced applicability determination, wherein the UE evaluates whether it has the ability to execute on an AI/ML-enabled configuration based on its own conditions, NW conditions and the configuration itself, and reports the result to the gNB.

Observation 6: The optimization offered by providing the UE “inference-related parameters” for applicability determination is minimal because it requires an additional round of messages and because the CSI-RS resources and most of the full configuration parameters are still required to be configured.

And proposed the following:

Proposal 1: As a key principle, study the design of future-proof enablers for the modular addition of new use cases as they are introduced, extending existing RRC functionalities, e.g., applicability reporting.

Proposal 2: When designing the 6G measurement and reporting framework(s), the configuration of physical resources for measurements and of measurement types for non-AI/ML and AI/ML-based measurement reporting, AI/ML performance reporting, and NW-side and UE-side data collection should be common.

Proposal 3: Study mechanisms to enable an AI/ML-enabled configuration to be immediately applicable, excluding the use of inference-related parameters.

Proposal 4: Study a fallback mechanism which minimizes reconfiguration overhead and fluid switching between AI/ML and AI/ML, and between AI/ML and non-AI/ML configurations.

Proposal 5: Study monitoring procedures used to aid the NW in reconfiguring the UE, informing the UE when performance is inadequate, and triggering data collection for offline training and fine tuning.

Proposal 6: RAN2 studies should include management procedures for UE-side ML (configuration, monitoring, data collection) for in-field post-deployment validation procedures, designed in collaboration with RAN4, which would help ensure consistent performance in the network.

Proposal 7: Study mechanisms for NW control of DL and UL model or model-related data transfer to a UE.