3GPP TSG RAN WG2 #131bis	R2-2507446

Prague, Czech Republic, October 13th-17th 2025

# Title:	[Draft] Reply LS on bundling period and SPS for ULBC

# Response to:	R2-2506754 / S4aA250258

# Release:	Release 20

# Work Item:	FS_ULBC

Source:	RAN WG2

To:	SA WG4

Cc:	RAN WG1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

RAN2 would like to thank SA4 for the LS asking question on whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN.

RAN2 discussed the potential issues of such periodicities and concluded that it will be possible to configure SPS periodicities of 120ms and 240ms that do not divide 10240. However, it is noted that such periodicity will cause the SPS occasion pattern to change every H-SFN and add complexity to UE and Network to maintain the start frame and number of SPS occasions after SFN wrap around until it is released. After SFN wrap around, the SPS occasion may overlap with other scheduling such as SIB1 and PSS/SSS which results in postponement of the SPS occasion. This may cause collision of UL SPS occasion among multiple UEs. Therefore, RAN2 recommends such periodicities to be avoided.

# 2. Actions:

# To SA4:

ACTION:

RAN2 kindly asks SA4 to take the above response into account.

# 3. Dates of Next RAN2 Meetings:

RAN2#132						17th to 21st November 2025		Dallas

RAN2#132						9th to 13th February 2026		Gothenburg