3GPP TSG RAN WG2 Meeting #131bis                          	   R2-2507465

Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on remaining RRC (RIL) issues

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

In this contribution, we discuss the remaining RRC design issues brought up from NES RRC review (RIL) [1].

# Discussion

## Applicability of PO bundling to SL relay

O004 RIL Id; WI; Class; Title. O004; NES, SLRelay; 1; Applicability of PO bundling to SL Relay. [Description]: It is not clear whether the paging adapation (i.e., PO bundling) feature can be applied to SL Relay UE (first/last Relay) and Remote UE. [Proposed Change]: R2 discuss and conclude the applicability of paging adapation (i.e., PO bundling) feature to SL Relay UE (first/last Relay) and Remote UE or not.

For UE-to-Network relay, the Relay UE(s) needs to monitor at a remote UE’s PO for paging indication on behalf of the remote UE. For UEs supporting paging adaptation (e.g., with Ns =8) , Release 19 IE for PO pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 is used (if configured) otherwise firstPDCCH-MonitoringOccasionOfPO.

If both remote UE and relay UE support Release 19 paging adaptation, network may configure Release 19 IE for PO allocation. If the remote UE is legacy UE and the relay UE is Release 19 UE supporting paging adaptation, which IE should be configured for PO?

Proposal 1. For U2N relay, RAN2 discusses if paging adaptation is supported for the following cases:

- Bothe remote UE and relay UE support paging adaptation,

- Only remote UE supports paging adaptation, and

- Only relay UE supports paging adaptation.

## Paging search space for paging adaptation

S035 RIL Id; WI; Class; Title. Xnnn; NES; 2; Paging search space for Adapative POs. [Description]: PagingSearchSpace can be zero or non zero. For paging search space zero, Ns can be 1 or 2 only. In order to have larger value of Ns for adaptive POs, it should be possible to configure non zero paging search space for adaptive POs irrespective of paging search space configuration for legacy POs. There should be no restriction on legacy POs for adaptive PO configuration. So, Paging search space should be separately configured for paging adaptation.

Search Space 0 is the default/common search space configured for paging and SI. It is associated with the Control Resource Set (CORESET#0) and used before dedicated configs are provided. For example, it can be used for RAR, SI and paging.

With Search Space 0, it may be overcrowded with extra POs if increasing Ns beyond value “2”.

Proposal 2. RAN2 discusses if Search Space 0 for paging adaptation should be discussed at RAN1.

# Conclusion

In this contribution, we discussed the remaining RRC design issues and concluded with the following proposals.

Applicability of PO bundling to SL relay

Proposal 1. For U2N relay, RAN2 discusses if paging adaptation is supported for the following cases:

- Bothe remote UE and relay UE support paging adaptation,

- Only remote UE supports paging adaptation, and

- Only relay UE supports paging adaptation.

Paging search space for paging adaptation

Proposal 2. RAN2 discusses if Search Space 0 for paging adaptation should be discussed at RAN1.

# References

- “NES Comments file v036” https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-19%202025-09/38331/CR%20ASN1%20review/NES