3GPP TSG-RAN WG2 Meeting #131bis	R2-2507498

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	MAC open issues

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss MAC open issues for maintenance as listed in R2-2507497 [2].

# Discussion

## Issue-1

Rapporteur proposal 1: For mode-B UEI reporting, PUCCH and type-1 CG PUSCH resource can be associated with different TAGs. Discuss UE behavior when the TAT for PUSCH is expired while the TAT for PUCCH is running.

- Option-1: release the associated PUCCH;

- Option-2a: not release PUCCH and transmit the Report Indication on PUCCH;

- Option 2b: not release PUCCH and not transmit the Report Indication on PUCCH.

PUCCH resource for UEI-CSI reporting is not dedicated for UEI-CSI reporting. Although it is a different PUCCH resource than SR, but it still can be used to transmit HARQ-ACK information or other CSI (e.g., periodic CSI, semi-persistent CSI on PUCCH). We think the PUCCH resource for mode-B should not be released if the associated TAT is still running since it can still be used to transmit HARQ-ACK information or other CSI.

Observation 1: PUCCH resource for mode-B UEI-CSI reporting is not dedicated for UEI-CSI reporting, it can also be used to transmit HARQ-ACK information and other CSI reporting on PUCCH.

Moreover, RAN1 specified that “For a UE configured with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, the UE expects multiple CSI-ReportConfig to be configured in the same CC, the same configuredPUSCHResourceOfModeB if reportTransmissionMode is configured as ‘ModeB’.”. However, for multiple CSI-ReportConfig with different eventType, there is no restriction that only UEI report configurations on the same CC (i.e., same Cell) can share the same PUCCH resource, meaning the same PUCCH resource can be used for different CSI-ReportConfig configured with different eventType and type-1 CG on different cells that are associated to different TAGs. In this case, even if the UEI report on PUSCH for CSI-ReportConfig #1 cannot be transmitted due to TAT expired, the same PUCCH resource can still be used if the same PUCCH is also allocated for CSI-ReportConfig #2 for which the PUSCH can be transmitted.

Observation 2: The same PUCCH resource can be shared for multiple CSI-ReportConfig with different eventType and type-1 CG PUSCH resources on different cells associated to different TAGs.

Another issue for autonomous release the PUCCH resource is that NW may not know when exactly UE releases it, thus release may not help resource reallocation for other usage.

Thus, releasing PUCCH resource when the associated TAT is still running will harm the efficient utilization of the resource to transmit HARQ-ACK information or other CSI reporting, and not beneficial for resource efficiency. We disagree with Option 1.

For Option 2a, it works with nothing broken, and we see the merit that by transmitting the Reporting Indication on PUCCH, the NW may know the configured event is met even without the report on PUSCH. And option 2a has no MAC specification impact in our view.

Some raise the concern for Option 2a that as the current TS 38.214 specifies that UE transmits mode-B UEI-CSI report in PUSCH after transmitting the UEI Report Indication on PUCCH, Option-2a makes this UE behavior not clear since UE cannot transmit the report on PUSCH due to the expired TAT. We think it is clear that the legacy behavior of no UL transmission when TAT expired is always applied, and the procedure of any new functionality is performed complying this legacy behavior.

For Option 2b, we don’t see much motivation to do optimization for TAT expiry which should not be common case. If majority support this option, we need to specify UE behavior clearly in MAC. For example, in clause 5.2 (Maintenance of Uplink Time Alignment): For mode-B UE-initiated CSI reporting, if either the PUCCH or the associated PUSCH is associated with a TAG for which the timeAlignmentTimer is not running, UE does not transmit the UEIRI on the PUCCH and the UEI-CSI report on the associated PUSCH.

Proposal 1: For mode-B UEI reporting, when the TAT for PUSCH is expired while the TAT for PUCCH is running, not release PUCCH. Consider the below options, option 2a is preferred.

- Option 2a: transmit UEIRI on PUCCH, no MAC impact, or

- Option 2b: not transmit UEIRI on PUCCH, and specify UE behaviour in MAC that “For mode-B UE-initiated CSI reporting, if either the PUCCH or the associated PUSCH in CSI-ReportConfig is associated with a TAG for which the timeAlignmentTimer is not running, UE does not transmit the UEIRI on the PUCCH and the UEI-CSI report on the associated PUSCH.”.

## Issue-2

Rapporteur proposal 2: Discuss whether to transmit UEI Report Indication on PUCCH if the BWP for the associated mode-B CG PUSCH is deactivated.

For legacy periodic CSI reporting and semi-persistent CSI on PUCCH, PUCCH resource is allocated for all BWPs in each CSI-reportConfig, so that the CSI can always be reported on the current active BWP. For legacy SP-CSI on PUSCH and AP-CSI reporting, it is triggered by indication in DCI so NW can always schedule the report on the active BWP.

However, for UEI-CSI reporting, mode-A/B PUCCH resource (and also mode-B type-1 CG) is allocated for a specific configured BWP in each CSI-reportConfig. To ensure UE can report on any BWP whenever it is activated, multiple CSI-ReportConfig should be signalled, each with a different BWP ID configured.

Based on this signalling, for a certain CSI-ReportConfig, if the BWP of the mode-B type-1 CG is deactivated, UE should not transmit UEIRI on the PUCCH resource configured in the same CSI-ReportConfig.

In the current specification, as the yellow highlighted part below, the CSI for a deactivated DL BWP will not be reported using any UL resource allocated for the CSI reporting (including PUCCH and PUSCH). For the case that the DL BWP for CSI measurement is activated and the PUSCH resource for CSI reporting is on a deactivated UL BWP, UE behaviour of not transmitting UERI on PUCCH should be specified. The TP is as follows (clause 5.15.1).

1>	if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP;

2>	not transmit UE Initiated Report Indication on PUCCH for CSI-ReportConfig that configures Type-1 configured grant only on the deactivated BWP for mode-B UE-initiated CSI reporting.

Proposal 2: For mode-B UE-initiated CSI reporting, UE does not transmit UE Initiated Report Indication on PUCCH for CSI-ReportConfig that configures Type-1 configured grant only on the deactivated BWP.

## Issue-5

Rapporteur proposal 5: For multipanelSchemeSDM, discuss if the UE should clear the CG/PUSCH for SP-CSI if any of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG of the expired TAT.

For multipanelSchemeSDM, UE transmits different MIMO layers of one TB on two UL panels using two different TCI states towards two TRPs. If the TAT of one TCI state is expired, the UE will not transmit the corresponding layers, and NW may have issue decoding the TB if only receiving layers from the other TCI state. Since the actual applied TCI states for PUSCH are indicated in DCI, NW should ensure to indicate both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB.

As the actual applied TCI states to be used for multi-panel SDM are indicated in DCI, even if the TAT for some of the activated TCI states in MAC CE becomes expired, the other activated TCI states with running TAT can still be used and DCI can the indicated TCI states among these. Thus, in our view there is no need to release the CG/PUSCH for SP-CSI if ANY of the activated TCI state(s) is associated with an expired TAT. It only needs to be released if ALL the activated TCI states are associated to expired TAT(s), which is the current behavior in our understanding, see below.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or

2>	if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running:

…

2>	else:

3>	if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or

3>	if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running:

…

3>	else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells:

4>	clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

4>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

Some raise the change “all the activated TCI state(s)”, in our understanding “all” is the intended behavior of the current procedure. If majority think the current procedure is not clear without mentioning “all”, we can consider a Rel-18 CR, which is a separate discussion.

Proposal 3: For multipanelSchemeSDM, UE clear the CG/PUSCH for SP-CSI if all of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG(s) of the expired TAT(s). This is aligned with legacy behaviour, no change is needed for this.

The current TS 38.321 clause 5.2 has specified that “The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.” For multi-panel SDM scheme, we can consider to specify that UE does not transmit any uplink transmission via multi-panel SDM scheme if any TCI state to be applied for the multi-panel SDM transmission is associated to a TAG for which the TAT is expired.

We can have a common understanding that NW should ensure to indicate in DCI both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB.

Proposal 4: For multi-panel SDM scheme, NW should ensure to indicate in DCI both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB. Discuss whether to specify that “UE does not transmit any uplink transmission via multi-panel SDM scheme if any TCI state to be applied for the multi-panel SDM transmission is associated to a TAG for which the TAT is expired.”

## Issue-10

Rapporteur proposal 8: Discuss if a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI (e.g., the NW does not receive UEIRI from the UE or the NW decides not to schedule the PDCCH), when the UE stops staying in DRX active time.

- Option 1: until the next PUCCH resource for the UEIRI.

- Option 2: Introduce a maximum PDCCH monitoring time period.

- Option 3: others

For SR, UE cancels a pending SR if UL grant becomes available. If a SR is sent and pending, UE in DRX keeps monitoring PDCCH. Similarly, for mode-A UEI-CSI reporting, before receiving DG for mode-A report UE keeps monitoring PDCCH.

The issue raised for UEI-CSI reporting, if valid, exists also for SR. However, we think the mentioned scenario is a corner case. PUCCH is normally reliable to transmit HARQ-ACK, CSI and SR, and NW missing PUCCH should be rare corner case. If the current beam becomes poor, the new event instance will be triggered continuously and the new UEIRI will be transmitted continuously. If NW has successfully received UEIRI on PUCCH, the correct NW behaviour is to schedule the UEI-CSI report on PUSCH, as specified in RAN1. Thus, we think the raised issue is a corner case and optimization is not needed.

Proposal 5: For DRX, no enhancement is pursued to reduce PDCCH monitoring for mode-A UEI-CSI reporting.

# Conclusion

Based on the discussion, we have the following proposal.

Issue-1:

Observation 1: PUCCH resource for mode-B UEI-CSI reporting is not dedicated for UEI-CSI reporting, it can also be used to transmit HARQ-ACK information and other CSI reporting on PUCCH.

Observation 2: The same PUCCH resource can be shared for multiple CSI-ReportConfig with different eventType and type-1 CG PUSCH resources on different cells associated to different TAGs.

Proposal 1: For mode-B UEI reporting, when the TAT for PUSCH is expired while the TAT for PUCCH is running, not release PUCCH. Consider the below options, option 2a is preferred.

- Option 2a: transmit UEIRI on PUCCH, no MAC impact, or

- Option 2b: not transmit UEIRI on PUCCH, and specify UE behaviour in MAC that “For mode-B UE-initiated CSI reporting, if either the PUCCH or the associated PUSCH in CSI-ReportConfig is associated with a TAG for which the timeAlignmentTimer is not running, UE does not transmit the UEIRI on the PUCCH and the UEI-CSI report on the associated PUSCH.”.

Issue-2:

Proposal 2: For mode-B UE-initiated CSI reporting, UE does not transmit UE Initiated Report Indication on PUCCH for CSI-ReportConfig that configures Type-1 configured grant only on the deactivated BWP.

Issue-5:

Proposal 3: For multipanelSchemeSDM, UE clear the CG/PUSCH for SP-CSI if all of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG(s) of the expired TAT(s). This is aligned with legacy behaviour; no change is needed for this.

Proposal 4: For multi-panel SDM scheme, NW should ensure to indicate in DCI both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB. Discuss whether to specify that “UE does not transmit any uplink transmission via multi-panel SDM scheme if any TCI state to be applied for the multi-panel SDM transmission is associated to a TAG for which the TAT is expired.”

Issue-10:

Proposal 5: For DRX, no enhancement is pursued to reduce PDCCH monitoring for mode-A UEI-CSI reporting.

# References

- RP‑242394	WID revision: NR MIMO Phase 5.

- R2-2507497 Report of Rel-19 MIMO MAC open issues for maintenance