3GPP TSG RAN WG2 #131bis	             			            		                                 R2-2507514

Prague, Czech Republic, Oct. 14th –17th, 2025

Agenda Item:	10.3.3

Source:	AT&T

Title:	Considerations on 6G AI/ML Data Collection and Management

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-252912 [1]. The following objectives are of relevance to the AI/ML framework and corresponding data collection in 6G [1]:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution we discuss the requirements and the core design principles for enabling an integrated AI/ML data framework in 6G. We then discuss how having such a standardized framework help reduce the barrier to entry for AI/ML use cases on the air interface.

# The need for an Integrated AI/ML for 6G

In an AI-native 6G system, AI/ML models are expected to be embedded across multiple architectural layers and domains, including the UE, gNB, OAM, and CN. AI/ML models may be deployed concurrently across the protocol stack to optimize network performance. Cross-layer AI/ML is also possible, where inputs from one or more layers are used to train another model, and the resulting inference outputs can be consumed by one or more layers.

Observation 1: An AI-native 6G allows simultaneous deployment of multiple models across layers and across domains in 6GR.

To enable AI-native 6G, cross-vendor collaboration and interoperability, between network infrastructure and UE vendors, and among infrastructure vendors, are essential to ensure AI/ML models are consistently supported and perform as intended across implementations.

Observation 2: Interoperability is needed for AI native 6G

Similarly, the network operator should be able to control data collection end to end, deciding when to collect data, enforcing privacy and security protections, and managing storage and sharing with relevant parties under appropriate SLAs.

Observation 3: Network visibility and control over different data collection aspects is needed for AI/ML models in 6GR.

The AI-native framework should provide flexibility for rapid model updates and timely enablement of new features based on current network needs. It should also be scalable to support quick development and deployment of AI/ML use cases with minimal framework changes and, where feasible, maintain backward compatibility with the 5G framework.

Observation 4: Native AI/ML framework design is flexible and scalable to continuously evolve and deploy AI/ML in 6GR.

Accordingly, a unified, scalable data-management framework across use cases is preferable to per-use-case solutions, which often lead to bulky, inflexible designs and fragmented data collection. A single framework covering data collection, transfer and delivery, and performance monitoring improves efficiency, interoperability, vendor consistency, and scalability, while safeguarding security and privacy.

## Comparison with 5G AI/ML Framework

AI/ML use cases and their related lifecycle management and corresponding data collection framework were not considered as part of the day 1 5G design.  As AI/ML models and use cases emerge at the UE and/ or the network side, their specification in 5G-Advanced is being retrofitted into the 5G architecture and design. The approach taken to standardization of AI/ML in the air interface in 5G-Advanced is very similar to that taken traditionally in 3GPP, to specify, enable, test and potentially deploy features based on specific use cases. The AI/ML field, however, is constantly evolving and having a “one and done” approach is not amenable to continuous innovation and disruption.

The 5G standardization effort for AI/ML use cases on the air interface focuses on the existing 5G radio design and allows for only limited enhancements to it to support AI/ML use cases. This results in a design which is not supportive of any native AI/ML methodology as we will need new enhancements whenever there is a new AI/ML use case. The AI/ML traffic consists of all data related to the AI/ML activities in the network and is not generated or terminated at any end user application.

Observation 5: The 5G framework for AI/ML on the air interface does not allow for native AI/ML integration in 6GR.

In the 5G framework, each use case separately discusses the enhancements to the interfaces to support AI/ML data transfer. This approach is not scalable as for every new use case, the discussion is restarted on how to transfer AI/ML related data across different termination points. This makes it difficult to design a native architecture as it is exceedingly difficult to get performance metrics for models at different layers and design them in a manner such that they do not impact each other’s performance.

Observation 6: The 5G framework for AI/ML is not scalable for AI/ML data transfer

Another issue with the existing framework is the limitation regarding the LCM operations and monitoring metrics that can be done on the control plane. Due to the limited bandwidth of the CP only short monitoring metrics can be reported to the user. Furthermore, using CP for AI/ML LCM is also not very scalable in case of frequent configuration/functionality activation/deactivation/switching/fallback as the load on the CP will increase with number of use cases.

Observation 7: The 5G framework for AI/ML limits LCM operations and metrics used for monitoring on the control plane

## Integrated Framework Principles and Design Requirements

An integrated AI/ML framework enables MNOs to manage network capacity effectively, fostering innovation, and unlocking new value opportunities. A unified approach to training and deploying AI/ML models supports continuous innovation in model development, ensuring the network evolves dynamically. Furthermore, a uniform life cycle management (LCM) framework is crucial for running multiple models simultaneously across different layers, meeting diverse service requirements end-to-end.

To realize this vision, the following core principles should be used for AI/ML framework design in 6GR:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 1: The following core principles are followed to design an AI/ML framework for 6GR air interface:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

To support these core principles, a universal framework to avoid fragmentation and per-use case specification and ensure scalability to new AI/ML use cases is needed. For this framework, multiple termination points for AI/ML data within the network should be supported, with MNO visibility. In addition, 6GR should be designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic.

Proposal 2: AI/ML framework in 6GR should support multiple termination points for AI/ML data within the network with MNO visibility

Proposal 3: 6GR is designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic

In the 5G architecture the control plane (CP) is responsible for signaling and control functions to establish, maintain, and terminate communication sessions between the user and the network, while the user plane (UP) is responsible for all the user data traffic. As the CP contains only the control related information to maintain and manage the communication link between the user and the network, CP by design has a limited amount of bandwidth, low latency requirements and any data transfer on CP is not charged to the users. The UP, however, contains all the data traffic requested by the user and user applications, therefore it has high bandwidth, variable latency/QoS requirements (based on user application) and the users are charged for the data usage on the UP.

The AI/ML models are typically trained by collecting a large real-world dataset and are then transferred and deployed to the users. The size of training dataset can be in range of 100k to 100M data sample, depending on various aspects such as use case, type of model, generalization aspects etc. Similarly, the model size can vary from 10k to 100M parameter model depending on the use case, model type and other aspects. Due to the large amount of data involved, it is important that the network has full visibility and control when the data is shared, to avoid any network impact. Additionally, the users should not be charged for the AI/ML model training data or model transfer.

Having a CP based solution does not work due to the large amount of data being involved and an UP based solution will not give any visibility to the network and can impact its performance.

There are 3 potential approaches that can be used to design data collection and data management for the native AI/ML in 6G

- Continue with the current framework and have a hybrid of CP and UP

- CP overhaul and redesign

- A new plane: AMP (AI/ML plane) in parallel with the current CP and UP

The first option is not well suited for a native AI/ML approach and borrows from the 5G framework. A hybrid approach using CP and UP requires significant specification impact and coordination across multiple work groups across RAN and CN. It also increases the signaling load on the CP due to reporting of when and what data is being shared. The increase in signaling load also makes such an approach not scalable with the number of use cases. In addition, it is very difficult to optimize data collection for multiple use cases in case some of the input data is common among the use cases.

The second option would require a significant specification impact and would result in the 6G framework to be entirely non compatible with the legacy 5G framework. Such a design can allow for several of the core issues with the AI/ML framework to be properly addressed. However, there will still be an issue of increased load on the CP with the number of use cases. In addition, unless we allow the new CP to have higher or adaptable bandwidth there will still be an issue with configuring resource utilization over the new CP, and with unnecessarily using the CP for latency tolerant data transfer/delivery.

The third option is to have an entirely new plane which we name AMP, short for AI/ML Plane, for all AI/ML related traffic. The AMP is responsible for the AI/ML related data traffic for the data collection and model transfer. We can also utilize AMP for transfer of AI/ML model monitoring metrics between the UE side and the network side. Furthermore, the AMP can be designed to perform lifecycle management (LCM) of AI/ML models and functions, including activation, deactivation, switching, and fallback. This will allow all AI/ML related aspects to be managed from within the AMP, which enables us to design new use cases quickly and efficiently for native AI/ML.

AMP, by design, combines benefits of both CP and UP solutions. Traffic that flows through the AMP is intended to be both originated and terminated either at the UE device or at different entities in the network, similar to CP traffic. Additionally, traffic at AMP will not be charged to the user. At the same time, the traffic characteristics of the AMP with regards to model transfer/delivery and data collection requirements are like those of the UP traffic.

The AMP offers better data management as the network can collect data for multiple use cases simultaneously, which results in improved efficiency (as there may be some common inputs for different AI/ML models operating at different layers or even at different domains/network functions). Furthermore, the network can add relevant additional information and labels to the data. The network can also ensure that the privacy is preserved, and proprietary and security aspects of the data are properly addressed before the data is shared with the network or UE vendors to develop different models. Additionally, it enables the network to control the timing of data collection and model transfer while minimizing the impact on system performance. Finally, the AMP architecture provides us with a framework that can be used to quickly develop and deploy AI/ML use cases based on the network requirements, reducing the standardization effort to onboard new use cases based on the established framework.

Proposal 3: for the AI/ML framework in 6GR, study the introduction of an AI/ML data management plane to manage data collection, model transfer/delivery and LCM aspects of different AI/ML use cases on 6GR interface

For both UE-side and network-side data collection, the AMP should ensure that AI/ML-related data is logically separated from other traffic on both the User Plane and Control Plane. This separation enables clean, scalable evolution of 6G networks with minimal impact on existing services and allows prioritization of AI/ML traffic when needed. To achieve these objectives, a dedicated radio bearer is required.

Proposal 4: For the AI/ML framework in 6GR, study the introduction of a new bearer over the Uu interface for carrying AI/ML-related data.

As new technologies and services emerge, the demand for network data collection continues to grow. A unified, service-agnostic data collection framework should therefore be established—one that supports a broad range of use cases, with AI/ML being only one among them. A core design principle is to preserve a clean, consistent end-to-end path for collected data, spanning collection, transport, and delivery – irrespective of the service involved. The framework should also natively support multiple termination points within the network. In summary, the mechanisms and principles proposed for an AI/ML-specific data collection framework apply equally to a common framework and should be interpreted in a service-agnostic manner.

Proposal 5: For 6GR, study the feasibility of a unified, service-agnostic data collection framework. All requirements proposed for AI/ML specific data collection framework are applicable to the unified framework.

# Conclusion

In this contribution, we discussed requirements for AI/ML data collection and management in 6G, we introduced a new framework to enable an extensible data collection framework. We made the following observations and proposals.

Observation 1: An AI-native 6G allows simultaneous deployment of multiple models across layers and across domains in 6GR.

Observation 2: Interoperability is needed for AI native 6G

Observation 3: Network visibility and control over different data collection aspects is needed for AI/ML models in 6GR.

Observation 4: Native AI/ML framework design is flexible and scalable to continuously evolve and deploy AI/ML in 6GR.

Observation 5: The 5G framework for AI/ML on the air interface does not allow for native AI/ML integration in 6GR.

Observation 6: The 5G framework for AI/ML is not scalable for AI/ML data transfer

Observation 7: The 5G framework for AI/ML limits LCM operations and metrics used for monitoring on the control plane

Proposal 1: The following core principles are followed to design an AI/ML framework for 6GR air interface:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 2: AI/ML framework in 6GR should support multiple termination points for AI/ML data within the network with MNO visibility

Proposal 3: 6GR is designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic

Proposal 4: For the AI/ML framework in 6GR, study the introduction of a new bearer over the Uu interface for carrying AI/ML-related data.

Proposal 5: For 6GR, study the feasibility of a unified, service-agnostic data collection framework. All requirements proposed for AI/ML specific data collection framework are applicable for the unified framework.

# References

[1] RP-252912, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic