3GPP TSG-RAN WG2 Meeting #131bis	R2-2507532

Prague, Czech, 13th – 17th October 2025

Source:	NTT DOCOMO, INC.

Title:	User plane corrections for XR Enhancements Ph3

Document for:	Discussion

Agenda Item:	8.7.3

- Introduction

This contribution is to provide our views on remaining issues for user plane.

- Discussion

  - Prevention from unnecessary delay caused by t-Reordering

The following issue was raised in post email discussion after #131 meeting.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-V01 |

For DL, the receiving side, i.e., the UE maintains t-RxDiscard at RLC layer and t-Reordering at PDCP layer, whose values are configured by the network respectively. When t-RxDiscard expires, the receiving side of a RLC entity stops waiting for the obsolete RLC SDUs and transmits a positively ACK STATUS report to the network. At the same time, a receiving PDCP entity would wait for the obsolete RLC SDUs from RLC layer and t-Reordering of the entity is running. However, if the network configures t-RxDiscard and t-Reordering with their values which are similar to each other, this wouldn’t cause unnecessary delay associated with t-Reordering and the PDCP entity could deliver PDCP SDUs to upper layer timely.

For UL, similarly as described above, the receiving side, i.e., the network both maintains t-RxDiscard and t-Reordering and configures them. If the network configures them with appropriate values, there is no unnecessary delay associated with t-Reordering.

In conclusion, configuring t-RxDiscard and t-Reordering with appropriate values by the network, in other words, the network implementation could resolve this issue.

Proposal 1.	Configuring t-RxDiscard and t-Reordering with appropriate values by the network, in other words, the network implementation could resolve unnecessary delay associated with t-Reordering.

  - The new definition of RX_Next

One company proposed the change to the new definition of RX_Next in alignment with t-RxDiscard expiry in the post email discussion as follows.

| Nokia | [N001] RX_Next in clause 7.1 The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires. [Rapp]: It is true, and we need to update it. Your poropsal is one way, which may miss some information for legacy case. Another way is to capture like: if t-RxDiscard is configured, xxx. Otherwise, xxxx. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-N01 |
| --- | --- | --- |

We agree with the necessity to update it. Regarding what the sentence is like, in order to keep the conventional definition as much as possible and clarify that XR functionality has an impact on the RX_Next update, the condition whether t-RxDiscard is configured or not should be introduced in the sentence. Based on this consideration, one of the new candidate definitions of RX_Next is as follows.

a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. If t-RxDiscard is not configured, it is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next. if t-Rxdiscard is configured, it is initially set to 0, and updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next and t-RxDiscard expires.

Proposal 2.	To keep the conventional definition as much as possible and clarify that XR functionality has an impact on the RX_Next update, the condition whether t-RxDiscard is configured or not should be introduced in the sentence.

  - Necessity to update NOTE in clause 5.3.3.3

One company proposed the NOTE in clause 5.3.3.3 should be updated as described below.

| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. [Rapp]: This issue was discussed in RAN2#130 based on Apple’s proposal. The agreement includes the ACK/NACK/Discard cases. I understand the motivation in Apple’s contribution is that in the case where a NACK has just been received, there is no need for polling since the status is already known, and the retransmission can be performed directly. While the current Note in the specification also states “may stop and reset,” so I think this should not be a major issue. But considering it has not been extensively discussed during RAN2#130, let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |
| --- | --- | --- |

Regarding the 1st suggestion of the proposal in the above table, the NOTE should be clarified that it is intended the case where stopReTxDiscardSDU is configured. Therefore, “If stopReTxDiscardedSDU is configured” should be added at the beginning of the sentence.

Regarding the 2nd suggestion, as long as the transmitting side of RLC entity is aware of all RLC SDU with SNs up to and including POLL_SN are already positively or negatively acknowledged, there is no need to transmit a poll bit and receive a STATUS report to/from the peer RLC entity, since all the transmitting side of RLC entity will do is retransmit negatively acknowledged PDUs to the peer RLC entity. Therefore, there is no problem to stop and reset the running t-PollRetransmit even when all RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged.

In conclusion, the NOTE should be updated as follows,

NOTE: If stopReTxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Proposal 3.	It is no problem to stop and reset the running t-PollRetransmit even when all RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged. There is no change of NOTE in 5.3.3.3 other than adding “If stopReTxDiscardedSDU is configured” at the beginning of the sentence

- Summary and proposal

Proposal 1.	Configuring t-RxDiscard and t-Reordering with appropriate values by the network, in other words, the network implementation could resolve unnecessary delay associated with t-Reordering.

Proposal 2.	To keep the conventional definition as much as possible and clarify that XR functionality has an impact on the RX_Next update, the condition whether t-RxDiscard is configured or not should be introduced in the sentence.

Proposal 3.	It is no problem to stop and reset the running t-PollRetransmit even when all RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged. There is no change of NOTE in 5.3.3.3 other than adding “If stopReTxDiscardedSDU is configured” at the beginning of the sentence

- References

[1]　 R2-2507017, “List of RLC open issues for R19 XR,” vivo, RAN2#131bis.