3GPP TSG-RAN WG2 Meeting #131bis	R2-2507542

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	10.3.1

Source: 	ASUSTeK

Title:  	Discussion on 6G User Plane design

Document for:	Discussion and Decision

- Introduction

In 6G SID [1], one objective is to study architecture and protocol design for User plane in 6G:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we’d like to provide analysis for baseline User plane design in 6G.

- discussion

6G User Plane protocol stack

The 5G protocol stack should serve as the baseline for the design of the 6G protocol stack. To support evolving service demands and to achieve a scalable 6GR design for diverse device types, the fundamental architectural of the 5G protocol stack offer a mature and well-adopted foundation to build on:

(38.300) Figure 4.4.1-1: User Plane Protocol Stack

Proposal 1:	NR User Plane protocol stack is considered as baseline for 6G User Plane protocol stack.

MAC sublayer design

For 6G MAC layer design, the services and functions provided in 5G NR MAC (e.g., logical channel prioritization, HARQ operation) should be leveraged as a starting point.

(38.300) ### 6.2.1 Services and Functions The main services and functions of the MAC sublayer include: - Mapping between logical channels and transport channels; - Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels; - Scheduling information reporting; - Error correction through HARQ (one HARQ entity per cell in case of CA); - Priority handling between UEs by means of dynamic scheduling; - Priority handling between logical channels of one UE by means of logical channel prioritisation; - Priority handling between overlapping resources of one UE; - Padding.

Proposal 2:	Services and Functions supported by NR MAC sublayer (i.e. in TS 38.321) are considered as a starting point for 6G MAC sublayer.

On the other hand, in previous RAN1 agreement on 6G design [2], it was agreed to study applicable device types for a scalable 6G design:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

As it is still uncertain for AIoT to be considered as an applicable device type, Service and functions for AIoT should not be considered as a starting point for 6G MAC design.

Proposal 3:	Services and Functions supported by NR AIoT MAC sublayer (i.e. in TS 38.391) are not considered as a starting point for 6G MAC sublayer.

RLC sublayer design

For 6G RLC layer design, supporting different transmission modes (TM, AM, and UM) should be considered as working assumption, and the services and functions provided in 5G NR RLC (e.g., ARQ) should be leveraged as a starting point:

(38.300) ### 6.3.1 Transmission Modes The RLC sublayer supports three transmission modes: - Transparent Mode (TM); - Unacknowledged Mode (UM); - Acknowledged Mode (AM). The RLC configuration is per logical channel with no dependency on numerologies and/or transmission durations, and ARQ can operate on any of the numerologies and/or transmission durations the logical channel is configured with. For SRB0, paging and broadcast system information, TM mode is used. For other SRBs AM mode used. For DRBs, either UM or AM mode are used. ### 6.3.2 Services and Functions The main services and functions of the RLC sublayer depend on the transmission mode and include: - Transfer of upper layer PDUs; - Sequence numbering independent of the one in PDCP (UM and AM); - Error Correction through ARQ (AM only); - Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs; - Reassembly of SDU (AM and UM); - Duplicate Detection (AM only); - RLC SDU discard (AM and UM); - RLC re-establishment; - Protocol error detection (AM only).

Proposal 4: 	Consider 6G RLC sublayer supporting TM, UM, and AM transmission modes as a working assumption.

Proposal 5:	Services and Functions supported by NR RLC sublayer (i.e. in TS 38.322) are considered as a starting point for 6G RLC sublayer.

PDCP sublayer design

For 6G PDCP layer design, the services and functions provided in 5G NR PDCP should be leveraged as a starting point:

(38.300) ### 6.4.1 Services and Functions The main services and functions of the PDCP sublayer include: - Transfer of data (user plane or control plane); - Maintenance of PDCP SNs; - Header compression and decompression using the ROHC protocol; - Header compression and decompression using EHC protocol; - Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only; - Ciphering and deciphering; - Integrity protection and integrity verification; - Timer based SDU discard; - For split bearers, routing; - Duplication; - Reordering and in-order delivery; - Out-of-order delivery; - Duplicate discarding.

Proposal 6:	Services and Functions supported by NR PDCP sublayer (i.e. in TS 38.323) are considered as a starting point for 6G PDCP sublayer.

SDAP sublayer design

For 6G SDAP layer design, the services and functions provided in 5G NR SDAP (e.g., QoS flow and DRB mapping) should be leveraged as a starting point.

(38.300) ## 6.5 SDAP Sublayer The main services and functions of SDAP include: - Mapping between a QoS flow and a data radio bearer; NOTE: When remapping an XR QoS flow carrying PDU sets (see clause 16.15) from one DRB to another (as exemplified in Annex A), all SDAP SDUs belonging to a PDU Set should be mapped to the same DRB. - Marking QoS flow ID (QFI) in both DL and UL packets. A single protocol entity of SDAP is configured for each individual PDU session.

Proposal 7:	Services and Functions supported by NR SDAP sublayer (i.e. in TS 37.324) are considered as a starting point for 6G SDAP sublayer.

- Conclusion

We have the following observations and proposals for 6GR protocol design:

Proposal 1:	NR User Plane protocol stack is considered as baseline for 6G User Plane protocol stack.

Proposal 2:	Services and Functions supported by NR MAC sublayer (i.e. in TS 38.321) are considered as a starting point for 6G MAC sublayer.

Proposal 3:	Services and Functions supported by NR AIoT MAC sublayer (i.e. in TS 38.391) are not considered as a starting point for 6G MAC sublayer.

Proposal 4: 	Consider 6G RLC sublayer supporting TM, UM, and AM transmission modes as a working assumption.

Proposal 5:	Services and Functions supported by NR RLC sublayer (i.e. in TS 38.322) are considered as a starting point for 6G RLC sublayer.

Proposal 6:	Services and Functions supported by NR PDCP sublayer (i.e. in TS 38.323) are considered as a starting point for 6G PDCP sublayer.

Proposal 7:	Services and Functions supported by NR SDAP sublayer (i.e. in TS 37.324) are considered as a starting point for 6G SDAP sublayer.

- Reference

[1] RP-251881

[2] Draft Report of 3GPP TSG RAN WG1 #122 v0.1.0