3GPP TSG-RAN WG2 Meeting #131-bis	R2- 2507551

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Remaining MAC issues in R19 mobility

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining MAC issues for R19 mobility are discussed.

Discussion

MAC-V04 and serving cell CLTM timer handling

Regarding the case when UE have valid CLTM TAT and receives an LTM CSC MAC-CE, in the last meeting RAN2 had the following agreement:

If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

Basically, it specifies the case when UE have valid TA for target cell, while network send LTM CSC MAC-CE with value “FFF” for the TA value, the UE will perform RACH-based LTM. (I.e., the case 1 in the following table)

The following table summarise the actions and TAT handlings for CLTM and pTAG in different cases when CLTM and LTM co-exist:

|  | LTM CSC with invalid TA=“FFF” |  | LTM CSC with valid TA (other than “FFF”) |  |
| --- | --- | --- | --- | --- |
| UE have valid TA for target | Case 1 | RACH-based LTM | Case 2: | RACH-less LTM |
|  |  | UE refresh PTAG TAT |  | UE refresh PTAG TAT |
|  |  | FFS CLTM TAT handling? |  | FFS CLTM TAT handling? |
| UE have NO valid TA for target | Case 3 | RACH-based LTM | Case 4 | RACH-less LTM |
|  |  | UE refresh PTAG TAT |  | UE refresh PTAG TAT |
|  |  | FFS CLTM TAT handling? |  | FFS CLTM TAT handling? |

For all four cases where the UE receives an LTM Cell Switch Command (CSC) MAC CE, there is currently no clear consensus on how the UE should handle the CLTM TAT timer.

A guidance can be found from the RAN2#129bis agreement, that both pTAG TAT and CLTM TAT will not be stopped upon CLTM cell switch, with different intentions:

- CLTM-TAT is used for potential subsequent CLTM in the near future.

- PTAG TAT is used for serving cell TA maintenance.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

Therefore, in the case when LTM CSC is received, the CLTM TAT should be still utilized for potential subsequent LTM.  So it should keep aligned with PTAG TAT. This could applied to all 4 cases .

Also, Rapporteur posts another issue that, whether UE should maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE for current serving cell. We think the same strategy should be applied and the CLTM-TAT should be updated for potential subsequent CLTM in the future.

Therefore, the CLTM-TAT and pTAG TAT for the serving cell should always be aligned.

Proposal 1: The CLTM-TAT and pTAG TAT for serving cell should always be aligned. This includes the following cases:

- Two TAT timers are refreshed by TA indicated by LTM CSC MAC CE

- Two TAT timers are refreshed by RACH procedure during LTM

- Two TAT timers are refreshed by legacy Timing Advance Command MAC CE

RACH-based for CLTM recovery (M01)

Regarding the miss of CLTM description on LTM fast recovery, the suggested TP in stage-2 open issue are copied as below:

When LTM execution attempt triggered by LTM cell switch command MAC CE fails or when the CLTM execution attempt by the UE fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

It is come from the agreement of RAN2#129 meeting, with the intention of reduce complexity and avoiding too much optimization:

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

While according to stage-3 spec, there is no clear guidance that UE will perform RACH-based LTM after CLTM recovery.  In RRC spec, two different conditions (CLTM recovery, fulfilment of CLTM condition) have the same action:

- 5.3.5.18.6 LTM cell switch execution …… 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3;

In MAC spec, either 5.18.35 (Enhanced) LTM Cell Switch Command or 5.y.3	Conditional LTM execution does not reflect the action for CLTM recovery case.  And it is not clear if CLTM fast recovery will go to 5.y.3 Conditional LTM execution, as the condition is not included:

- 5.y.3 Conditional LTM execution The conditional LTM cell switch procedure is triggered when: - the MAC entity determines that the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.y.2; or - the event(s) for conditional LTM is satisfied based on L3 measurements indicated by upper layers.

In fact, the whole LTM fast recovery seems not well reflected in MAC spec. For R18 LTM fast recovery, UE does not have other stored TA value, so it is natural to fallback to RACH-based LTM. While for R19 CLTM fast recovery, it is better to be specified somewhere to clarify UE’s behavior, as UE may have other stored TA and may be capable to perform RACH-less LTM.

Proposal 2: RAN2 to discuss whether to capture that “UE only could perform RACH-based LTM for CLTM recovery” in stage-3 spec, or keep it in stage-2 only.

Conclusion

In this contribution, the following proposal are made:

Proposal 1: The CLTM-TAT and pTAG TAT for serving cell should always be aligned. This includes the following cases:

- Two TAT timers are refreshed by TA indicated by LTM CSC MAC CE

- Two TAT timers are refreshed by RACH procedure during LTM

- Two TAT timers are refreshed by legacy Timing Advance Command MAC CE

Proposal 2: RAN2 to discuss whether to capture that “UE only could perform RACH-based LTM for CLTM recovery” in MAC spec, or keep it in stage-2 only.

References

- R2_131_R18SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2-250xxxx_List of MAC open issue for R19 mobility_v14_Rapp2