3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507605

Prague, Czech Republic, Oct 13th - 17th 2025

Source:        ZTE Corporation

Title:           [Z408][K103][H400][H403][Z409]RIL Issues for MIMO

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

According to ASN.1 review, some RIL issues are left for the further discussion.

| RIL Id | Title | Status |
| --- | --- | --- |
| Z408 | Redundant servCellIndex-r19 for the UEIBR mode B configuration | ToDo |
| K103 | Redundant information in mode B CG configuration | ToDo |
| H400 | In which serving cell to include UE-initiated CSI reports is unclear | ToDo |
| H403 | Support of LTM cell switch | ToDo |
| Z409 | Change the ASN.1 coding for the additionalOneSlotOffsetDoppler-r19 | PropReject |

In this paper, we’d like to share our views on these RIL issues.

- Discussion

In this chapter, we will discuss Redundant servCellIndex issue in [Z408][K103][H400] first, then share our views on LTM with PL offset feature, and finally give our further consideration on the [Z409].

2.1 [Z408][K103][H400] Redundant servCellIndex

All three RILs are related to cell/BWP indication for cross-cell scenarios. Regarding these scenarios, RAN1 has reached several key agreements.

Agreement #1 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied. Agreement #2 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. Agreement#3 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. Agreement#4 - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability Agreement #5 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate one from {SpCell, PUCCH-Scell} as the cell of the configured PUCCH.

For the discussion convenience, we can note the cells for the indicated TCI state, current beam/new beam, first PUCCH and the PUSCH as Cell A/B/C/D respectively:

- Cell A: The cell on which the indicated TCI state used to determine the current beam RS

- Cell B: The cell of the current beam and the new beams resource location

- Cell C: The cell of the PUCCH resource location

- Cell D: The cell of the PUSCH resource location

Based on the above agreements, Cell A/B/C/D can be totally different cells, and according to the current Asn.1 signaling in the running CR, they are signaling as below:

Table 1 Asn.1 Signaling for each Cell

| Cell | Asn.1 Signaling |
| --- | --- |
| Cell A: Indicatd TCI state of the current Cell | CSI-ReportConfig->csi-ReportUE-IBR-r19-> tci-ServCellIndex-r19 |
| Cell B: The cell of the current beam and the new beams | CSI-ReportConfig->Carrier |
| Cell C: The cell of the PUCCH | CSI-ReportConfig->pucch-Resource-r19-> ul-BWP-Id-r19/pucch-Cell-r19 |
| Cell D: The cell of the PUSCH | CSI-ReportConfig->reportTransmissionMode-r19 modeB-r19-> ul-BWP-Id-r19/servCellIndex-r19 |

For the cell A/B/C, the current signaling is OK, but for the cell D, according to the above agreements #2, the Cell D should be the Cell in which the CSI report is configured.

Agreement #2 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

Thus the cell D has been implicitly indicated by the corresponding ServingCellConfig of the CSI-ReportConfig (e.g. ServingCellConfig-> CSI-MeasConfig->CSI-ReportConfig).

Now in the 3 RILs, this problem was raised and the solutions are from the different perspectives:

- Option 1: As in K103, it can use MAC level Index (ConfiguredGrantConfigIndexMAC) to avoid BWP and serving cell index indication

- Option 2: As in the H400, remove the field pucch-Cell-r19 and update the description of CSI-ReportConfig.

- Option 3: As in the Z408, remove the “servCellIndex-r19” from the CSI-ReportConfig-> reportTransmissionMode-r19 ->modeB-r19-> pusch-ResourceOfModeB-r19.

Regarding Option 1, it would still require at least some restriction on the serving cell index (e.g., that the Type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB is on the same component carrier as the corresponding CSI-ReportConfig). In other words, the redundancy persists, albeit in an implicit form. Option 2, however, appears misaligned with RAN1's agreements and could impact the RAN1 specification.

To maintain alignment with RAN1's agreements, we think the serving cell of the CSI-MeasConfig can be used to indicate Cell D, making it unnecessary to explicitly indicate the serving cell index for the Mode B CG PUSCH.

Proposal 1: Remove the “servCellIndex-r19” from the CSI-ReportConfig-> reportTransmissionMode-r19->modeB-r19-> pusch-ResourceOfModeB-r19.           .

2.2 [H403]LTM and PL Offset

The H403 is about the LTM and PL offset, the description and the proposed change is as follows:

[Description]: At LTM cell switch, the UE applies the LTM candidate TCI state indicated in the LTM cell switch command until a TCI state is indicated in the target cell. An LTM candidate TCI state cannot indicate pathlossoffset, so it can only indicate the UL TRP that has associated DL, but not the other UL TRP. In some scenario, this may result in insufficient transmission power for cell switch, resulting in failure. [Proposed Change]: Add pathlossOffset to CandidateTCI-UL-State-r18: .....

In the last meeting, this issue is also postponed as follows:

R2-2505693 Discussion on PL offset in the case of LTM Lenovo discussion Rel-19 =>Discussion is postponed and should take place in MIMO first. In October during ASN.1 review cross-WI discussion we may discuss whether the feature can be extended to LTM

Because there are more important issues to be processed, this topic was not addressed in the MIMO session. It remains unclear whether it should be discussed in the MIMO session or a cross-Work Item (WI) session.

Regarding the MIMO session, we could first discuss whether to include the Path Loss (PL) offset in the LTM Cell Switch Command. According to company contributions, this PL offset could be applied to the RACH procedure or the first PUSCH transmission in a RACH-less procedure. However, based on RAN1 discussions, the RACH procedure on the UL-Only TRP is currently only intended for Timing Advance (TA) acquisition. Other purposes for the RACH procedure have not even been discussed in RAN1. Therefore, it is premature to include the PL offset in the cell switch command for use in subsequent RACH procedures or the first PUSCH transmission of a RACH-less procedure.

Proposal 2: No need to include the PL offset in the LTM Cell Switch Command.

Based on Proposal 2, we can further discuss the utility of including the PL Offset in the candidate cell configuration. According to the RAN1 discussions, the current design in R19 does not support UE access to a target cell via a UL-only cell, as the RACH procedure on a UL-Only TRP is solely for TA acquisition. Consequently, the only potential use for the path loss indication in the candidate cell would be for early UL synchronization.

If it were to be used for early UL synchronization, several issues remain unresolved. Beyond the pre-configuration of the PL offset, it is unclear whether this offset should be considered for preamble transmission. In normal asymmetric cases, whether the PL offset is used in preamble transmission is determined by corresponding bits in the DCI, which in turn depends on the RRC parameter pathlossOffsetPRACH-DCI-1-0. A similar issue would need to be discussed for the LTM candidate cell configuration, which is evidently not solely a RAN2 matter.

Furthermore, if current specifications do not support UE access to the target cell from a UL-only TRP, then early UL synchronization itself may be unnecessary. Thus, it is preferable to first clarify the specific use case for configuring the PL offset in the candidate cell before deciding on its inclusion.

Proposal 3: RAN2 should first clarify the usage scenario for configuring the PL offset for the candidate cell, and subsequently determine whether and how to include it.

2.3 [Z409] Additional One Slot Offset Doppler

The [Z409] is about the additionalOneSlotOffsetDoppler-r19. The description and also the proposed change are copied as follows:

[Description]: The parameter in the corresponding RAN1 table and the running CR is as follows: NZP-CSI-RS-ResourceSet; additionalOneSlotOffset Dopp; For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet; {enabled}. [[ kdopp-r19 SEQUENCE { numberOfResourceGroups-r19 ENUMERATED {n4, n8, n12}, numberOfResourcesPerGroup-r19 ENUMERATED {n2, n3, n4} } OPTIONAL, -- Need R additionalOneSlotOffsetDoppler-r19 CHOICE { resourceGroup4 BIT STRING(SIZE(4)), resourceGroup8 BIT STRING(SIZE(8)), resourceGroup12 BIT STRING(SIZE(12)) } OPTIONAL -- Need R Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. ]]. However, RAN1’s intention is not to define this parameter per Group, instead this parameter is for all of the groups. Thus there is no need to define this parameter as a list. [Proposed Change]: No need to define additionalOneSlotOffsetDoppler-r19 as a list, instead it should be define as ENUMERATED {enabled} and applied for all groups. additionalOneSlotOffsetDoppler-r19 ENUMERATED {enabled} OPTIONAL -- Need R [Comments]: [Ericsson(Lian)] This was hanging for many meetings when RAN2 at the end decided to insert this parameter prer group. It seems better to raise this directly in RAN1 to get a clear direction in RAN2. [Nokia (Andrew)] We also think this is defined incorrectly right now but are not completely sure whether ZTE’s proposal is the intended way either. Perhaps RAN2 can ask RAN1 to clarify.

In the RAN1 parameter list, it has indicated this parameter as {enabled}, however, it seems that RAN2 companies have different understandings on this parameter. Thus it’s better to send an LS to confirm this issue first before any decision.

Proposal 4: Send an LS to RAN1 to confirm whether there is a need to define additionalOneSlotOffsetDoppler-r19 as a list.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Remove the “servCellIndex-r19” from the CSI-ReportConfig-> reportTransmissionMode-r19->modeB-r19-> pusch-ResourceOfModeB-r19.

Proposal 2: No need to include the PL offset in the LTM Cell Switch Command.

Proposal 3: RAN2 should first clarify the usage scenario for configuring the PL offset for the candidate cell, and subsequently determine whether and how to include it.

Proposal 4: Send an LS to RAN1 to confirm whether there is a need to define additionalOneSlotOffsetDoppler-r19 as a list.