3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507606

Prague, Czech Republic, Oct 13th - 17th 2025

Source:        ZTE Corporation

Title:           Consideration on the Type 2 and Type 4 UE Capability Reporting

Agenda item:   8.20.1

Document for:  Discussion and Decision

- Introduction

In the last meeting, the CRs on the “intra-band non-collocated EN-DC/NR-CA deployment phase 2” have been agreed/endorsed [1][2], but the LS is postponed.

R2-2506248 Draft LS to RAN4 on Type 4 for non-collocated deployment Noted, can discuss the LS in the next meeting

The main issue is that whether the UE can report the type 2 or type 4 UE capabilities for the super BC. In this paper we share our views this issue.

- Discussion

In the current spec, the MR-DC UE capabilities for the type2 and type 4 UE are defined as below:

| requirementTypeIndication-r18 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16. | UE | No | No | FR1 only |
| --- | --- | --- | --- | --- |
| interBandMRDC-WithOverlapDL-Bands-r19 Indicates whether the UE supports TDD-TDD inter-band (NG)EN-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and inter-band UE RF requirements for inter-band EN-DC including 7.10B in TS38.101-3 [4] (i.e. Type 4 UE). Type 4 supported UE shall also meet Type 1 UE requirements which are TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and inter-band UE RF requirements excluding 7.10B in TS38.101-3 [4] (i.e. Type 1 UE). If the capability is not reported, the UE supports the Type 1 UE requirements. A UE supporting this feature shall also support the capability of requirementTypeIndication-r18. A UE supporting this feature shall also support network controlled indication of MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-v19xy for inter-band non-collocated MRDC, as defined in TS 38.331 [9]. | BC | No | N/A | FR1 only |

Then according to RAN4 spec, the supported CC numbers on the NR side are different for the type 2 and type 4 requirements:

- For the BC with Type 4 requirement, it supports only single CC on the NR side, e.g. DC_42A_n77A;

- For the BC with Type 2 requirement, it can support at most 2 CCs on the NR side e.g. DC_42A_n77C;

In the last meeting, one remaining issue is whether and how to report these Type2/4 capabilities in the super BCs, e.g. the BC include more bands or more CCs. There are 2 different understandings:

- Understanding 1: The super BC is not allowed to report these 2 capabilities.

- Understanding 2: The super BC is allowed to report these 2 capabilities, but these 2 capabilities are only workable for the fullback part that are defined in RAN4 spec.

For the understanding 2, it can further include 2 cases:

- Understanding 2a: For the super BC with type 4 capability, the UE can only support this capability on the fallback BC with only single CC on the NR side.

- Understanding 2b: For the super BC with type 2 capability, the UE can only support this capability on the fallback BC with at most 2 CCs on the NR side.

For example, for the UE supports type 4 capability (i.e. interBandMRDC-WithOverlapDL-Bands-r19) on the DC_42A_n77A, the UE can report a super BC with DC_42A_n77C to save the signaling overhead, at the network side, the network would only take the interBandMRDC-WithOverlapDL-Bands-r19 as supported for the 42A_n77A part.

Proposal 1: Allow the UE to report the super BC with type 4 capability (i.e. interBandMRDC-WithOverlapDL-Bands-r19), with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side.

For the type 2, the case is a little different. Currently, according to the RAN4 table, at most 2 CCs at NR side is defined for the related BC, thus the supper BC can only include more bands or more CCs on the other bands. Take the DC_42A_n77C as an example, RAN4 has not defined any BC with 42A_n77D components now. However, to avoid forward compatibility issue, if it’s allowed to report the type 2 capability (i.e. requirementTypeIndication-r18 ) with the super BC, it’s better to add a restriction on the CC number.

Proposal 2: Allow the UE to report the super BC with type 2 capability (i.e. requirementTypeIndication-r18), with the restriction that this capability is applicable only to the fallback BC with at most 2 CCs on the NR side.

Furthermore, considering that the type 2 capability (i.e. requirementTypeIndication-r18) is the prerequisite of the type 4 capability (i.e. interBandMRDC-WithOverlapDL-Bands-r19), for a super BC, if it can’t support the corresponding type 2 capability, the UE can’t report the type 4 capability for this super BC. For example, if the UE support DC_42A_n77A with the type 4 capability but doesn’t support the type 2 capability on the DC_42A_n77C, the UE should not report the DC_42A_n77C with type 4 capability, otherwise the NW may assume that the UE also supports DC_42A_n77C with type 2 capability.

Proposal 3: For a given super BC, if the UE does not support the corresponding type 2 capability, it shall not report the type 4 capability for that super BC.

If the above proposals are agreed in RAN2, an LS can be sent to RAN4 for the information.

Proposal 4: If the above 3 proposals are agreed in RAN2, send an LS to RAN4 for the information.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Allow the UE to report the super BC with type 4 capability (i.e. interBandMRDC-WithOverlapDL-Bands-r19), with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side.

Proposal 2: Allow the UE to report the super BC with type 2 capability (i.e. requirementTypeIndication-r18), with the restriction that this capability is applicable only to the fallback BC with at most 2 CCs on the NR side.

Proposal 3: For a given super BC, if the UE does not support the corresponding type 2 capability, it shall not report the type 4 capability for that super BC.

Proposal 4: If the above 3 proposals are agreed in RAN2, send an LS to RAN4 for the information.

- Reference

- R2-2506254	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	draftCR	Rel-19	38.306	18.6.0

- R2-2506256	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	CR	Rel-19	38.331	18.6.0	5479	-	B