3GPP TSG-RAN WG2 Meeting #131b	R2-2507625

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Remaining RRC issues for NR NTN

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss some remaining issues related to RILs raised in ASN.1 review [1].

Discussion

DL CE

(Z256) Missing linked SS in paging search space configuration

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z256 | NTN | 2 | Missing linked SS in paging search space configuration | None | ZTE (Zhihong) |  | v012 | ToDo |

[Description]: SearchSpaceLinkingIdCE is only used to linked two SS, while searchSpaceId for the additional SS for pagingSearchSpace is still needed to be added in PDCCH-ConfigCommon

[Proposed Change]: Add pagingSearchSpaceExt in PDCCH-ConfigCommon per below:

(omit)

====

This issue is related to the following RAN1 agreement.

Agreement For intra-slot PDCCH repetition of PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero,: - Specify an explicit linkage of two SS for intra-slot PDCCH repetition. That is, a RRC parameter is introduced for SS linkage and enabling intra-slot PDCCH repetition. The starting symbol of monitoring occasion of the second SS is located right after the ending symbol of monitoring occasion of the first SS. A new parameter in SIB1 e.g. searchSpaceLinkingId-r19 is introduced or Rel-17 parameter (i.e. SearchSpaceLinkingId) is re-interpreted with modification to capture RAN1 agreements.

Based on the RAN1 agreement, RAN2 is required to link two SSs for intra-slot PDCCH repetition. Several alternatives to fulfill this requirement are considered below.

- Alt 1: introduce new searchSpaceLinkingId to SearchSpaceExt IE, and the explanation of the linkage of SS for CSS is described in its field description.

  - This approach has already been proposed (E012(PropAgree)), and the association and usage are described in the field description (FD) of this new searchSpaceLinkingId. Thus, there is no need to introduce an additional IE such as pagingSearchSpaceExt.

- Alt 2: legacy searchSpaceLinkingId-r17 (for linkage of SS not for CSS) is reused, and the explanation of the linkage of SS for CSS is added in the field description

  - This solution doesn’t need introducing additional parameter but needs to add FD in the legacy parameter’s FD. In this case, association and usage is described in FD of this new searchSpaceLiningId, and additional IE (pagingSearchSpaceExt) is also not needed.

- Alt 3: introduce pagingSearchSpaceExt to SIB1

  - This alternative can directly associate two SS, and in this case, if searchSpaceLiningId is used, it is duplicated indication. So, searchSpaceLiningId is not needed for indicating association for paging SS.

After evaluating the above alternatives, Alt1 and Alt2 are both technically feasible. However, to minimize complexity in the field descriptions and avoid potential confusion from parameter reuse, we recommend adopting Alt1, which introduces a new searchSpaceLinkingId to SearchSpaceExt IE. Then, we propose that pagingSearchSpaceExt in PDCCH-ConfigCommon is not needed to introduce.

Proposal 1: (Z256) pagingSearchSpaceExt in PDCCH-ConfigCommon is not needed to introduce.

MBS

(C003) How UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned.

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C003 | NTN | 1 | clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | ToDo |

[Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope.

[Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIBXX.

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

[Comments]:

[Rapp] Even thought it is unlikely that both ways of delivering the information may be configured simultaneously, RAN2 should further discuss which one is prioritized (if needed).

[Qualcomm] Which ever area UE wants to use should work pointing to (may be not exactly same) almost same region. Otherwise this should be error case.

====

The situation intended in this issue is as follows.

- The UE receives the service area coverage specified in the USD and the frequency information for the delivered services.

- The UE receives the ISA contained in the SIBXX.

- At this time, a mismatch occurs between the area coverage specified in the USD and the ISA coverage specified in the SIB.

In such a situation, if the UE is located inside the ISA, executing the action to establish the MRB is consistent with the agreement reached at the RAN2#127 meeting.

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions. - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In TS26.502, the MBSF generates the USD, and this information is transmitted to the RAN via the Nmbstf. The RAN generates the MBS session information based on this information. That is, the information about the area intended by the content provider may not be simultaneous, but it can be said that alignment with the RAN is implicitly maintained. Therefore, the situation proposed in this issue may be a corner case.

Observation 1: Inconsistencies in area information between USD and SIBXX may be a corner case.

Information inconsistencies are likely to occur when either USD or SIBXX is received, which indicates either a failure in the NW-side functionality or a lack of alignment. Area information between USD and SIBXX has been synchronized at least once, after which SIBXX is delivered to the UE. Additionally, upon entering a cell providing MBS service, the basic operation of Rel19 UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure. So, the RRC procedures (e.g. MRB establishment procedure) should be performed according to the SIBXX ISA when inconsistencies occur between SIBXX and USD.

Observation 2: The area information between USD and SIBXX has been synchronized at least once. In addition, UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure.

Proposal 2: (C003) As the ISA is not aligned with the target service area in USD, the RRC procedures should be performed according to the SIBXX ISA.

(V206) Need code for mbs-SessionAreaList-r19 should be Need S

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V206 | NTN | 1 | Need code for mbs-SessionAreaList-r19 should be Need S | Yes, R2-250xxxxx | vivo (Stephen) |  | v005 | ToDo |

[Description]: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area. So Need S is supposed to be used for mbs-SessionAreaList-r19

[Proposed Change]: Change Need R to Need S for mbs-SessionAreaList-r19

[Comments]:

[Rapp] In our understanding, based on RAN2 previous agreements, if the ISA (ID) is not provided for a certain service, then legacy mechanisms apply (it is intended for the whole area). Usually, legacy behaviour does not need an explanation, it is only the new functionality that needs to be clarified in new parameters. This should be discussed in the next meeting.

====

This issue discusses whether the ISA list should be 'Need S' or 'Need R'.

ISA information is mainly used for MCCH acquisition procedure, MRB establishment/release procedure.  As shown in TS 38.331 below, ISA information can be used to determine whether it is necessary to acquire the MCCH. However, this requires an examination of the information contained in SIB XX. Furthermore, as this is a UE implementation, there is little need to include an explanation for 'Need S' in the feature description.

NOTE 2: It is up to UE implementation to use the cell/tracking area list and/or the Target Service Area in the USD or the ISA(s) in SIB27 to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Additionally, the MRB establishment procedure is documented separately for processes that reference the relevant ISA in the initiation procedure, and for those that do not. Therefore, no further explanation is required in the field description of SessionAreaList-r19.

5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service which is not associated with any ISA and the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in, upon entering the ISA(s) associated with a MBS broadcast service the UE is interested in. The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service, upon leaving the ISA(s) associated with an MBS broadcast service the UE is interested in.

Based on the above, there is no need to add further explanation for this parameter, and the Code can remain as “Need R”.

Proposal 3: (V206) The Need code for mbs-SessionAreaList-r19 is fine in “Need R”.

Conclusion

In this paper, we made the following observations and proposals;

Proposal 1: (Z256) pagingSearchSpaceExt in PDCCH-ConfigCommon is not needed to introduce.

Observation 1: Inconsistencies in area information between USD and SIBXX may be a corner case.

Observation 2: The area information between USD and SIBXX has been synchronized at least once. In addition, UE is to confirm the association between the SIBXX ISA and the ISA ID in MCCH before initiating the MRB establishment procedure.

Proposal 2: (C003) As the ISA is not aligned with the target service area in USD, the RRC procedures should be performed according to the SIBXX ISA.

Proposal 3: (V206) The Need code for mbs-SessionAreaList-r19 is fine in “Need R”.

Reference

- NTN Review file v022.docx