3GPP TSG-RAN WG2 #130	R2-2507635

# Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.9.4

Source:	Ericsson

Title:	Remaining open issues for store and forward

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address the last remaining open issues for Store and Forward the use of the already agreed indications (time information and barring bit), its extension to neighbour cells, and the specification impact of S&F cells de-prioritization.

# 2	Discussion

## 2.1	Specification impact of S&F cell de-prioritization

In RAN2#131, the following was agreed:

A Rel-19 UE may deprioritize the neighbour cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review).

Regarding the procedural perspective, during the post email discussion, the agreement was captured as informative procedural text in a note within the section for Cells with cell reservations, access restrictions or unsuitable for normal camping.

NOTE: If the highest ranked cell or best cell according to absolute priority reselection rules is cell operating in store and forward mode the UE may deprioritse or not consider this cell as candidate cell for cell reselection and continue considering other cells not operating in store and forward mode for cell reselection.

According to the 3GPP Drafting Rules (Annex A), "Notes are used to provide informative guidance only; they shall not contain requirements or instructions." Placing the de-prioritization behaviour in a NOTE would render it non-binding, allowing different UE implementations to handle S&F cells inconsistently, which could lead to undesirable mobility decisions and negatively impact the user experience in NTN environments.

Furthermore, cell prioritization and de-prioritization rules for idle/inactive mode are consistently defined in normative text. For example, TS 38.304 specifies mandatory rules for cell reselection based on absolute priorities, and TS 38.331 defines how a UE shall handle cells with specific restrictions. Placing the S&F cell handling rule in a NOTE would be a deviation from established specification practice for similar functionalities.

To ensure consistent UE behaviour and prevent reselection to cells where immediate service is unavailable, it is more appropriate to allow the UE to treat such cells as unsuitable and exclude them from the set of candidate cells for cell reselection.

Capture in normative text that a Rel-19 UE may exclude S&F cells as candidate cell for cell reselection.

## 2.2	Operation mode transition without cell change

Previous discussions have brought up the situation in which a satellite switches between S&F and non-S&F operation modes without a corresponding cell change, meaning that no PCI change occurs during the mode transition. This situation raises technical considerations.

### 2.2.1	System information update

In NB-IoT, the SIB1-NB modification period is statically configured to 4096 radio frames (i.e., 40.96 seconds). This duration often exceeds the remaining service time of a NGSO satellite following an operation mode transition. Consequently, if an operation mode transition occurs during the service period of a satellite, it may not be feasible to maintain service on the same cell, as the available time after the transition may be insufficient to accommodate SIB1-NB updates using existing mechanisms. Therefore, operation mode transitions are currently not optimally supported in dynamic NTN scenarios.

The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which may be longer than the remaining NGSO satellite service time after the operation mode transition.

The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

As a result, updates to parameters such as cellBarred-NTN and the “S&F operation” indication may not be aligned in real-time with satellite operation mode changes. This misalignment can result in situations where UEs that do not support S&F are not barred after the satellite transitions to S&F mode, as depicted in Figure 1. Due to the extended SIB1-NB modification period, this condition may persist for a considerable duration, thereby adversely affecting S&F incapable UEs that can’t operate in the cell and should ideally re-select prior to the transition.

Figure 1. S&F incapable UEs are not barred when the satellite transits to S&F mode

We proposed that, upon an operation mode transition, i.e., following t-ModeSwitching, the UE immediately updates and re-acquire SIB1-NB. This procedure ensures that the UE determines the cell barring status based on the latest SIB1-NB, thereby supporting timely and accurate access control.

Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the satellite operation mode transition.

### 2.2.2	Cell access upon barring due to operation mode transition

According to TS 36.304, when a UE identifies a cell as barred, it must exclude that cell from the cell reselection candidate list for 300 seconds. Depending on the intraFreqReselection parameter configuration, the UE may extend this exclusion to the entire frequency. This mechanism, while robust, presents challenges in certain geographical areas where satellite operation mode frequently transition.

A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

As an illustrative example, consider a scenario where a UE lacking support for S&F finds a cell that is currently operating in S&F mode but is scheduled to transition to non-S&F operation soon. In accordance with the existing procedure, the UE is mandated to exclude this cell (or, depending on parameter configuration, the entire frequency) from its cell reselection candidate list for a duration of 300 seconds. Given that this exclusion period is comparable to, or may even exceed, the typical service time of LEO satellites, the UE may be prevented from re-selecting to the cell even after it becomes available (i.e., the barring bit is deactivated). Consequently, this can lead to missed opportunities for connectivity and suboptimal service continuity in dynamic NTN environments.

During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode (e.g., non-S&F) and the barring bit is deactivated.

To address the identified limitation regarding prolonged cell exclusion in dynamic NTN environments, it is proposed that when a neighbouring cell is barred due to S&F operation prior to its t-ModeSwitching, the UE excludes this cell from the cell reselection candidate list only until the specified operation mode transition time. Upon expiry of the operation mode transition time, the UE can reassess the barring status based on the most recent system information. This approach aims to reduce unnecessary exclusion durations, thereby enhancing the likelihood of timely connectivity and improving both network efficiency and user experience.

The UE excludes a barred neighbouring cell due to S&F operation mode only until the operation mode transition time.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which may be longer than the remaining NGSO satellite service time after the operation mode transition.

Observation 2	The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

Observation 3	A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

Observation 4	During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode (e.g., non-S&F) and the barring bit is deactivated.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture in normative text that a Rel-19 UE may exclude S&F cells as candidate cell for cell reselection.

Proposal 2	Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the satellite operation mode transition.

Proposal 3	The UE excludes a barred neighbouring cell due to S&F operation mode only until the operation mode transition time.