3GPP TSG-RAN2#131bis	R2-2507642

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.9.3

Source:	Ericsson

Title:	MAC IoT NTN issues

Document for:	Discussion, Decision

# 1	Introduction

Here we discuss some outstanding issues for IoT NTN phase 3.

# 2	Discussion

Regarding backoff for CB-Msg3 attempts:

For efficient control of temporary peak in load on the CB-Msg3-EDT resources, it must be possible to spread failing users access attempts in the future CB-Msg3 transmission windows. The back off values must cover many multiples of CB-Msg3 periodicity.

The current backoff values are for eMTC are in MAC Table 7.2-1 and vary between 0, 10, 20, …, 960 ms.

As there can be up to 64 repetitions for eMTC CE mode A for extreme coverage – we may need longer values than 960 ms.

The current backoff values for NB-IoT are in Table 7.2-2 and vary between 0, 256, …, 524288 ms.

For NB-IoT, in extreme coverage scenarios with many repetitions and 3.75 kHz SCS, the highest values in the table seem to cover any useful multiples of the length of one CB-Msg3 transmission, yhus no strong need to update this.

However, for constructing a simple algorithm for back off and to use the same method for eMTC and NB-IoT, and to get better spreading for eMTC, it is easier if the back of is expressed as a number of CB-Msg3 transmission window periodicities. We propose:

The back off index 0 to 15 indicate a back off parameter value of 3, 6, 9,… 45, 48 multiples of the CB-Msg3-transmission window periodicity.

Regarding HARQ ACK resources:

This is a major change to RAN1 spec to add more than 4 HARQ ACK resource indications that could be used in CB-Msg4.

Adding more indications for HARQ ACK resources for eMTC is a new feature for RAN1 that require many changes in RAN1 spec. RAN1 work for this WI is now in maintenance.

RAN2 do not add more HARQ ACK resources indications for CB-Msg4 for eMTC.

Regarding CB-RNTI used for CM-Msg3:

The Msg3 is not scrambled any RNTI, also not by CB-RNTI…

RAN2 do not mention CB-RNTI in relation to the CB-Msg3.

# 3	Conclusion

In this paper we have made these observations:

Observation 1	Adding more indications for HARQ ACK resources for eMTC is a new feature for RAN1 that require many changes in RAN1 spec. RAN1 work for this WI is now in maintenance.

We propose:

Proposal 1	The back off index 0 to 15 indicate a back off parameter value of 3, 6, 9,… 45, 48 multiples of the CB-Msg3-transmission window periodicity.

Proposal 2	RAN2 do not add more HARQ ACK resources indications for CB-Msg4 for eMTC.

Proposal 3	RAN2 do not mention CB-RNTI in relation to the CB-Msg3.