3GPP TSG RAN WG2 Meeting #132         	                                   R2-2508688

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.0.2

Source:	Xiaomi, Samsung

Title:	Discussion on Per Band and Per Band Combination

Document for:	Discussion and Decision

# Introduction

Regarding the granularity of ‘per band and per band combination’, RAN2 asked RAN1 how to interpret the relationship between per band and per BC capabilities [2]:

Question 1: what is the relationship between per band and per BC capabilities i.e. what is the final capability in the following cases? - Case 1: when UE indicates both per band and per BC capability - Case 2: when per BC capability is indicated but one of the bands in the BC doesn’t indicate per band capability - Case 3: when the UE reports the per band capability but does not include the per BC capability for a certain BC Question 2: how do we define pre-requisite if pre-requisite is also per band and per BC capabilities?

Based on which, RAN1 further discussed based on Rel-19 features and replied to RAN2 in [1].

In this contribution, we discuss how to capture RAN1 understanding for Rel-19 and previous releases.

# Discussion

Based on RAN1 reply LS in [1], here we summarized the common understanding for ‘per band and per BC’.

Question 1: what is the relationship between per band and per BC capabilities i.e. what is the final capability in the following cases?

In general, features defined with ‘per band and per band combination’ have the following two categories:

      - The capability/component is not counted across CCs, e.g., maximum value of a band or a BC. This includes some examples such as: supported maximum rank, maximum number of CSI-RS resource in a resource set, a list of supported combinations of {max # of resources and total # of Tx ports} per CC, etc;

      - The capability/component is counted across CCs within a band or BC, e.g., a list of supported combinations of {max # of resources and total # of Tx ports} across all CCs.

For category 1, RAN1 replied that the minimum capability between per BC and per band is applied to a band in case of band combination. Additionally, RAN1 also mentioned that they will continue discuss the case ‘when the capability/component is not counted across CCs and CA is not configured’. However, in our understanding, the interpretation of UE capability should not be impacted by whether CA is configured or not. No matter CA is configured or not, the minimum capability between reported per band and per BC applies. For category 2, RAN2 can further wait for RAN1 feedback, as replied in [1].

Proposal 1: No matter CA is configured or not, if the capability/component is not counted across CCs, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA).

Proposal 1-1: For the capability/component is counted across CCs, wait for RAN1 for the conclusion.

For both Case 2 and Case 3, where UE only reports one of ‘per band’ or ‘per BC’, RAN1 confirmed RAN2 understanding.

Proposal 2: When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability, the capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs.

Proposal 3: When the UE reports the per band capability but does not include the per BC capability for a certain BC, the UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs.

Question 2: how do we define pre-requisite if pre-requisite is also per band and per BC capabilities?

RAN1 replied that only the same granularity is expected as prerequisite.

For per band and per BC capabilities, it is expected that UE indicates the same granularity as the pre-requisite

This means, for example, if Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’:

- A UE supporting feature A per band shall also indicate support of feature B at the given band;

- A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

Proposal 4: If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’:

- A UE supporting feature A per band shall also indicate support of feature B at the given band;

- A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

The final question is whether we can adopt the above general principles in Proposal 1/2/3/4 for all features defined by RAN1 with granularity of ‘per band and per band combination’. We observe there are some cases that cannot apply this general principle, e.g., feature with some ‘per band’ components and some ‘per BC’ components. This kind of feature need to be implemented by defining different capability with different field description.

For example, RAN1 FG 42-1 [4] spatialAdaptation-CSI-Feedback-r18 and spatialAdaptation-CSI-FeedbackPerBC-r18.

| 42-1 | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS resource IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Note: SD-type1 refers to all sub-configurations that contain one port subset Note: SD-type2 refers to all sub-configurations that contain list of CSI-RS resource IDs Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Note: Components 6 and 7 are signaled per BC | Optional with capability signaling |
| --- | --- | --- | --- | --- |

Another example is power class, as defined since Rel-15 [3]:

| 2-9 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | Yes | gNB cannot know the UE power class | Type 1 and Type 3 (see remarks column) | No Need | No Need |  | Capability signalling - FR1 UE power class ( Type 1 (per band)) - FR2 UE power class ( Type 1 (per band)) - FR1 UE power class for EN-DC (Type 3 (per band combination)) - FR1 UE power class for NR CA (Type 3 (per band combination)) Note that default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. Note that the component 2) is also used as power class for intra-band contiguous NR-CA in FR2 | RAN4 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class. | Mandatory to report non-default power class if UE supports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Furthermore, RAN1 further clarified there may be some capabilities need to be defined with clear ranking other than the general principle:

- RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19, e.g., in some cases a clear “ranking” between capabilities may need to be clarified in the Notes

For those capabilities, a NOTE can be added in field description of such specific feature.

Based on this:

Proposal 5: Define ‘per band and per band combination’ as new granularity in TS 38.306 with the following definition:

Per band and per BC capability means that A UE capability parameter defined in both per band and per BC with same feature components. The capability is signalled in per band and also signalled in per band combination.

Proposal 6: RAN2 captures the interpretation on final capability of ‘per band and per BC’ (i.e., Proposal 1/2/3) and understanding of ‘per band and per band combination’ prerequisite (i.e., Proposal 4) in Section 4.2.1 TS 38.306 as general principle for the feature with granularity ‘per band and per band combination’.

- The general principle of ‘per band and per band combination’ doesn’t apply to features with some ‘per band’ components and some ‘per BC’ components, e.g., power class, etc.

- For feature with special handling, adding a NOTE in field description.

In the end, we observe that this ‘per band and per band combination’ granularity has been used by RAN1 since Rel-16 for features with all components defined with granularity ‘per band and per band combination’ (e.g., codebook features). It is suggested to clarify the understanding since Rel-16 to avoid misalignment.

Proposal 7: The general principle of ‘per band and per band combination’ is corrected since Rel-16.

# Conclusion

Proposal 1: No matter CA is configured or not, if the capability/component is not counted across CCs, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA).

Proposal 1-1: For the capability/component is counted across CCs, wait for RAN1 for the conclusion.

Proposal 2: When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability, the capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs.

Proposal 3: When the UE reports the per band capability but does not include the per BC capability for a certain BC, the UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs.

Proposal 4: If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’:

- A UE supporting feature A per band shall also indicate support of feature B at the given band;

- A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

Proposal 5: Define ‘per band and per band combination’ as new granularity in TS 38.306 with the following definition:

Per band and per BC capability means that A UE capability parameter defined in both per band and per BC with same feature components. The capability is signalled in per band and also signalled in per band combination.

Proposal 6: RAN2 captures the interpretation on final capability of ‘per band and per BC’ (i.e., Proposal 1/2/3) and understanding of ‘per band and per band combination’ prerequisite (i.e., Proposal 4) in Section 4.2.1 TS 38.306 as general principle for the feature with granularity ‘per band and per band combination’.

- The general principle of ‘per band and per band combination’ doesn’t apply to features with some ‘per band’ components and some ‘per BC’ components, e.g., power class, etc.

- For feature with special handling, adding a NOTE in field description.

Proposal 7: The general principle of ‘per band and per band combination’ is corrected since Rel-16.

# Reference

[1] R1-2507981 LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122bis

[2] R2-2506565 LS on per band and per BC capability

[3] R4-1907593 "RAN4 NR UE features", contribution to TSG-RAN WG4 meeting #XX.

[4] R1-2501388: "Updated RAN1 UE features list for Rel-18 NR after RAN1#120".