3GPP TSG-RAN WG2 Meeting #131bis                                                                 R2-2507765

Prague, Czech Republic, October 13th – 17th, 2025

Title:	[Draft] Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

Response to:	R2-2506747/S2-2507636

Release:	Release 20

Work Item:	IoT_NTN_Ph4-Core

Source:	vivo [To be RAN2]

To:	SA2

Cc:                              SA4, CT1, SA3, SA1, RAN1

Contact Person:

- Name:	Xiaodong Yang

- E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  None

# 1. Overall Description:

RAN2 thanks SA2 for the LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC. RAN2 has discussed the relevant issues, and would like to provide the following replies:

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

RAN2 reply to Question 1: RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

RAN2 reply to Question 3: Scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

RAN2 reply to Question 6: RAN2 understands that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

RAN2 reply to Question 9 and 10: RAN2 understands it is technically feasible to introduce SRB with RLC UM for voice packet.

RAN2 also discussed the CP and UP solution for the support of IMS voice over NB-IoT NTN. Majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

# 2. Actions:

To SA2 group:

ACTION: RAN2 respectfully asks SA2 to take the reply and information above into consideration.

# 3. Date of Next RAN2 Meetings:

RAN2#132		17th to 21st November 2025			Dallas, US