3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507770

Prague, Czech Republic, 13th - 17th October 2025

Agenda item:	9.7.1

Source:	Qualcomm Incorporated

Title:	Report on [305][IoT NTN Ph4] Reply LS to SA4

Document for:	Discussion and Decision

- Introduction

This document provides the summary of the following offline discussion.

[AT131bis][305][IoT NTN Ph4] Reply LS to SA4 (QC)

Scope: Discuss the content of a reply LS to SA4

Intended outcome: Draft LS (in R2-2507766)

F2F time: FFS

Deadline for offline discussion summary: Wednesday 2025-10-15 17:00

- Discussion

## SA4 LS [1]:

3GPP SA4 has an ongoing feasibility study on Ultra Low Bitrate Speech Codec (FS_ULBC) based on the approved study item description in SP-250378 including the following objective:

- Study GEO channel characteristics and derive service-related dependencies, e.g. bitrates, mouth-to-ear delay or loss/delay/jitter profiles.

Followings are the actions:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

For CP/UP down selection/AS layer optimization:

Since RAN2 and SA2 would need coordination to make decision, rapporteur suggests informing that no conclusion has reached for down selection of CP/UP solutions and any AS layer optimization.

Proposal 3: RAN2 responds to SA4 that RAN2 is working on the down-selection of solutions and has not yet concluded on the option.

Continue in offline 305

Add in SA4 response that no conclusion has been reached yet for down selection of CP/UP and any AS layer optimization (may be updated based on online discussion).

Comments:

E/// wants to clarify the decision is not up to RAN2 to decide specially on non-IP, RTP/UDP/IP. Huawei agrees.

ZTE disagrees. CATT wonders if we will have discussion on down section and update.

MAC overhead

Rapporteur thinks RAN2 also needs consider the TBS size design considered by SA4 for low codec rate for voice over NB-IoT over GEO channel. Lager the TBS size, more problematic over the air for NB-IoT UL transmission in half duplex mode. So lets consider the TBS size of 144 bits.

Table 1 TBS and PHY bitrate for 80ms bundling [3]

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.1 (88 bits) | 2.5 (200 bits) | 3.4 (272 bits) | 4.6 (368 bits) |

The TBS size design includes 2 bytes of MAC header. If 2 bytes of MAC header in the TBS size is not realistic, then this will impact the TBS size design.

Table 1 Likelihood of total MAC header in a TBS size

| TBS size (available UL grant size) | Single voice packet size (total RLC PDU) | Total MAC header | Likelihood |
| --- | --- | --- | --- |
| X byte | (X-1) byte | 1 byte Fields E = 0, L absent, no padding | low |
| X byte | (X-2) byte | 2 byte 1 byte Padding + field E = 0 | low |
| X byte | < (X-2) byte | > 2 byte Fields E = 1, L present, + E =0 | High |

Proposal 1: RAN2 responds to SA4 that a packet overhead of 1 byte of MAC header for voice packets is not realistic based on the current MAC spec.

-	QC thinks also 2 bytes is not realistic.

-	Ericsson thinks this depends on the TBS

Rapporteur suggests:

- #1: Indicate to SA4 that in most likely case, both 1 byte and 2 bytes of MAC headers are not realistic, and header size will be 3 bytes or more.

- #2: 1 byte MAC header is possible in some scenario where TBS size fits one UL data without padding but in common scenarios header size will be 3 bytes or more.

Comments:

Samsung thinks 1 byte might be realistic in some scenario.

ZTE thinks 2 byte will be highly possible.

Nokia prefers to say 1 byte is not realistic even if it is possible. And not talk about 2 byte as it was not asked.

RAN2 assumes either 1 byte or 2 byte or 3 byte MAC header is possible depending on whether the TB size matches with the grant. However, in most common scenario, total MAC header size is likely to be 3 bytes.

RoHC header:

When RoHC is used for RTP/UDP/IP traffic, there can be different modes of operation and different types of RoHC headers used. The commonly used mode is the Bidirectional optimistic mode, i.e., O-mode. In this mode, commonly used headers are UO-0 (1 byte for voice packet in SO state), UO-1 (2 byte for SID packet in SO/FO state) and UOR-2 type header (8 to 13 bytes) to synchronize with decompressor at FO state.

We observe that the silent period is very common during voice call, which means it is very common to use UO-1 (of size 2 bytes) header for mainly two reasons, first to adjust the inter arrival time without changing TS_STRIDE and second to mark the beginning of the talk spurt.

UP solution:

RoHC header: 3 byte minimum and 4 byte considering silent to talk transition

PDCP header: 1 byte

RLC header: 1 byte

MAC header: 1 or 2 byte only in some conditions but mostly 3 byte or more.

CP solution:

RRC header: 2 byte

RLC header: 1 byte

MAC header: 1 or 2 byte only in some conditions but mostly 3 byte or more.

Proposal 2: RAN2 responds to SA4 with the L2 protocol header size for service data based on the current specifications for NB-IoT.

Continue in offline 305

- Indicate to SA4 that minimum RoHC header size is 3 bytes but due to frequent talk spurts, RoHC header size of 4 byte will be used frequently.==> discuss online.

For PDCP and RLC in UP solution and for RRC and RLC in CP solution, indicate header size according to the current specification for NB-IoT (i.e., 1 byte for PDCP, 1 byte for RLC UM, 2 byte for RRC, and 2 byte for RLC AM).

Discuss whether to indicate to SA4 that minimum RoHC header size is 3 bytes but due to frequent talk spurts, RoHC header size of 4 byte will be used frequently.

## SA4 LS [2]

RAN2 also received second LS from SA4 with the following action.

ACTION:

SA4 kindly asks RAN2 to confirm whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, and provide feedback, if any.

Once initialized at SFN start and Subframe start, the SPS occasion occurs periodically. If the SPS periodicity divides the 10240, then every H-SFN the SPS occasion pattern remains same, i.e., if SPS occasion is SFN 1, subframe 1 in H-SFN 1, then every H-SFN after SFN wraparound, the SPS occasion falls on SFN 1, Subframe 1. This helps network to manage resource efficiently avoiding any potential collisions.

If the SPS periodicity does not divide the 10240, then the SPS occasion pattern changes every SFN wrap around as shown in Figure 1.

Figure 1 An example of SPS occasion pattern change with 120ms of SPS periodicity

Such periodicities like 120ms and 240ms add complexity to UE and Network to maintain the start frame and number of SPS occasions after SFN wrap around until it is released. After SFN wrap around, the SPS occasion may overlap with other scheduling such as SIB1 and PSS/SSS which results in postponement of the SPS occasion. This may cause collision of UL SPS occasion among multiple UEs. Therefore, we think periodicities such as 120ms and 240ms should be avoided.

Indicate to SA4 that RAN2 has not started work on SPS for voice but SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 may need to be avoided for NB-IoT.

- Conclusion

Following proposals are made:

Proposal 1	Add in SA4 response that no conclusion has been reached yet for down selection of CP/UP and any AS layer optimization (may be updated based on online discussion).

Proposal 2	RAN2 assumes either 1 byte or 2 byte or 3 byte MAC header is possible depending on whether the TB size matches with the grant. However, in most common scenario, total MAC header size is likely to be 3 bytes.

Proposal 3	For PDCP and RLC in UP solution and for RRC and RLC in CP solution, indicate header size according to the current specification for NB-IoT (i.e., 1 byte for PDCP, 1 byte for RLC UM, 2 byte for RRC, and 2 byte for RLC AM).

Proposal 4	Discuss whether to indicate to SA4 that minimum RoHC header size is 3 bytes but due to frequent talk spurts, RoHC header size of 4 byte will be used frequently.

Proposal 5	Indicate to SA4 that RAN2 has not started work on SPS for voice but SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 may need to be avoided for NB-IoT.

- References

[1] R2-2505068, “LS on the RAN simulation assumptions for ULBC”, Bengaluru, India, 25th - 29thAugust 2025.

[2] R2-2506754, “LS on bundling period and SPS for ULBC”, Prague, Czech Republic, 13th - 17th October 2025.

[3] S4-251585, “Permanent Document FS_ULBC”, Version 0.2.0.