3GPP TSG-RAN2 Meeting #131bis			R2-2508832

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.4

Title:	Discussion on 6G mobility

Source:	CMCC

Document for:	Discussion

# 1	Introduction

RAN Plenary#108 agreed the RAN Working Group SID “Study on 6G Radio” in [1], including an objective related to mobility aspects:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Following are the agreements made in last RAN2 meeting[2]:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

This paper is to provide our consideration on mobility in 6G.

# 2	Discussion

2.1	Intra-RAT mobility

2.2.1	Mobility in RRC_CONNECTED

In last RAN2 meeting, multiple mobility solutions like L3 based HO, LTM and conditional mobility were discussed and it is agreed to further study. Since both low frequency like 2.6GHz and high frequency such as 7GHz may be used in 6G based on the 6G discussion, and there’re coverage/data rate differences among different frequency spectrums, we think the mobility solutions discussion should consider different scenarios.

Low Frequency scenario:

As we mentioned in [3], the L3 based handover success probability is almost 99% on FR1 in 5G, which can ensure UE’s experience very well. Therefore, it can be seen as a candidate solution in 6G low frequency scenario, especially considering that 5G spectrum may be refarmed to 6G. When it comes to LTM, though LTM can achieve cell switch with less interruption time, resource reservation in the network side is required as a trade-off . In low frequency scenario, since the coverage of the cell is quite large and it’s hard to predict UE’s mobility, then it increases the difficulty to configure suitable LTM candidate cells to the UE. Even if LTM candidate(s) is configured to the UE, due to the large coverage, the overhead for resource reservation will also be enlarged. From this point of view, LTM is not suitable to mobility in low frequency scenario. Therefore, in low frequency scenario, we think L3 based HO should be the baseline mobility solution.

Observation 1: L3 based HO can achieve high mobility success probability with less complexity and  less overhead in network side, in low frequency scenario.

High Frequency scenario:

When it comes to high frequency scenario, the coverage of the cell becomes small and this may result in UE’s frequent mobility, accompanied with service interruption, while the mobility design in 6G is committed to minimizing interruption time, maintaining service continuity and guaranteeing throughput. In our view the interruption in mobility procedure is mainly comes from UE’s reconfiguration and protocol reset/re-establishment. Even to LTM, in some case, in which L2/L3 handling can be avoided, the minimum interruption is almost 13ms, since UE’s RRC processing time cannot be avoided.

Observation 2: RRC processing time is the main component of mobility interruption time.

Therefore, the following two directions to reduce interruption can be considered:

- Cell-free mobility

- Mobility procedure optimization

- Cell-free mobility

The cell-free mobility here means that in a specific area, the UE’s mobility doesn’t bring RRC reconfiguration, especially no high layer configuration change, while only physical layer configuration changes. Taking Figure 1 as an example, that when UE moves from Location 1 to Location 2, UE’s serving TRPs/beams are changed from the yellow beams to green beams based on UE’s L1 measurement results, while high layer configuration is maintained.

Figure 1: Cell-free mobility example

Cell-free mobility relies on the cell-free deployment, which is a different alternative, compared to cell-based deployment, as depicted in Figure 2. The specific area is called cell-free area(CFA), which consists of multiple cells/TRPs. From deployment perspective, all cells/TRPs within a CFA may connect to one BBU, which can be regarded as ideal backhaul (i.e., ideal delay/frequency/phase synchronization). Typical deployment scenarios include hotspot, campus, factory, etc.

Figure 2: Cell-Free deployment

With Cell-free mobility, not only the interruption time can be avoided or reduced since RRC processing in the subsequent mobility is avoided, but also UE’s throughput can be insured since multi TRPs can be configured to the UE simultaneously or subsequently.

Observation 3: Cell-free mobility can avoid or reduce UE’s interruption time since the same RRC reconfiguration is used in a specific area and it can make sure UE’s service continuity and throughput via using multi-TRP. And the typical deployment scenarios include hotspot, campus, factory, etc.

- Mobility procedure optimization

Though Cell-free based mobility can achieve a good mobility performance, it is more suitable to hotspot scenario in a limited area, therefore we still need to optimize mobility procedure to reduce potential interruption for UE’s mobility between two CFA or mobility between two normal cells (cells is not configured with CFA).

Based on our commercial network data, the minimum L3 based HO latency is almost 24 ms, while for LTM, it can be minimized to almost 13ms and Figure 3 is an example for LTM interruption time calculating. Therefore, solutions to achieve interruption reduction in LTM can be taken as the baseline for 6G mobility in high frequency, including early DL/UL synchronization, pre-configurations and early CSI acquisition.

Observation 4: Early DL/UL synchronization, pre-configurations and early CSI acquisition can be inherited in 6G for interruption reduction.

Figure 3: Components of Mobility Latency for LTM

Each component of mobility latency is described in the table below. The value of TRRC is specified in clause 12 of TS 38.331 [4].

Table 1: Components of Mobility Latency

| Component | Meaning |
| --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations |
| TMAC | Time for processing the LTM cell switch command MAC CE |
| TRRC-target | Time for processing and applying the target configuration |
| TDL-sync | Time for DL synchronization |
| TRA | Time until RA transmission |
| TRAR | Time until RAR reception |
| Tfirst-data | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR |

As we mentioned in the previous part, the main interruption time of LTM comes from UE’s RRC processing, that UE only process the related target configuration once it receives the LTM cell switch command. However, as all LTM candidate configurations are pre-configured to the UE, and the purpose of LTM is to avoid unnecessary RRC signalling, that’s to say, it can be assumed that the RRC configuration will not be changed (frequently) from the time of being configured to the time of cell switch. Therefore, we prefer the UE to perform RRC processing once it receives LTM configuration.

Observation 5: The LTM interruption time can be reduced if the UE can perform RRC processing once it receives LTM configuration rather than perform RRC processing when it receives LTM cell switch command.

High speed mobility scenario:

The above discussion does not take UE’s mobility speed into consideration, while for high-speed UE, the robustness is the most important issue. In [3], we included CMCC’s field test on CHO for high-speed scenario, based on the trial results, CHO can avoid the deterioration of UE experience and ensure handover successful probability, therefore, the conditional mobility should be supported in 6G, and both L1 based and L3 based should be considered.

Observation 6: Conditional mobility can avoid the deterioration of UE experience and handover successful probability, which can be considered in 6G for high-speed scenario.

Based on the discussion above, we propose as following for different mobility scenarios:

Proposal 1: For low frequency scenario, L3 based mobility is taken as baseline for 6G;

For high frequency scenario, the following directions can be considered for mobility interruption reduction:

- Cell-free based mobility that UE’s mobility would not bring RRC reconfiguration and multi-TRPs can be configured to the UE within CFA;

- Mobility procedure optimization, including early DL/UL synchronization, pre-configurations and early CSI acquisition and UE pre-processing RRC configurations when it receives;

For high-speed scenario, L1/L3 based conditional mobility is supported to insure mobility robustness.

Based on the discussion above, in different scenario, different solutions including configuration, measurement and triggering entity are used. To avoid function and configuration redundancy, unified design should be considered for 6G mobility considering the following aspects:

Measurement:

The 5G NR measurement framework is highly capable, its evolutionary path exhibits a distinct "patchwork" characteristic, leading to issues such as fragmented configurations, disjointed mechanisms, and signaling redundancy. To construct a streamlined, efficient, and future-proof 6G system, an unified redesign of the measurement framework is imperative.

The fragmentation of the current 5G NR measurement framework limits its scalability and efficiency for future 6G scenarios:

- Measurement Resource/Reporting Configuration:

  - Separation between RRM and CSI: RRM measurements for mobility decisions and CSI measurements for link adaptation are decoupled at the RRC signaling level, increasing signaling overhead and UE measurement burden.

  - Inconsistent Event Definitions: Traditional L3 handover relies on cell-level measurement events (e.g., A3/A5), while R19 introduced finer-grained beam-level events (e.g., LTM2/LTM3) for LTM, alongside specialized configurations for scenarios such as NTN and UAV. The coexistence of these event definitions at varying granularities across disparate procedures increases network-side decision complexity and UE implementation challenges.

  - Limited extensibility: To support new features such as LTM, the protocol introduced independent L1 measurement configurations, resulting in fragmented configuration management. For emerging scenarios like NTN and UAV, their unique measurement requirements are also challenging to integrate elegantly into the existing framework.

- Fragmented Measurement Reporting:

  - Multiple Reporting Ways: Conventional HO/CHO measurement reports are transmitted via RRC messages (SRB bearers), which offer high reliability but introduce significant latency. To minimize delay, LTM introduces reporting mechanisms based on MAC CE and UCI (PUCCH/PUSCH), while CSI reporting adheres to an independent and highly complex set of UCI carrying rules.

Mobility configuration:

Along with the mobility discussion, the signaling structure of mobility configuration evolves from supporting only one single target configuration to multiple candidate configurations, and further to support reference configuration with delta configuration(s). The evolution enables the signaling structure to support multiple candidate cells’ configurations and subsequent mobility with low signaling overhead. And there’s similarity among different mobility mechanism’s configuration, for example, configuration provided by target(candidate) cell(s) and execution conditions for UE triggered mobility, and for a specific candidate cell, the related configuration/parameters for different mobility mechanism may be identical, while current separate configuration results in redundancy in RRC reconfiguration. Therefore, a unified mobility configuration framework can be considered in 6G.

LTM signaling structure can be seen as a typical example, in which both candidate configuration and reference configuration are used. Also, it can be associated with execution conditions to achieve UE triggered conditional cell switch. What’s more, it can support functions like early synchronization with candidate cell(s), simplified layer 2 handling and subsequent mobility, which are not supported by HO or CHO. Therefore, it’s better to harmonized LTM signaling structure to support (C)HO configuration, especially if there’s a unified measurement framework in 6G.

Besides, the procedures to accelerate cell switch among multiple cells and simplify UE behavior, including early synchronization and simplified layer2 handling in LTM can also be supported in 6G for the unified mobility mechanism.

Proposal 2: The following aspects should be considered to achieve a unified design for 6G mobility:

- Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework;

- Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

2.2.2	Mobility in RRC_IDLE

For UE in RRC_IDLE state, cell selection/reselection procedure is used to help UE to find a suitable cell for camping. In 5G, the UE scans all RF channels in the NR bands according to its capability to find a suitable cell and on the one it camps, it can acquire SIB1 and monitor paging.

Based on the discussion on 6G, multi carrier for RRC_IDLE/INACTIVE UE, such as hyper cell (single cell associated with multi carrier) is proposed to balance the network loading and for network energy saving, as discussed in our contribution  [5]. For this case, though multi carrier is associated with one cell( or used for different procedures), there could be no CD-SSB ( no SSB or only NCD SSB) is broadcasted in non-anchor cell, therefore, such non-anchor carriers may not be selected or camped by the UE, that’s to say, the 5G procedure can also be applied to both single cell associated with one carrier and multi-carrier cases.

Figure 4: an example of hyper cell

Proposal 3: For mobility in RRC_IDLE/INACTIVE, i.e., 5G cell selection and reselection, can be inherited in 6G, and they can be applied to both single cell associate with one carrier and single cell with multiple carriers (hyper cell) cases.

What’s more, like the discussion on RRC_CONNECTED mobility, high-speed scenario should also be considered for RRC_IDLE/INACTIVE UEs.

In 5G, priorities can be provided to UE’s cell reselection, and this applies to most scenarios. However, when it comes to high speed train scenario, the normal network is not suitable for serving UEs on the high speed train, due to frequently handover. The main purpose of deploying a high-speed-railway dedicated network (HSDN) is to provide better radio connectivity and dedicated optimizations for UEs on the high speed train, which will camp on the dedicated network and therefore may switch to RRC-CONNECTED and initiate service in time. As shown in Figure 5, one of the optimizations for HSDN is that, the HSDN cells are deployed alongside the railway, in which around 8~12 RRUs are concatenated in line and work as only one single cell in order to reduce the frequency of handover and cell reselection, and lower the signalling overhead at the same time. In our deployment, HSDN cells and normal cells can be deployed either on the same frequency or different frequency by shifting the SSB center frequency.

HSDN cells always has higher TX power than normal cells in order to compensate the penetration loss of the train body. However, we found a problem in the real network, where the railway and HSDN which are deployed in the dense population urban area, a large amount of low mobility state UEs near the railway which are not on the train may reselect to the HSDN cell and perform random access to HSDN cell, due to the high TX power of the HSDN cell. When the train is coming, there will be less resources available on the HSDN cell to accept the HO request of the UEs on the train. The issue of low mobility state UE camping on the HSDN cells will frequently cause congestion of the HSDN cells, which is not desired from the network operation point of view. In real network, we observed that such issue results to call drop, low bit rate, handover rejection, RACH failure for the high mobility state UE.

Figure 5: Overlapped coverage between HSDN cells (with Multiple RRUs concatenated as one single cell) and normal cells

In LTE and NR, the same mechanism was introduced as HSDN solution, including providing 1) information for UE’s mobility state estimation; 2) indication whether the serving cell is a HSDN cell; 3) neighboring HSDN cell list. Based on the information provided by the network, the UE can estimate its mobility state and consider the right cells as highest priority. The UE behavior is captured in TS 38.304: when the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e. higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e. lower than network configured priorities).

HSDN related solution was not introduced in the first release of LTE and NR, which impacts the commercial deployment. Considering high speed train railway network is constructed widely in the world, especially in China, we propose to support HSDN solution from 6G day1, that the network provides information for UE’s mobility state estimation and for HSDN cell judgement.

Proposal 4: For high speed train scenario, it is proposed to reuse NR HSDN solutions from 6G day1, in which the network provides information for UE’s mobility state estimation and for HSDN cell priority handling.

  - Inter-RAT mobility

In 5G, CMCC has deployed 2.59 million base stations on FR1. Considering the deployment costs of 5G and 5G’s wide coverage, 5G network will exist for a long time, co-existing with 6G in the future. Therefore, from our point of view, 5G and 6G inter-RAT mobility should be supported.

For RRC_CONNECTED state UE, it should be noticed that different from 4G in which only cell level (L3 based) measurement is supported, 5G supports both beam level (L1 based) and cell level (L3 based) measurement, therefore, both L1 and L3 based inter-RAT handover/cell switch can be supported. Besides, re-direction is the other way of inter-RAT mobility for RRC_CONNECTED UEs, in which UE will be indicated with a frequency in RRCRelease message, and UE will reselect to a cell of this frequency after it is released to RRC_IDLE state. In our real 5G network, the mobility of RRC_CONNECTED UE from 5G to 4G is via handover, while from mobility 4G to 5G is via re-direction, that’s because in this way, the impact to 4G network can be minimized, for example, the 4G network only need to configure 5G frequency to UE, but not need to configure 5G related inter-RAT measurement.

As to RRC_IDLE state UE, similar to 5G, inter-RAT frequency cell re-selection information can be provided to UE to assist UE’s cell reselection procedure.

Proposal 5: 5G and 6G inter-RAT mobility is supported for both RRC_CONNECTED and RRC_IDLE state UE:

- For RRC_CONNECTED state, both handover and re-direction can be supported for inter-RAT mobility, and both L1 based and L3 based handover/cell switch are considered;

- For RRC_IDLE state, inter-RAT cell reselection information can be provided to UE.

# 3	Conclusion

In this contribution, we provide our view on the mobility solutions supported and potential optimization in 6G and considerations on different mobility scenarios. Following are the observations and proposals:

Observation 1: L3 based HO can achieve high mobility success probability with less complexity and overhead in network side in low frequency scenario.

Observation 2: RRC processing time is the main component of mobility interruption time.

Observation 3: Cell-free mobility can avoid or reduce UE’s interruption since the same RRC reconfiguration is used in a specific area and it can make sure UE’s service continuity and throughput via using multi-TRP. And the typical deployment scenarios include hotspot, campus, factory, etc.

Observation 4: Early DL/UL synchronization, pre-configurations and early CSI acquisition can be inherited in 6G for interruption reduction.

Observation 5: The LTM interruption time can be reduced if the UE can perform RRC processing once it receives LTM configuration rather than perform RRC processing when it receives LTM cell switch command.

Observation 6: Conditional mobility can avoid the deterioration of UE experience and handover successful probability, which can be considered in 6G for high-speed scenario.

Proposal 1: For low frequency scenario, L3 based mobility is taken as baseline for 6G;

For high frequency scenario, the following directions can be considered for mobility interruption reduction:

- Cell-free based mobility that UE’s mobility would not bring RRC reconfiguration and multi-TRPs can be configured to the UE within CFA;

- Mobility procedure optimization, including early DL/UL synchronization, pre-configurations and early CSI acquisition and UE pre-processing RRC configurations when it receives;

For high-speed scenario, L1/L3 based conditional mobility is supported to insure mobility robustness.

Proposal 2: The following aspects should be considered to achieve a unified design for 6G mobility:

- Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework;

- Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

Proposal 3: For mobility in RRC_IDLE/INACTIVE, i.e., 5G cell selection and reselection, can be inherited in 6G, and they can be applied to both single cell associate with one carrier and single cell with multiple carriers (hyper cell) cases.

Proposal 4: For high speed train scenario, it is proposed to reuse NR HSDN solutions from 6G day1, in which the network provides information for UE’s mobility state estimation and for HSDN cell priority handling.

Proposal 5: 5G and 6G inter-RAT mobility is supported for both RRC_CONNECTED and RRC_IDLE state UE:

- For RRC_CONNECTED state, both handover and re-direction can be supported for inter-RAT mobility, and both L1 based and L3 based handover/cell switch are considered;

- For RRC_IDLE state, inter-RAT cell reselection information can be provided to UE.

# 4	References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO;

[2] RAN2#131bis chairnotes;

[3] R2-2506899,Consideration on the mobility in 6GR, CMCC, FS_6G_Radio;

[4] TS 38.331, Radio Resource Control (RRC) protocol specification; Stage 2 (Release 18);

[5] R2-2508835, Discussion on 6G initial access and spectrum aggregation, CMCC, FS_6G_Radio.