3GPP TSG-RAN WG2 Meeting #132		R2-2509177

Dallas, USA, Nov. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to R19 LP-WUS UE Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RAN2 agreed in RAN2-132 that RAN2 specify one capability for minimumTimeGap for both OOK and OFDM. Update capabilities as per this agreement. Add the table for values of minimumTimeGap-r19 in each SSB periodicity. - Align terminology on “RRM relaxation and offloading” among different specs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define only one capability for minimumTimeGap for both OOK and OFDM. - Added reference for “Relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements” capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE capabilities for minimumTimeGap are not aligned with RAN2 agreement - It is unclear what the “relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements” means if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| airToGroundNetwork-r18 Indicates whether the UE supports air to ground network access. If the UE indicates this capability the UE shall support the following ATG essential features, e.g., acquiring ATG cell specific SIB22 and ATG cell specific P-Max. | UE | No | No | FR1 only |
| crossCarrierSchedulingConfigurationRelease-r17 Indicates whether the UE supports using crossCarrierSchedulingConfigRelease to release the configurations configured by crossCarrierSchedulingConfig. | UE | No | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| dl-DedicatedMessageSegmentation-r16 Indicates whether the UE supports reception of segmented DL RRC messages. | UE | No | No | No |
| drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| drx-PreferenceCellDTX-DRX-r19 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving and its preference on cell DTX/DRX related parameters for PCell in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| gNB-SideRTT-BasedPDC-r17 Indicates whether the UE supports gNB-side RTT-based PDC, as specified in TS 38.300 [28]. A UE supporting this feature shall also support rtt-BasedPDC-CSI-RS-ForTracking-r17 and/or rtt-BasedPDC-PRS-r17. | UE | No | No | No |
| hardSatelliteSwitchResyncNTN-r18 Indicates whether UE supports hard satellite switch with re-sync, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. When UE supports this feature and does not support softSatelliteSwitchResyncNTN-r18, this UE is able to perform hard satellite switch with re-sync in a network supporting soft satellite switch with re-sync, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_INACTIVE as specified in TS 38.331 [9]. This capability is not applicable to NCR-MT. | UE | Yes | No | No |
| inactiveStateNTN-r17 Indicates whether the UE supports RRC_INACTIVE in NTN as specified in TS 38.331 [9]. It is mandated if the UE indicates the support of nonTerrestrialNetwork-r17. | UE | CY | No | No |  |
| inactiveStatePO-Determination-r17 Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state. | UE | No | No | No |
| inDeviceCoexInd-r16 Indicates whether the UE supports reporting of affected NR carrier frequencies in IDC assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| inDeviceCoexIndAutonomousDenial-r18 Indicates whether the UE supports IDC autonomous denial as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndFDM-r18 Indicates whether the UE supports reporting of affected NR carrier frequency ranges in IDC assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndTDM-r18 Indicates whether the UE supports reporting of IDC TDM assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| lpwus-SupportedBandList-r19 Indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode for a list of frequency bands. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of LP-WUS operation for the frequency band. The UE supporting this feature shall also indicate the support of at least lpwus-OOK-r19 or lpwus-OFDM-r19. The UE supporting this feature shall also indicate the support of at least lpwus-OOK-r19 or lpwus-OFDM-r19. For each supported band, the capability signalling comprises of the following parameters: - supportedBandIndicator-r19 indicates the frequency bands where UE supports LP-WUS operation in IDLE/INACTIVE mode; - lpwus-OOK-r19 indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode based on OOK signal. The UE indicating this feature supports LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal, LP-SS based RRM measurement, all M values {1, 2, 4} for FR1 for LP-WUS, M value 1 for 120 kHz SCS FR2 for LP-WUS and all M values {1, 2, 4} for LP-SS; - lpwus-OFDM-r19 indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence. The UE indicating this feature supports LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence, SSB-based RRM measurement, all M values {1, 2, 4} for FR1 for LP-WUS, M value 1 for 120 kHz SCS FR2 for LP-WUS; - lpwus-LP-SS-r19 indicates whether the UE supports LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured. The UE indicating this feature also supports all M values {1,2,4} for LP-SS. A UE supporting lpwus-LP-SS-r19 shall also indicate support of lpwus-OFDM-r19; NOTE: If LP-SS overlaid sequence is configured, and if both SSB based and LP-SS based thresholds are configured for RRM measurement, it is up to UE implementation which threshold to use. - minimumTimeGap-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in below table: SSB periodicity (ms); cap1 (ms); cap2 (ms); cap3 (ms). 5/10/20; 70; 500; 900. 40; 130; 600; 1000. 80; 250; 800; 1200. 160; 490; 1200; 1600. | UE | No | No | No |
| maxBW-Preference-r16, maxBW-Preference-r17 Indicates whether the UE supports providing its preference of a cell group on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes (Incl FR2-2 DIFF) |
| maxCC-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |

# 6	Conditionally mandatory features without UE radio access capability parameters

| Features | Condition |
| --- | --- |
| Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList | It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9]. |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |
| AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE | It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports" |
| ATG specific P-max | It is mandatory to support the ATG specific P-max configured by network for UEs supporting airToGroundNetwork-r18. |
| Downlink SDAP header | Either NAS reflective QoS or as-ReflectiveQoS is supported. |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |
| IMS emergency call | It is mandatory to support IMS emergency call over PLMN for UEs which are IMS voice capable in NR. It is mandatory to support IMS emergency call over SNPN for UEs that are SNPN capable and IMS voice capable over SNPNs. |
| Logged measurements suspension due to IDC interference | It is mandatory to support Logged measurements suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network and in-device coexistence indication as specified in TS 38.331 [9]. |
| MAC subheaders with LX field | It is mandatory to support MAC subheaders with LX field for UEs supporting MAC SDU(s) using the LCID value(s) as specified in Table 6.2.1-2c in TS 38.321 [8]. |
| MAC subheaders with one-octet eLCID field | It is mandatory to support MAC subheaders with one-octet eLCID field for UEs/IAB-MTs/NCR-MTs supporting MAC CEs using extended LCID values as specified in TS 38.321 [8]. |
| Paging cause in RAN paging message | It is mandatory for a UE to support paging cause in RAN paging if UE supports paging cause in CN paging. |
| Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | It is mandatory for a UE supporting NR sidelink in shared spectrum and when shared spectrum channel access must be used to support receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol and monitor a total up to the number reported in pscch-RxSidelink-r16 of PSCCHs in a slot in the 1st and 2nd starting symbols. A UE supporting this feature shall indicate support of sl-Reception-r16. |
| Receiving UE to UE COT sharing information | It is mandatory for a UE supporting NR SL in shared spectrum where shared spectrum channel access must be used to support monitoring SCI to read COT sharing information and transmitting NR SL based on COT sharing information subject to COT sharing conditions. A UE supporting this feature shall indicate support of sl-DynamicChannelAccess-r18. |
| SON report in PNI-NPN | It is mandatory for a UE to support a SON report in PNI-NPN if UE supports PNI-NPN and supports the SON report in PLMN. |
| Skipping UL configured grant if no data to transmit, as specified in release-15 version of TS 38.321 [8]. | Either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 or configuredUL-GrantType2 or configuredUL-GrantType2-v1650 is supported. |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 or uplinkTA-ReportingATG-r18 as specified in TS 38.321 [8]. |
| Inter-frequency configuration for less than 5MHz in SIB4 | It is mandatory to support configuration of dl-CarrierFreq-r18 and frequencyBandList-r18 as specified in TS 38.331 [9] for UEs supporting support5MHz-ChannelBW-20PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18 or support3MHz-ChannelBW-Asymmetric-r18. |
| Relaxation of serving cell and further relaxation of neighboring cell RRM measurements and offloading of serving cell RRM measurements | It is mandatory to support relaxation of serving cell and neighboring cell RRM measurements as specified in clause 5.2.4.12.1 in TS 38.304 [21] and offloading of serving cell RRM measurements as specified in clause 5.2.4.12.3 in TS 38.304 [21] if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21]. A UE supporting this feature shall also indicate the support at least one of lpwus-OOK-r19 and lpwus-OFDM-r19. |

END OF CHANGE