3GPP TSG RAN WG2#131bis	R2-2506714

Prague, Czech Republic, 13th - 17th October 2025

3GPP TSG RAN WG1 #122			R1-2506535

Bengaluru, India, Aug 25th – 29th, 2025

# Title:	LS on updated text proposal for 36.300 for IoT NTN TDD mode

# Release:	Release 19

# Work Item:	IoT_NTN_TDD

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

# 1. Overall Description:

RAN1 has agreed to an updated TP for 36.300 for IoT NTN TDD mode. The change is to correctly capture that there are two guard periods every N radio frames: one before the uplink subframes and one after the uplink subframes. Also, it is clarified that the frame structure is defined at the ULSRP.

-------------------- Start of TP for CR of 36.300 -------------------- *** Unchanged parts are omitted *** ### 5.0 Frame structures and channels Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported: - Type 1, applicable to FDD; - Type 2, applicable to TDD; … For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard periods (GP) at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [6]. This pattern is repeated every N radio frames. IoT-NTN TDD mode is applicable for the IoT-NTN TDD band (1616-1626.5 MHz) specified in [36.102]. *** Unchanged parts are omitted *** -------------------- End of TP for CR of 36.300 --------------------

# 2. Actions:

# To RAN2:

ACTION:

RAN1 requests RAN2 to take the above information into account.

# 3. Dates of Next RAN1 Meetings:

RAN1#122-bis	Oct 13 – 17, 2025	Prague, CZ

RAN1#123	Nov 17 – 21, 2025	Dallas, US