3GPP TSG RAN WG2#131bis	R2-2506738

Prague, Czech Republic, 13th - 17th October 2025

3GPP TSG-RAN WG4 Meeting #116	R4-2512232

Bengaluru, India, 19th – 23rd May 2025

Title:	LS on event triggered L1-RSRP reporting if eventDetectionTimeWindowLength-r19 is configured

Response to:

Release:	Release 19

Work Item:	NR_MIMO_Ph5-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:	Stefan Brueck

E-mail Address:	sbrueck@qti.qualcomm.com

- Overall Description:

RAN4 has made the following agreement on event-triggered measurement reporting delay on event triggered L1-RSRP reporting if eventDetectionTimeWindowLength-r19 is configured:

Agreement: For the window-based solution: if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel - eventInstanceCount-r19 is configured by NW - L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19). - TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

These agreements will be specified in TS 38.133 sections 9.5 and 9.13.

2. Actions:

RAN4 respectfully asks RAN1 to take the above agreements into account.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #116bis	October 13 – October 17, 2025	Prague, CZ

RAN WG4 Meeting #117	November 17 – November 21, 2025	Dallas, US