3GPP TSG RAN WG2#131bis	R2-2506747

Prague, Czech Republic, 13th - 17th October 2025

3GPP SA WG2 Meeting #170	S2-2507636

Gothenburg, Sweden, 25-29 August 2025

# Title:	LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

# Response to:	-

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	SA2

To:	RAN2, SA4, CT1, SA3, SA1, RAN1

Cc:	-

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Under the study item FS_5GSAT_Ph4_ARC SA2 is studying the support for IMS voice over NB-IoT NTN connected to EPC. In the context of this study item SA2 is investigating solutions in TR 23.700-019 that could potentially use IP or non-IP type (i.e. with and without IP/UDP header) for the (voice) traffic and would like to ask some questions in order to help SA2 to evaluate the alternative solutions.

When IP PDN connection is used, in order to achieve the target KPIs for support of IMS voice over NB-IoT NTN connected to EPC, it is expected that the support for Robust Header Compression (RoHC) will likely be required. RoHC is supported between UE and eNB when "user plane" DRBs are used to transport the data or between the UE and MME when "control plane CIoT" and SRBs are used to transport the data.

In order for SA2 to be able to evaluate the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions:

During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state).

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

Voice packets of different sizes can be observed over the NB-IoT NTN link e.g. due to talk/silence transitions, or occasionally RoHC state change, etc. The size difference of voice packets during talk/silent transition is described in S2-2506371/S2-2507021/S2-2507102 (neither endorsed nor agreed).

More information on the RoHC related issues is described in S2-2506371, S2-2507021 and S2-2507102 that are neither endorsed nor agreed by SA2.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

In the context of using "Non-IP" for voice media traffic there is no  header compression. To reduce the header overhead, it is necessary to understand which parts of the RTP header, e.g., sequence number, timestamp, SSRC (Synchronization Source ID), Payload Type (PT) is essential to the receiver support IMS voice over NB-IOT NTN.

Question 4 (To SA4): What are the essential RTP header fields for the minimum information that needs to be provided in the RTP header for IMS voice over NB-IoT NTN?

Question 5 (To SA4): Does SA4 have any views on how to reduce the RTP header overhead for IMS voice over NB-IoT NTN?

SA2 has observed that RAN2 has enforced the limitation that the capability of a NB-IoT UE supports a maximum 2 DRBs. In order for SA2 to reach a conclusion to use the user plane and DRBs to support IMS voice, SA2 would like to understand whether NB-IoT Access Stratum protocols can support more than 2 DRBs (e.g. 3 DRBs) in Rel-20, then the UE can support simultaneous voice and other services even during the call.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

In addition to user-plane based solutions to carry voice media, several other proposed solutions under Key Issue #1 (Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC) rely on control-plane-based mechanisms, where Control Plane CIoT EPS optimisation and SRBs are used to transport SIP signaling and/or voice media.

For this category of “control plane solutions”, SA2 understands that CT WG1 in rel.19 under the WI NORDAT_CP specified a new NAS message for data transfer over NAS with a reduced NAS layer overhead of 2 bytes, NAS layer security overhead of 4 bytes for MAC (integrity protection) and 1 byte for SN (Total = 7 bytes). SA2 is evaluating whether it is possible to reduce the overhead further. Specifically SA2 has the following questions:

Question 7 (To SA3): Considering that in the context of alternative solutions documented in TR 23.700-19 for the support of IMS voice over NB-IoT NTN connected to EPC, a specific SRB (i.e. via a dedicated EPS bearer for Data over NAS) will be used for transfer of voice media packets only, is there a concern to eliminate the 5 bytes of NAS layer security overhead?

Question 8 (To CT1): In addition to question 7 related to security, is there any further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN?

Currently, SRBs are only supported over RLC Acknowledged Mode (AM) (TS 36.331), and the possible impacts of this are discussed in S2-2507107, which is not endorsed or agreed by SA2.

In order for SA2 to be able to evaluate between user-plane and control-plane solutions to carry voice media, the use of RLC Unacknowledged Mode (UM) for SRBs, SA2 has the following question:

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

According to TS 22.228, it is possible for the UE to support more than one IMS voice calls e.g. simultaneously using the communication HOLD service that enables a user to suspend media within a session and resume that media at a later time.

In addition, SA2 observed that Dual Tone Multi-Frequency (DTMF) signalling defined in TS 23.014 and TS 26.114 Annex G would also require DTMF events as telephone events within the same RTP media stream as the speech, i.e. "inband", using the same IP address and UDP port as the RTP for speech, but using a different RTP Payload Type number.

These requirementswill have an effect on the overall protocol stack overhead for a non-IP based solution to support IMS voice over NB-IoT NTN.

SA2 assumes that requirements for multiple calls and DTMF still apply to support of IMS voice over NB-IoT NTN bus since these are service requirement, SA2 would like verification from SA1.

Question 11 (To SA1): Can SA1 confirm, whether support for more than one IMS voice calls and support for DTMF is required for IMS voice over NB-IoT NTN?

2. Actions:

To RAN2, SA4, SA3, CT1, SA1, RAN1:

ACTION: 	SA2 requests the respective WGs in “TO” to answer the questions addressed to them as per above and provide any other feedback they think is necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, PRC

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, USA