3GPP TSG RAN WG2#132	R2-2508199

Dallas, USA, 17th - 21st, November 2025

3GPP TSG-RAN WG1 Meeting #122bis	R1-2508064

Prague, CZ, 13th – 17th October, 2025

Title:	Reply LS on precompensation for NB-IoT NTN TDD mode

Response to:	(R1-2506731) LS Reply on precompensation for NB-IoT NTN TDD mode

Release:	Release 19

Work Item:	IoT_NTN_TDD-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact person:	Alberto Rico

albertor@qti.qualcomm.com

# 1	Overall description

RAN1 has reached the following agreement on segmented precompensation, related to the working assumption on the previous RAN1 LS:

Agreement

Confirm the following working assumption with modifications:

For precompensation, from RAN1 perspective:

- The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

- The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18)

  - Segmented precompensation is not supported.

  - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission

- FFS: whether spec impact is in RAN1, RAN4 or both.

NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

The RAN4 CR may not be aligned with the latest RAN1 agreement. RAN1 would like to ask RAN4 if they have concerns on the updated agreement, and:

- If RAN4 has concerns on the RAN1 agreement, RAN1 will modify its specifications to introduce segmented precompensation within the set of 8 consecutive uplink subframes.

- If RAN4 has no concerns on the RAN1 agreement, we recommend RAN4 to update their specifications to clarify that segmented precompensation is not supported in IoT NTN TDD mode.

# 2	Actions

To RAN4

ACTION: 	RAN1 asks RAN4 to take the above information into account.

# 3	Dates of next WG RAN1 meetings

RAN1#123	2025-11-17    -    2025-11-21	Dallas, US

RAN1#124	2026-02-09    -    2026-02-13	Gothenburg, Sweden