3GPP TSG RAN WG2 Meeting #131bis         	                          R2-2506772

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	10.2

Source:	Xiaomi

Title:	General considerations on RAN2 6G study

Document for:	Discussion and Decision

# Introduction

In RAN#106 meeting, RAN plenary study item on “Study on 6G scenarios and requirements” [1] was approved. In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [2] was approved.

During RAN#109 meeting, RAN further discussed the requirements for architecture and migration, 6G device type, requirements on ISAC, etc.

In this contribution, we discuss the general principles/guidance for 6G, support of AI/ML and Sensing (including the role split between different WGs), and provide our view on the potential RAN2 impact from deployment scenarios and architectures.

# Discussion

## 2.1 General principles/guidance for 6G

Following principles/guidance included in 6G WG SI [2] should also be applied for RAN2 6G discussion:

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Based on the discussion in RANP, the common understanding from companies is that we should simplify the system design for 6G (e.g., common design to meet different service requirements as much as possible, avoid to adopt multiple options for the same functionalities), power saving shall take both network and device into account.

Observation 1: Important principle/guidance defined in 6G WG SID are that 6G shall aim to simplify the system design (e.g., common design to meet different service requirements as much as possible, avoid to adopt multiple options for the same functionalities), power saving shall take both network and device into account.

Lesson learnt from 5G (1): Multiple drops of ASN.1 freeze in 5G first release delayed the deployment of the 5G, and also led market fragments.

It has been agreed in RAN plenary#102 meeting that (in RP-233985 [3]) Release 21 is expected to be delivered with a single drop (i.e., a single code freeze);

Lesson learnt from 5G (2): Multiple options for the same feature as compromise between companies, which increase standards work load, increase UE/network complexity and also lead market fragment.

It has been addressed in 6G WG SID as “minimizing the adoption of multiple options for the same functionality”;

Proposal 1: In 6G RAN2 study, RAN2 should aim to

- Simplify the system design for 6G

  - Common design to meet different service requirements as much as possible

  - Avoid to adopt multiple options for the same functionalities

  - Support of multiple options shall not be the compromise way forward

- Single ASN.1 freeze for 6G first release instead of multiple freezes as 5G.

Lesson learnt from 5G (3): The power saving and energy saving features are introduced quite late, which leads the complicated design for backward compatibility, and increases the burden to be deployed.

6G WG SID has mentioned to consider “Energy efficiency and energy saving: both for network and device”. Based on RAN2 agenda “More detailed aspects related to energy efficiency and power savings can be discussed as part of UP/CP/Common design”, the energy efficiency should be discussed per feature.

Following UE power saving features have been specified for 5G and 5G enhancements.

| Release | UE power saving features |
| --- | --- |
| Rel-15 | C-DRX |
|  | BWP switch |
|  | UE overheating assistance information report |
| Rel-16 | DCP: WUS signal for C-DRX adaptation |
|  | Cross slot scheduling |
|  | UE power saving UAI report (e.g. MIMO layer, bandwidth, C-DRX preference, etc.) |
|  | IDLE/INACTIVE state RRM measurement relaxation (e.g. based on low-mobility/not-at-cell-edge criteria) |
| Rel-17 | PEI |
|  | Provision of connected mode TRS configurations to idle/inactive-mode UEs |
|  | RLM/BFD relaxation |
|  | Connected mode RRM measurement relaxation |
|  | SSSG switching |
|  | PDCCH skipping |
| Rel-18 | LP-WUS study |
| Rel-19 | LP-WUS for idle mode and connected mode operation |

Observation 2: 5G has specified quite many UE power saving solutions and this actually makes it difficult for operators to deploy all of those. Some simplification would be needed in 6G design.

Following are NES features specified in 5G.

| Release | NES features |
| --- | --- |
| Rel-18 | Cell DTX/DRX |
|  | SSB-less SCell for inter-band CA |
| Rel-19 | OD-SIB1 |
|  | OD-SSB |
|  | Paging adaptation |
|  | RACH adaptation |

Observation 3: Due to the need of the existence with legacy UEs, 5G NES features either make cells not accessible for legacy UEs, or support two mechanisms, one for legacy UEs, another for new UEs which actually increase network energy consumption. Some NES features increase UE’s complexity and degrade UE’s performance.

In addition, as discussed in [4], when evaluating the AI/ML use cases, one of the critical criteria for use case selection is power consumption. As the deployment of AI/ML use cases on the User Equipment (UE) side increases, potential power consumption is expected to rise accordingly; therefore, more power-efficient use cases are preferable for enhancing user experience. This is not only applied for AI/ML use case selection. We should take into account the trade-off between energy saving and performance when evaluating CP, UP, mobility and AI related features.

Proposal 2: In 6G RAN2 study on energy efficiency:

- Power saving/Energy saving solutions specified in 5G can be used as the starting point for the 6G evaluation. However, the solution should be adopted for 6G only if justified.

- The design of each feature, including CP, UP, mobility, AI, etc, shall take into account the trade-off between energy saving and performance.

- Energy efficiency of UE and network should be jointly evaluated and unilateral power saving mechanism at UE or NW side should not lead to unacceptable performance degradation or increment on complexity/power consumption at the other side.

Lesson learnt from 5G (4): RedCap features were introduced at late stage (Rel-17, Rel-18) on top of the system designed for commercial UEs which led the complicated design for backward combability, and impacted the deployments of RedCap features;

As described in [5], Internet-of-Things (IoT) is developed for many years and the relative branches of technologies have been evolving for several releases in 3GPP. In 4G LTE and 5G NR, IoT technologies includes NB-IoT, LTE MTC, LTE Cat. 1/1b/4, NR RedCap, NR eRedCap. In 4G and 5G, IoT technologies were designed from Day-2, which means that the basic system is firstly designed to support normal UE, and then the enhancement, such as complexity reduction, coverage enhancement and power consumption reduction, were designed based on the basic system to support IoT UE. This procedure can cause potential fragmentation and complication of the system design, and limitation of IoT design. According to the SID [2], 6GR targets scalable and forward compatible design for diverse device types. It is important to take IoT device into account in 6G Day-1 design, targeting a common system design for both IoT and non-IoT devices.

Figure 1: Support of IoT devices

From system design perspective, 6G should make a harmonized system supporting different device type, e.g., co-existence of normal UE and IoT device. To satisfy the diverse IoT requirements, on the one hand, the system shall be designed based on the smallest maximum capability of the 6GR lowest-tier IoT device, e.g., for cell search procedure. On the other hand, the support of lowest end of IoT device in 6G shall not impact the performance of commercial UEs.

Proposal 3: In 6G RAN2 study on diverse device type, RAN2 should aim to support diverse device types, including IoT devices, with single framework from Day 1. The design to support IoT device shall not impact the performance of commercial UEs.

## 2.2 Organization on the support of AI/ML and Sensing

In [2], AI/ML, Sensing related works are described in different objectives and for different WGs.

For Sensing, as agreed in RAN, RAN1 work shall be triggered by RAN, i.e., RAN shall discuss the use case and requirements first. During RAN#109 meeting, RAN discussed whether the work in RAN1 can be started early than April, 2026, but no consensus. Some companies think that RAN may not complete use case and requirement discussion early. It is unclear how to organize the discussion from different WGs, including RANP.

Observation 4: Further clarifications are needed on how to organize the discussion among WGs and RAN for AI/ML and Sensing.

### 2.2.1 Support of AI/ML

Lesson learnt from 5G (5): Ambiguous role split between RAN2 and RAN1 on LCM delayed the RAN2 progress

During 5GA study on AI/ML air interface, we observed that there’s unclear leading group when discussing AI/ML life cycle management between RAN1 and RAN2. For example, applicability reporting procedure was first discussed in RAN2 motivated to allow UE determine its functionality applicability. It is observed that, during the whole Rel-19 WI phase, majority of RAN1’s main focus is to define inference and monitoring behaviour after UE receives inference configuration of applicable functionalities. At the end of Rel-19, for applicability reporting procedure, RAN1 only defines what inference configurations and applicability configurations need to be provided to the UE, even though RAN2 (as the objective leading group) hold the discussion for several meetings. In 6GR, as captured in [2], RAN2 is the leading group of LCM procedure and data collection and data management of AI/ML framework, the study on such aspects should have no RAN1 dependency, i.e., RAN2 can study in parallel with RAN1.

To support AI service, RAN2 needs to consider:

| Feature | Pre-requisite for RAN2 study | Remark |
| --- | --- | --- |
| Architecture, Protocol and Life cycle management framework | No prerequisite for architecture and protocol study aspects, collaboration with RAN3. AI/ML based use case studied in 5GA can be used as baseline for the discussion. | RAN2 can start discussion on general LCM framework without waiting for RAN1 6G study on use cases. RAN2 can start the study on protocol and architecture aspects based on the study outcome of LCM framework. |
| Data Collection and Transfer Methods | Define data metrics parameters for different AI-based use cases (RAN1/2) | Only impact RAN2 in WI phase when capturing RAN1 parameters. |
|  | QoS/latency/data size from 5GA RAN1 study can be considered as baseline; | RAN2 can start the study based on QoS/latency/data size from 5GA RAN1 study RAN2 can also start the study on data transfer aspects, including procedure, protocol, transfer tunnel, etc. |
| RAN2 centric AI/ML use cases (e.g., AI-based mobility, AI-based power saving, AI-based traffic prediction, etc) | RAN2 study on how to support basic features without AI/ML for RAN2 use cases; | Wait for RAN2 basic feature discussion, then consider AI-based use cases on top of basic design. |
| RAN1 centric AI/ML use cases | Inference configuration depends on RAN1 use case study | RAN1 aspects only impact RAN2 in WI phase when capturing RAN1 parameters. |

Based on the lessons learnt from 5G, RAN2 should lead the discussion on AI/ML framework, i.e., LCM procedures, data collection and data management without waiting for RAN1 study. This is also aligned with the objective in 6G WG SI [2]. The only unclear part is, the relationship with RAN3 architecture objective. To our understanding, RAN2 can lead the discussion on the required architecture change, and coordinate with RAN3 when necessary.

Proposal 4: In 6G RAN2 study on AI/ML, the role split should be:

- RAN2 leads the discussion on AI/ML framework, i.e., LCM procedures, data collection and data management without waiting for RAN1 study

- RAN2 also leads the discussion on the required architecture change based on the discussion on LCM procedures, data collection/data management, and in coordination with RAN3 when necessary

### 2.2.2 Support of Sensing

In Rel-20 5GA ISAC SI, only gNB monostatic is considered, and only RAN1 and RAN3 are involved in the discussion. Based on the SID [6] of 5GA ISAC, the work split between RAN1 and RAN3 are:

- RAN1 is responsible for the evaluation on the performance, and identify the measurement for it

- RAN3 is responsible for the study on the architecture, procedures and signalling between RAN and CN to support ISAC;

- RAN1 and RAN3 study are handled in parallel, i.e., RAN3 starts the study without waiting for RAN1 progress;

Observation 5: In Rel-20 5GA ISAC SI, RAN3 starts the study on architecture, procedure in parallel with RAN1 without waiting for RAN1 input.

The RAN2 scope and role split among RAN1, RAN2 and RAN3 for 6G Sensing is more similar to NR positioning considering all sensing modes are considered in 6G as agreed in last RANP in [7], i.e., UE is involved.

Rel-16 is the first release for NR to support RAT dependent positioning. According to [8], the role split in Rel-16 on NR positioning study is:

- RAN1 is responsible for the evaluation on the performance, study and evaluate potential solutions of positioning technologies

- RAN2 is the primary WG on the Study of positioning architecture for location services, functional interfaces, protocol, and procedures for supporting NR dependent positioning technologies; RAN3 is the secondary WG to check the decision from RAN2;

- RAN1 and RAN2 study were discussed in parallel, i.e., RAN2 started the study without waiting for RAN1 progress;

Observation 6: In Rel-16 NR positioning SI, RAN2 is the primary WG to lead the study on architecture, procedure in parallel with RAN1 without waiting for RAN1 input.

To support sensing service, RAN2 needs to consider:

| Feature | Pre-requisite for the study | Remark |
| --- | --- | --- |
| Phy design and evaluation for sensing, e.g., Waveform, numerology, frame structure, RS design, etc | Use case and sensing mode related; | Mainly impact RAN2 in WI phase, wait for RAN1 |
| Measurement metrics | Use case and sensing mode related; | Related to data framework study in RAN2, and the configuration, reporting details. Mainly impact RAN2 in WI phase, wait for RAN1 |
| Aspects of integration with communication services | The details of physical design | Mainly impact RAN2 in WI phase, wait for RAN1 |
| Architecture/higher layer procedures and protocol aspect, including sensing Tx/Rx management, sensing node selection, etc. | Sensing mode related; | RAN2 can start the study as RAN has agreed to study all 6 sensing modes [7], in addition, RAN also agreed the use case in [10] as The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. |

Based on the lessons learned from 5G, RAN2 should lead the discussion on higher layer procedures and protocol aspects, which is aligned with the objective in 6G WG SI [2]. However, the end-to-end architecture is also related to higher layer procedure since the protocol should be terminated between UE and the Sensing function. Therefore, similar to positioning, RAN2 should be the primary group on architecture discussion instead of RAN3. RAN3 should be responsible for the study on interface/protocol between RAN and CN and internal RAN architecture, interface and protocol

Another question is when RAN2 can start the discussion, should we wait for RAN1 or not? To our understanding, based on the experience from previous release discussion on positioning, and R20 5GA ISAC, RAN2 can start the study on architecture, procedures and protocol in parallel with RAN1 discussion. RAN2 study on architecture, higher layer procedure and protocol, has no dependency on the RAN1 work.

Lesson learnt from 5G (6): RAN2 is the primary working group (WG) responsible for studying the architecture, procedures, and protocols of NR positioning, working in parallel with RAN1.

Proposal 5: In 6G RAN2 study on ISAC, the role split should be:

- RAN2 leads the study on overall architecture, procedure for Sensing, in coordination with SA2, RAN3

- RAN2 is responsible for the protocol between UE and SF

- RAN3 is responsible for the study on interface/protocol between RAN and CN and internal RAN architecture, interface and protocol

- RAN2 can start the study on sensing architecture and procedures in Feb meeting, including: sensing function location; sensing node selection/management and procedure, etc., without waiting for RAN1 progress.

## 2.3 Deployment scenarios and RAN architecture aspects (RAN2 impact)

5G NR supports CU-DU split. We assume similar function split architecture will be inherited in 6G as shown in figure 2 below.

Figure 2: 6G function split at RAN

Lesson learnt from 5G (7): DU related configurations are configured to the UE in inefficient way

The drawback of current CU-DU split architecture is that DU related L1/L2 configurations needs to be configured via CU RRC, and therefore the signalling has to go through the route of DUCUDUUE, which leads unnecessary signaling exchange, and adds significant latency compared with direct transmission from DU to UE. From function management perspective, it would be good to let the DU handle the configuration managed by DU (independent from CU) directly. Considering it is related to RAN2 RRC signalling design, it is RAN2 scope to study how to improve the signalling between UE and RAN for DU related configurations.

Proposal 6: If CU-DU split architecture is considered in 6G, study how to improve the signalling between UE and RAN for DU related configurations.

# Conclusion

Based on the discussion, we have following proposals:

Lesson learnt from 5G (1): Multiple drops of ASN.1 freeze in 5G first release delayed the deployment of the 5G, and also led market fragments.

Lesson learnt from 5G (2): Multiple options for the same feature as compromise between companies, which increase standards work load, increase UE/network complexity and also lead market fragment.

Proposal 1: In 6G RAN2 study, RAN2 should aim to

- Simplify the system design for 6G

  - Common design to meet different service requirements as much as possible

  - Avoid to adopt multiple options for the same functionalities

  - Support of multiple options shall not be the compromise way forward

- Single ASN.1 freeze for 6G first release instead of multiple freezes as 5G.

Lesson learnt from 5G (3): The power saving and energy saving features are introduced quite late, which leads the complicated design for backward compatibility, and increases the burden to be deployed.

Proposal 2: In 6G RAN2 study on energy efficiency:

- Power saving/Energy saving solutions specified in 5G can be used as the starting point for the 6G evaluation. However, the solution should be adopted for 6G only if justified.

- The design of each feature, including CP, UP, mobility, AI, etc, shall take into account the trade-off between energy saving and performance

- Energy efficiency of UE and network should be jointly evaluated and unilateral power saving mechanism at UE or NW side should not lead to unacceptable performance degradation or increment on complexity/power consumption at the other side.

Lesson learnt from 5G (4): RedCap features were introduced at late stage (Rel-17, Rel-18) on top of the system designed for commercial UEs which led the complicated design for backward combability, and impacted the deployments of RedCap features;

Proposal 3: In 6G RAN2 study on diverse device type, RAN2 should aim to support diverse device types, including IoT devices, with single framework from Day 1. But the design to support IoT device shall not impact the performance of commercial UEs.

Lesson learnt from 5G (5): Ambiguous role split between RAN2 and RAN1 on LCM delayed the RAN2 progress

Proposal 4: In 6G RAN2 study on AI/ML, the role split should be:

- RAN2 leads the discussion on AI/ML framework, i.e., LCM procedures, data collection and data management without waiting for RAN1 study

- RAN2 also leads the discussion on the required architecture change based on the discussion on LCM procedures, data collection/data management, and in coordination with RAN3 when necessary

Lesson learnt from 5G (6): RAN2 is the primary working group (WG) responsible for studying the architecture, procedures, and protocols of NR positioning, working in parallel with RAN1.

Proposal 5: In 6G RAN2 study on ISAC, the role split should be:

- RAN2 leads the study on overall architecture, procedure for Sensing, in coordination with SA2, RAN3

- RAN2 is responsible for the protocol between UE and SF

- RAN3 is responsible for the study on interface/protocol between RAN and CN and internal RAN architecture, interface and protocol

- RAN2 can start the study on sensing architecture and procedures in Feb meeting, including: sensing function location; sensing node selection/management and procedure, etc., without waiting for RAN1 progress.

Lesson learnt from 5G (7): DU related configurations are configured to the UE in inefficient way

Proposal 6: If CU-DU split architecture is considered in 6G, study how to improve the signalling between UE and RAN for DU related configurations.

# Reference

[1] RP‑251395, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, “New SID: Study on 6G Scenarios and requirements”

[2] RP-252912, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[3] RP-233985, High-level Considerations for 6G Timeline, TSG Chairs

[4] R1-25xxxx, Initial consideration on AI/ML in 6GR interface, Xiaomi

[5] RP-252197, Discussion on the requirements for IoT towards 6G, Xiaomi

[6] RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator)

[7] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services	CMCC

[8] R2-1816755	RAN2 scope and work plan on NR positioning SID	Intel Corporation

[9] RP-182155, “Revised SID: Study on NR positioning support”, Intel Corporation, Ericsson

[10] RP-252882	TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements	CMCC