3GPP TSG RAN WG2 Meeting #131bis         	                                   R2-2506773

Prague, CZ, Oct 13th – 17th, 2025

Agenda Item:	10.2

Source:	Xiaomi

Title:	General Consideration on 6GR UE Capability

Document for:	Discussion and Decision

# Introduction

As captured in 6G SID [1], one of the objectives of 6G study is to design a single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements, including supporting target scalable and forward compatible design for diverse device types (if any).

In this contribution, we mainly focus on UE capability aspects, discussing the general principle and considerations on 6GR UE capability design, based on the lesson learnt from 5GNR.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# Discussion

## Organization of UE Capability Discussion

At the beginning of the whole discussion, we would like to first discuss why UE capability needs to be discussed at the early stage of 6GR SI.

It might be the conventional understanding that UE capability can only be discussed after people have a clear understanding on what feature(s) will be introduced. Therefore, normally UE capability discussion starts at the end of a certain release, including 5GNR UE capability framework design in Rel-15.

It is true that, for defining the UE capability signalling, a feature needs to be clearly defined and the ASN.1 signalling can be considered at late WI phase. However, there are other design aspects of UE capability that don’t have any dependency with what feature to be supported. For example, UE capability management and retrieval, UE capability framework and procedure, etc.

One example in 5GNR is on RACS. Due to the late start of UE capability discussion during SI, RACS was agreed by RAN2 at the late phase of Rel-15 WI. By that time, there’s no time for SA2 to study and support RACS in the first release of 5GNR, which is a pity [2].

Following guidance from TSG SA, SA2 discussed the topic in its SA2#127 meeting and observed the following: - The proposed functionality for UE capability handling based on UE capability ID is not deemed essential for the 5G System to operate in Release 15, the SA2 understanding being that a baseline (optimised) mechanism for UE capability signalling over Uu is being defined by RAN2 that will be available in Rel-15 whether or not additional optimisations based e.g. on UE capability ID are introduced. Given the above, SA2 concluded that additional optimisation of UE capabilities handling should be investigated as part of a dedicated study towards Release 16. A study item will be drafted accordingly.

Moreover, during 5GNR practice, sometimes RAN1/2/4 didn’t have enough collaboration on feature list design. For instance, the undefined granularity level ‘per band and per band combination’ was used by RAN1 for codebook related capabilities in Rel-16. However, due to limited time of implementing Rel-16 UE capabilities, RAN2 implemented those features without a clear understanding on the exact meaning of this new granularity (e.g., prerequisite, dependency, etc). In the end, RAN2 agreed to let UE report both per band capability and per BC capability for the same feature. On one hand, this increases the signalling overhead. On the other hand, due to unclear relationship between per band capability and per BC capability, UE can only report its per BC capability based on the lowest capability supported over a certain band, which limits UE implementation flexibility and makes this per BC capability becomes useless. To resolve issues observed in 5G, for 6GR, RAN1/2/4 are expected to have much tighter and earlier collaboration on the definition and the design of UE capability parameters, especially RAN2 should provide clear guidance to RAN1/4 on how to define UE capability parameters with a proper granularity.

Lesson learnt from 5G (1): Due to the late start of UE capability discussion, there’s limited time for cross-WG coordination for an efficient and proper UE capability design (e.g., postpone of supporting a useful feature, unclear definition of granularity, etc).

Proposal 1: Study UE capability management, framework, signalling framework design as early as possible with tight and efficient coordination cross WGs (RAN1/4, RAN3/SA2).

## UE Capability Management and Retrieval

As shown in Figure 1, we summarize UE capability management and retrieval functions supported in 5GNR. In Rel-15, based on the network configuration, UE reports its full UE capabilities supported (on the network requested frequency band(s)) as a container to gNB. Then, gNB further propagates the received radio capability container to CN for long-term storage. Based on the release supported by the gNB, gNB uses the relevant capabilities based on its own release. Later on, a new gNB can receive the corresponding UE capability from CN after UE is moved to the new gNB.

Figure 1. UE Capability Management and Retrieval

However, it is observed that UE radio capabilities for paging were not supported in 5GNR Day 1, as the basic paging function are assumed mandatorily supported without signalling. In later releases, when paging features, such as PEI, LP-WUS, on-demand SIB1, were introduced, those capability(ies) related to paging are forwarded via Xn interface across gNBs as separate IEs, instead of RRC container. This leads to some drawbacks for new paging features, for example, if a UE initially accesses to an old release gNB then moves to a new release gNB, due to lack of radio capability for paging, the UE cannot be paged by the new gNB using the newly supported feature. Even RAN2 introduced radio capability for paging as a container in Rel-19, gNBs with legacy releases can only support such new paging feature by upgrading itself by network implementation. Furthermore, UE may still need to report the same capability for paging in the container for inter-node message, as well as over the air interface (e.g., which is now under-discussion in Rel-19 LP-WUS), which further causing duplicated capability reporting over the air interface. Additionally, capability for paging doesn’t need to be exchanged between gNBs, if those capabilities are included in capability container to CN from Day 1.

Lesson learnt from 5G (2): Incomplete UE capability management solution in Day 1 may bring drawbacks to later releases, e.g., duplicate reporting over air interface, forward-compatibility issue, etc.

In 6GR Day 1, RAN2 should first study how to manage the capabilities reported by the UE, including:

- What capabilities need to be stored;

- Where to store the reported UE capabilities;

- How the reported capabilities are maintained across gNBs/CN during UE’s mobility, etc.

5GNR UE capability management and retrieval framework can be considered as the starting point for 6GR design.

Proposal 2: Study UE capability management (e.g., entities storing UE radio capability, UE capability exchange between gNB and CN, UE capability exchange between gNBs, etc) based on the following principles:

- Support forward-compatibility for UE capability exchange between nodes;

- Minimize the frequency of UE reports its full capability during UE’s mobility;

- Minimize duplicate reporting of the same UE capability over the air interface.

Consider 5GNR UE capability management and retrieval framework as starting point for 6GR, including:

- Storing UE static capability at CN;

- RAN obtains UE capability from CN when available;

- RAN obtains UE capability from UE only when CN stored UE capability is not available or not complete.

## UE Capability Framework

### Capability vs. Configuration

As captured in TS 38.306 [3], while configuring and scheduling the UE, network needs to respect and consider UE capability, i.e., the configuration from network cannot exceed UE capability.

The network needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

The intention of UE capability is to avoid the mismatch between UE supported feature(s) and network configuration(s). However, during 5G practice, we observed following issues:

- Scenario 1: No configuration defined corresponding to the UE capability

One example in RAN2 #131 meeting TEI19 [UAI-cellDTRX], an optional capability with signaling was introduced to indicate whether the UE supports providing its preference of a cell group on DRX parameters for power saving and its preference on cell DTX/DRX related parameters for PCell in RRC_CONNECTED. However, there’s neither network configuration introduced for this feature nor clear network behavior defined. It is a clear example on the misusage of the UE capability signalling.

- Scenario 2: Network only provides subset configuration(s) corresponding to UE reported capability

Even configuration is defined for certain UE capability, due to lack of network supports, network may only provide the configuration(s) to the UE reported capability corresponding to network supported feature(s). However, due to this unawareness of the network supported features, the UE unnecessarily reports its capabilities of feature(s) that are not supported by the network, which further causes the increase of UE capability message size. In 5GNR, in order to reduce the capability message size, RAN2 discussed the capability filtering scheme, however, in the end, only filtering based on RAT, frequency band, etc are introduced, so that UE doesn’t need to report unnecessary signalling on other frequency bands. In 6GR, it would be good to consider how to reduce the unnecessary capability reporting from Day 1.

Based on the above observations from 5GNR, from 6GR Day 1, it is important to define UE capabilities only if there’s a corresponding configuration provided by the network. Furthermore, UE awareness of network supported features should also be considered.

Lessons learnt from 5G (3): During 5GNR practice, there are some cases that network configurations are mismatch with UE capability (e.g., some capabilities don’t have any configuration defined, some capabilities are reported unnecessarily).

### Changeable Capability

In 5GNR, UE RRC capability reporting procedure can only be initiated by network when network needs (additional) UE radio access capability information. Moreover, whenever UE wants to update its capability, UE either needs to perform TAU or DETACH from the network for capability reporting.

Based on the practice of 5GNR, it has been observed that network-initiated capability reporting is not sufficient in 5GNR:

First of all, there are some scenarios where UE needs to report its temporary restricted capabilities to network, e.g., MUSIM, IDC, preference of temporarily reducing capabilities (e.g., overheating, the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of MIMO layer, AI/ML applicability reporting, etc, as listed in Annex). In 5GNR, the above UE capability updates are reported to network as UE Assistance Information. However, due to lack of specified network behaviour, it is possible that network does not react based on the updated UE capability. The capability update via UAI is not that useful since the network’s action is not specified.

Lesson learnt from 5G (4): Temporary restriction on UE capability reporting in 5GNR misused UAI without clear network behaviour specified, which raises a risk for those capabilities being ignored by the network.

Moreover, as shown in Figure 2, shipment volume of foldable phone rapidly increases during the past 5 years in China [4].

Figure 2. Foldable Phone Shipment Volume (China Market)

When the foldable phone is folded/unfolded, the form factor of foldable phone can be changed, which further leads to the change of UE capability, e.g., MIMO layer, the number of supported ports, the maximum RS number, etc. There might be some concerns on signalling overhead from the network point of view, especially when handling frequent UE capability reporting. However, based on the survey [5], the top 5 scenarios of using unfolded screen (Figure 3) show that the foldable phone will keep unfolded for a long time based on the user behaviour, i.e., user will not frequently fold/unfold the phone. This further proves that the foldable phone will not change its capability frequently.

Figure 3. Usage of Unfoldable Phone in Unfolded Status

Hence, allowing UE-initiated capability update with predicable network action becomes more important in 6GR.

Last but not least, as listed earlier, the AS layer configuration(s) will be impacted after receiving UE capability updates. It would be good to allow UE simply use AS-triggered reporting to quickly inform RAN for RRC reconfiguration.

Observation 1: Changeable mobile phone form factors increase the need for UE to update its UE capability to network, where clear network behaviour is strongly expected in 6GR.

UE capability parameters in 5GNR have hierarchical structure, where ‘Per’ indicates the level of the associated parameter is included, including ‘Per frequency range (FR1/FR2)’, ‘Per band’, ‘Per band combination’, ‘Per feature set (per band per band combination)’, ‘Per feature set per CC’, ‘Per band pair per band combination’, ‘Per UE’, etc.

We understand that the key motivation of defining multiple levels of granularity is to provide UE flexibility based on different UE implementation for deployment in different frequency/band/carrier. During 5GNR, there were many practices to discuss how to understand each granularity and also the prerequisite between each granularity. For example, for dependent capabilities with prerequisite capability in a finer granularity, the UE should indicate support of the prerequisite capability in at least one finer granularity. And the dependent capability is supported only in the finer granularity where the prerequisite capability is supported.

Based on the well-established knowledge we built in 5GNR, 5GNR UE capability granularity can be considered as a starting point for 6GR UE capability granularity level discussion.

Observation 2: RAN2 has built a good knowledge about different UE capability granularities in 5GNR.

Based on the above analysis, 6GR UE capability framework design should at least follow the below principles.

Proposal 3: UE capability framework design should follow the below principles:

- UE capabilities with signalling should be defined only if there’s a corresponding configuration provided by the network;

- UE should be aware of network supported features before UE capability reporting;

- UE should be allowed to perform AS-triggered UE capability update without network request, instead of using UAI;

- 6GNR UE capability granularity level should support sufficient UE flexibility based on the real deployment needs. 5GNR UE capability granularity levels are considered as starting point for further analysis;

## UE Capability Signalling Design

Based on the above proposed principles, RAN2 can further study how to design UE capability signalling for 6GR.

### Capability Signalling Category

5GNR defines five capability signalling categories with the following usages:

- Mandatory without capability signalling

  - This category is mainly used for capabilities that has no possibility for UEs to signal different values, e.g., for initial access.

- Mandatory with capability signalling

  - This category is used to indicates whether the feature has been successfully tested, i.e., IODT purpose. The UE can only report the relevant signalling if UE is passed IODT with more than one vendor.

- Conditional mandatory with capability signalling

  - This category is used to represent that the feature is conditional mandatory and has a prerequisite of a mandatory feature with capability.

- Optional with capability signalling

  - This category is used to represent that the feature is optional for UE to support. If UE has implemented this feature and successfully being tested, UE can indicate its optional capability with signalling.

- Optional without capability signalling

  - This category is now mainly used for features that need to be supported in UE RRC_IDLE/INACTIVE state without network configuration.

If the UE implements, tests and wants to support the corresponding feature, the UE needs to send signalling to the network, in order to get the corresponding configuration(s). There’s no difference between mandatory with capability signalling and optional with capability signalling from implementation and deployment perspective. It would be good to avoid the political discussion in 3GPP in order to save valuable online time.

Observation 3: Mandatory with capability signalling has no difference compared to optional with capability signalling from implementation and deployment perspective.

Furthermore, if 6GR defines different device types (if any), UE capability signalling structure and how to use the above capability signalling categories can also be impacted. As observed from 5GNR, device-type specific capabilities (e.g., reduced optional capabilities) are captured in specification in the below five approaches:

Table 1. Reduced Capability in 5GNR

| Approach | Description | Example | Comment |
| --- | --- | --- | --- |
| Approach 1 | Introduce single bit optional capability signalling to represent mandatory without capability signalling features for certain device type. | supportOfRedCap-r17, supportOfERedCap-r18. | Efficient and straightforward for UE to understand what mandatory features need to be supported by single bit signalling. |
| Approach 2 | For each mandatory without capability signaling feature (specified in TR 38.822) that is considered as optional for (e)RedCap, IAB-MT and NCR-MT, new optional capability with signalling is introduced. | longSN-RedCap-r19 for (e)RedCap longSN-NCR-r18 for NCR-MT | Multiple duplicated capabilities with the same meaning need to be introduced. |
| Approach 3 | For those mandatory capabilities that can be considered as a feature group, e.g., CA, MR-DC, etc, a general description was captured in TS 38.306, neither mentioning the specific capabilities nor introducing new optional capabilities. | TS 38.306 Section 4.2.15.1 for IAB-MT; Section 4.2.21/22.1 for (e)RedCap; Section 4.2.23.1 for NCR-MT. | It is not clear and accurate on which specific capabilities are not supported by the UE |
| Approach 4 | For (conditional) mandatory features with capability signalling, adding ‘It is optional for xxx UEs’ in each description field of features that are not supported by (e)RedCap, IAB-MT or NCR-MT | pdsch-256QAM-FR1 field description: It is optional for (e)RedCap UEs, IAB-MT, and NCR-MT, and mandatory with capability signalling for other UEs. | Spread across the whole specification. This further makes reader difficult to understand the full picture of what features can be supported by the reduced capability device type. |
| Approach 5 | For optional capability with signalling where reduced capability device type has constraints, limitations for reduced capability devices are added in the field description of the corresponding optional capabilities. | bwp-DiffNumerology field description: For the UE which is a (e)RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of the CORESET#0 (if configured) and SSB for PCell. For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). |  |

Due to different types of capability signalling approach, reader needs to have the basic understanding of the capability framework in order to understand what features are supported/not supported for different device types.

Based on the above discussion, if device types are defined in 6GR, at least the following key questions need to be addressed when defining UE capability signalling framework:

- Question 1: What are the common mandatory/optional capabilities shared by all device types and what mandatory/optional capabilities are device-type specific?

- Question 2: Whether and how to signal device-type specific mandatory features?

  - For example, can mandatory features of diverse device types be implicitly represented by device type without capability signaling, or should be explicitly signalled to network.

Therefore, based on the understanding of different capability signalling categories, in 6GR, a UE capability signalling framework harmonized for different capability signalling aspects of diverse device type (if any) should be studied.

Furthermore, currently, most of capabilities in TS 38.306 [3] and UE capability ASN.1 are organized based on protocol layers, e.g., PHY parameters, MAC parameters, RLC parameters, etc. There are also some sections organized based on the feature, e.g., IAB parameters, SON parameters, (e)RedCap parameters, Aerial UE parameters, etc. It would be good to make it clearer on what capabilities should be supported in order to support a feature.

Lesson learnt from 5G (5):

- Unnecessary political discussion on mandatory with capability signalling and optional with capability signalling.

- Complex approaches of defining reduced capabilities requires readers to have the basic understanding of the capability framework, in order to understand what capabilities are supported/not supported for different features/device types (if any).

### Message Size

As the technology evolved and features introduced in different releases, the size of UE capability message increases significantly. In 5GNR, several techniques have been studied, including RRC message compression (not supported in 5GNR), UE Radio Capability Signalling Optimisation (RACS), UL RRC segmentation, filtering, etc. The UE capability signalling message size is increased not only because the finer capability reporting granularity and the increased number of supported features, but also because of UE’s unawareness of network supported features (as discussed earlier in Section 2.3.1). In 6GR, instead of introducing fragmented solutions, RAN2 should study simplified methods to fundamentally reduce the UE capability reporting size from Day 1.

Lesson learnt from 5G (6): 5GNR introduced different mechanisms to reduce capability message size, but didn’t efficiently and fundamentally solve the issue.

Proposal 4: Study 6GR UE capability signalling framework in the following aspects:

- Signalling categories (i.e., mandatory/optional with/without capability signalling) for 6GR, aiming for 1) a harmonized design for minimum mandatory capabilities of different device types (if any), 2) easy readability to understand what capabilities should be supported in order to support a device type (if any)/feature;

- Simplified method to reduce UE capability message size (e.g., signalling simplification, overhead reduction, avoid unnecessary capability reporting, etc);

## 6GR UE Capability Specification

In 5GNR, UE capabilities are captured in three specifications, i.e., TS 38.306 [3], TS 38.331 [6], TR 38.822 [7]. In Table 2. 5GNR UE Capability Specifications, we summarize the key information/content delivered by each specification and the key take-aways from the specifications.

Table 2. 5GNR UE Capability Specifications

| Specification | Component | Key Information and Content | Key Take-Aways |
| --- | --- | --- | --- |
| TS 38.306 | Whole Spec | - UE Capability parameters and field description - Optional/Mandatory - Granularity Level - Prerequisite | - Clear capability necessary information - The specification is organized per protocol layer, some capabilities are organized per feature |
| TS 38.331 | Section 6.3.3 | - ASN.1 and necessary field description | - RAT information - Signalling structure |
| TR 38.822 | Whole Spec | - Mandatory features without capability signalling - RAN1/2/4 feature list mapping to UE capability parameter | - The specification is organized per release per feature group, which is easier for readers to understand what capabilities are needed for each feature. |

However, since ASN.1 and UE capability parameter are split among TS 38.331 [7] and TS 38.306 [3], it is difficult for readers know the connection and linkage between the signalling structure and capability field description. In addition, cross checking is needed between TS 38.331 [7] and TS 38.306 [3] in order to keep alignment. Furthermore, since most of capability parameters in TS 38.306 [3]/TS 38.331 [7] are organized by protocol layer, it is not always easy to get full set of capabilities from physical layer to higher layer for certain feature(s).

Lesson learnt from 5G (7): Different components of UE capability are spread across different specifications, making it difficult to connect between signalling structure and field description and difficult to maintain for cross-spec alignment.

In 6GR, RAN2 can study an efficient approach to organize capability specification(s).

Proposal 5: Study the specification structure between 331/306/822 based on the following principle:

- Avoid split between ASN.1 and field description;

- Easy approach to identify capabilities (mandatory/optional) introduced for different features/device types (if any) in each release.

## Early Identification

Device Type Early Identification

Different early identification mechanisms have been studied and supported in 5GNR. In Table 3, we summarized the existing mechanisms for device type early identification, together with the corresponding pros/cons [9]:

Table 3. Device Type Early Identification

| Early Identification | Device Type | Pros | Cons |
| --- | --- | --- | --- |
| Msg1 (RACH partitioning) | (e)RedCap: RedCap-specific PRACH resource | - Earliest coverage recovery from Msg2 - Earliest device type capability availability - Differentiate device type during contention resolution - Enables device type to operate in an initial BWP which is different from the device type UE bandwidth, where Msg2 can scheduled on the real device type UE bandwidth - Support different RRC processing delay requirements | - Reduction in PRACH user capacity - Increase of UL OH from PRACH - Support limited number of device types |
| Msg3 | (e)RedCap: Indication in the form of RedCap-specific LCID value for CCCH/CCCH1 | - Earlier coverage recovery from Msg4 - Earlier device type capability availability - Support different RRC processing delay requirements | - Poor forward compatibility to support early device type/feature identification due to limited size of Msg3 - Cannot be used for additional coverage recovery from Msg2 |
| Msg5 | IAB-MT: IAB node indication NCR-MT: NCR node indication | - No message size limitation as in Msg3 - No need for RACH partitioning | - Additional coverage recovery and device-type specific resource scheduling cannot be used during initial access procedure - RRC reject for device-type specific access restriction cannot be supported |
| MsgA | (e)RedCap: Indication in the form of RedCap-specific LCID value for CCCH | - Same as Msg1/3 | - Same as Msg1/3 |

As discussed in [8], system design should be harmonized based on the minimum requirement for different device types; on the other hand, to allow the network to provide the proper services (e.g., to schedule messages with better efficiency, restrict UE’s access to the network) for different device types, it is also essential for the device types to be identified by the network as early as possible.

In 6GR Day 1, according to the need and use case of different device types, RAN2 can further study how and from which step to differentiate different device types (if any), based on the initial access design in 6GR.

Feature Early Identification

It is further observed that, in 5GNR, early identification techniques are not only used to identify different device types, but also used for service early identification and early capability indication, which are summarized in Table 4.

Table 4. Feature Early Identification

| Early Identification | Feature | Motivation |
| --- | --- | --- |
| Msg1 (RACH partitioning) | Service identification: - URLLC/MPS; - Slicing. | Shorter latency for initial access with less congestion; For Slicing, resource isolation to meet different SLA for different slices supported by RAN |
|  | Early capability indication: - Msg1/3 repetition; - SDT. | Early capability indication for network scheduling/processing |
| Msg3 | Early capability indication: - Msg4 PUCCH repetition and Msg4 repetition - Early SRS/CSI-RS measurement (under discussion in 5GA R20) | Early capability indication for network configuration/processing |
|  | Message Size indication: - CCCH/CCCH1 | Early indication for network processing |
| Msg5 | IDLE/INACTIVE related: - Measurement availability (including idle measurement, reselection measurement, QoE measurement, etc); - Mobility state; - Flight path info availability. | Early acquisition of measurement/status/information from IDLE/INACTIVE UEs when goes to RRC_CONNECTED mode |
|  | Early capability indication: - MUSIM capability restriction indication; - UL RRC segmentation. | Early capability indication for network configuration/processing |
| MsgA | Same as Msg1 | Same as Msg1 |

As summarized in the above table, it is observed that feature early identification has been used for different purposes, e.g., service identification, early capability indication, etc. There’s a clear fragmentation on how early service and capability identification are supported during initial access in 5GNR.

Lesson learnt from 5G (8): 6GR should avoid introducing multiple solutions for feature/device type (if any) early identification.

Aiming to have a lean and simplified 6GR design, a unified approach should be studied to support feature early identification for different purposes. Additionally, to support forward compatibility for new features introduced in future releases, feature early identification scheme introduced in 6GR Day 1 should also be scalable and easy to extend for new features.

Proposal 6: Based on 6GR initial access procedure, study 1) how and from which step to differentiate device types (if any) within the harmonized system framework; 2) a unified and scalable approach for early feature identification with different purposes (e.g., early service identification, early capability acquisition, early information acquisition during state transition).

## Minimum Mandatory Capability

During LTE era, the UE category has been used to define different types of UE with different maximum DL/UL throughput (i.e., maximum number of DL-SCH/UL-SCH transport block bits received within a TTI), where the throughput is calculated based on the combination of MIMO, aggregated BW, modulation, etc. L2 buffer is further calculated based on the throughput. The UE vendors can only announce the support of particular category if the UE meets the maximum DL/UL throughput based on the predefined combination of MIMO, BW, aggregated BW, modulation. There is no way for UE vendors to produce better UEs unless the corresponding category is introduced.

To provide better user experience with faster connectivity and higher throughput, as time goes by, the number of UE category in LTE grew explosively. On the one hand, the increased number of UE categories leads to market fragmentation due to diverse branding. On the other hand, the LTE category was used as marketing branding, due to marketing competition reason, it makes defining new UE categories difficult, especially considering it was always challenging to reach consensus on the maximum throughput that can be supported by the newly introduced UE category.

When it comes to the era of 5GNR, higher throughput is no longer the only experience pursued by customer. On top of higher throughput and lower latency, 5GNR introduced many useful features to further improve user experience and bring additional business values to support vertical use cases, including slicing, NTN, XR, AI/ML, etc. In addition, companies would like to decouple the category from marketing branding purpose. Motivated by this, the definition of 6G device type should not be limited by the supported throughput and should not be used as market branding purpose.

Lesson Learnt from 4G (9): Defining UE categories only by achievable throughput is not only challenging to introducing new UE categories in later releases due to marketing competition and singalling overhead, but likely to leave market fragmentation as well.

Based on the lessons learnt from LTE, the UE category was not introduced in 5GNR Day 1. Instead, based on eMBB devices, 5GNR defines the mandatory maximum requirement for eMBB devices, including UE Rx/Tx antenna number, the maximum bandwidth, the maximum MIMO layer, and the maximum modulation.

During later releases in 5GNR, (e)RedCap device type was introduced to lower the device cost and complexity, compared to high-end eMBB and URLLC devices in Rel-15/Rel-16, especially for industrial sensors, wearable devices. However, since eMBB device type (i.e., high-end device type) is considered as the baseline when defining the mandatory features in 5GNR Day 1, there are several drawbacks when introducing the lower-end device type in later releases of 5GNR:

- To avoid non-backward compatibility, the lowest bandwidth of RedCap device is limited by the bandwidth (i.e., 20MHz) required by CORESET#0, which was defined by 5GNR Day 1. Even though supporting of spectrum less than 5MHz was introduced in Rel-18, it was not designed to be compatible with the existing system, but instead designed for the specific spectrum;

- To achieve cost reduction, the lower-end device type cannot support all mandatory features required for eMBB device type (e.g., Tx/Rx antenna, bandwidth, MIMO layer, power class, throughput, etc). Therefore, capability specification has to specify which (mandatory/optional) features introduced in previous releases are not supported by lower-end device type.

Lesson learnt from 5G (10): Introducing the lower end device type later than commercial devices limits the support of low-end device type.

During 5GNR, the following reduced capabilities were studied for (e)RedCap from different aspects (including baseband, RF, higher layer parameters, etc):

- Reduced bandwidth (support 20MHz in FR1 compared to 100MHz for handheld UE, 100MHz in FR2 compared to 200MHz for handheld UE);

- Reduced MIMO layer (1/2 MIMO layer) and Tx/Rx antenna (1/2 Tx/Rx);

- Reduced power class;

- Reduced higher layer capabilities due to limited throughput, including DRB number, PDCP SN length, RLC AM SN length (12bits);

- Reduced L2 buffer;

- Limited support of additional features, e.g., DC/CA, etc.

During RAN #109 meeting, device type has been agreed to be further investigated in the following aspects [10]:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type The value(s) for the identified parameters for a device type

As observed from the above list, parameters/factors are summarized mainly based on physical layer parameters (e.g., baseband, RF, etc). For different device types, similar as (e)RedCap, the minimum mandatory capability should also consider higher layer aspects, especially considering the cost brought by memory size, protocol design, etc.

Observation 4: Higher layer parameters/factors should also be considered as key components to define minimum mandatory capabilities of each device type.

From RAN2 point of view, Table 5 shows an example of different higher layer parameter requirements to support mandatory features for different device types:

Table 5. Mandatory Feature and Device Type

| Features | Feature group | Components | Devices not mandating to support Day 1 mandatory feature |
| --- | --- | --- | --- |
| 0. General (including supported bearer types) | Basic EN-DC procedures | 1) MCG DRB with LTE/NR PDCP 2) SCG DRB with NR PDCP 3) SN addition, modification, and release via RRC connection reconfiguration 4) Joint processing on the combined RRC messages 5) Failure handling (including both MN and SN) | (e)RedCap, IAB-MT, NCR-MT |
|  | SRB | 1) Split SRB with one UL path 2) SRB3 | (e)RedCap, IAB-MT, NCR-MT |
|  | DRB | 1) Maximum number of DRBs 2) Split DRB with one UL path 3) Split DRB with both UL MCG and SCG paths | IAB-MT, NCR-MT |
| 1. PDCP | Basic PDCP procedures | 7) 18bits SN | (e)RedCap, NCR-MT |
| 2. RLC | Basic RLC procedures | 2) RLC AM with 18bits SN | (e)RedCap, NCR-MT |
|  | RLC UM with long SN | RLC UM with long SN | NCR-MT |
| 5. SDAP | QoS | 1) Flow-based QoS 2) Multiple flows to 1 DRB mapping 3) AS reflective QoS | IAB-MT, NCR-MT |
|  | HD format | 1) DL SDAP HD 2) UL SDAP HD 3) SDAP End-marker | IAB-MT, NCR-MT |
| 6. Inactive | RRC inactive | RRC inactive | NCR-MT |
| 7. Mobility | Handover | 1) Intra-frequency HO 2) Inter-frequency HO 3) HO between TDD and FDD 4) HO from NR to LTE 5) HO from NR to LTE with 5GC 6) HO between FR1 and FR2 | NCR-MT |
| 9. RRC | RRC processing time | 2) RRC connection resume without SCell addition/release and SCG establishment/modification/release 6) RRC connection reconfiguration with SCell addition/release or SCG establishment/modification/release 7) RRC connection resume 9) Counter check | NCR-MT; IAB-MT and (e)RedCap for SCell/SCG related processing time |

Therefore, instead of reducing capabilities from mandatory features supported by high-end device, it is more desirable to define the minimum mandatory capabilities from 6GR Day 1. From RAN2 point of view, higher layer parameters/factors can be further studied.

Proposal 7: RAN2 to study higher layer parameters/factors of minimum mandatory capabilities.

# Conclusion

Lesson learnt from 5G (1): Due to the late start of UE capability discussion, there’s limited time for cross-WG coordination for an efficient and proper UE capability design (e.g., postpone of supporting a useful feature, unclear definition of granularity, etc).

Proposal 1: Study UE capability management, framework, signalling framework design as early as possible with tight and efficient coordination cross WGs (RAN1/4, RAN3/SA2).

Lesson learnt from 5G (2): Incomplete UE capability management solution in Day 1 may bring drawbacks to later releases, e.g., duplicate reporting over air interface, forward-compatibility issue, etc.

Proposal 2: Study UE capability management (e.g., entities storing UE radio capability, UE capability exchange between gNB and CN, UE capability exchange between gNBs, etc) based on the following principles:

- Support forward-compatibility for UE capability exchange between nodes;

- Minimize the frequency of UE reports its full capability during UE’s mobility;

- Minimize duplicate reporting of the same UE capability over the air interface.

Consider 5GNR UE capability management and retrieval framework as starting point for 6GR, including:

- Storing UE static capability at CN;

- RAN obtains UE capability from CN when available;

- RAN obtains UE capability from UE only when CN stored UE capability is not available or not complete.

Lessons learnt from 5G (3): During 5GNR practice, there are some cases that network configurations are mismatch with UE capability (e.g., some capabilities don’t have any configuration defined, some capabilities are reported unnecessarily).

Lesson learnt from 5G (4): Temporary restriction on UE capability reporting in 5GNR misused UAI without clear network behaviour specified, which raises a risk for those capabilities being ignored by the network.

Proposal 3: UE capability framework design should follow the below principles:

- UE capabilities with signalling should be defined only if there’s a corresponding configuration provided by the network;

- UE should be aware of network supported features before UE capability reporting;

- UE should be allowed to perform AS-triggered UE capability update without network request, instead of using UAI;

- 6GNR UE capability granularity level should support sufficient UE flexibility based on the real deployment needs. 5GNR UE capability granularity levels are considered as starting point for further analysis;

Lesson learnt from 5G (5):

- Unnecessary political discussion on mandatory with capability signalling and optional with capability signalling.

- Complex approaches of defining reduced capabilities requires readers to have the basic understanding of the capability framework, in order to understand what capabilities are supported/not supported for different features/device types (if any).

Lesson learnt from 5G (6): 5GNR introduced different mechanisms to reduce capability message size, but didn’t efficiently and fundamentally solve the issue.

Proposal 4: Study 6GR UE capability signalling framework in the following aspects:

- Signalling categories (i.e., mandatory/optional with/without capability signalling) for 6GR, aiming for 1) a harmonized design for minimum mandatory capabilities of different device types (if any), 2) easy readability to understand what capabilities should be supported in order to support a device type (if any)/feature;

- Simplified method to reduce UE capability message size (e.g., signalling simplification, overhead reduction, avoid unnecessary capability reporting, etc);

Lesson learnt from 5G (7): Different components of UE capability are spread across different specifications, making it difficult to connect between signalling structure and field description and difficult to maintain for cross-spec alignment.

Proposal 5: Study the specification structure between 331/306/822 based on the following principles:

- Avoid split between ASN.1 and field description;

- Easy approach to identify capabilities (mandatory/optional) introduced for different features/device types (if any) in each release.

Lesson learnt from 5G (8): 6GR should avoid introducing multiple solutions for feature/device type (if any) early identification.

Proposal 6: Based on 6GR initial access procedure, study 1) how and from which step to differentiate device types (if any) within the harmonized system framework; 2) a unified and scalable approach for early feature identification with different purposes (e.g., early service identification, early capability acquisition, early information acquisition during state transition).

Lesson Learnt from 4G (9): Defining UE categories only by achievable throughput is not only challenging to introducing new UE categories in later releases due to marketing competition and singalling overhead, but likely to leave market fragmentation as well.

Lesson learnt from 5G (10): Introducing the lower end device type later than commercial devices limits the support of low-end device type.

Proposal 7: RAN2 to study higher layer parameters/factors of minimum mandatory capabilities.

# Reference

[1] RP-252912	Revised SID: Study on 6G Radio	NTT Docomo

[2] S2-184608	LS on optimisation of UE capability signalling

[3] TS 38.306	NR; User Equipment (UE) radio access capabilities

[4] 2025 iiMedia Research

[5] 2024.2 iResearch Inc.

[6] TS 38.331	NR; Radio Resource Control (RRC)

[7] TR 38.822	NR; User Equipment (UE) feature list

[8] R2-2506772	General considerations on RAN2 6G study

[9] TR 38.875	Study on support of reduced capability NR devices

[10] RP-252873	Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)

# Annex: A List of examples for UE assistance information related to UE AS capability change

| Area | Information item | Direct Relevant UE capability |
| --- | --- | --- |
| Overheating Related | reducedMaxMIMOLayers-FRx) | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | reducedMaxBW-FRx | supportedBandwidthDL supportedBandwidthUL |
|  | reducedMaxCC | ca-BandwidthClassDL-NR ca-BandwidthClassUL-NR |
| MUSIM Related | musim-MIMO-Layers-DL/UL | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | musim-SupportedBandwidth-DL/UL | supportedBandwidthDL supportedBandwidthUL |
|  | musim-AvoidedBandsList | supportedBandList supportedBandCombinationList |
| IDC Related | IDC assistance information (idc-Assistance idc-FDM-Assistance idc-TDM-Assistance) | supportedBandList supportedBandCombinationList |
| Power Saving Related | maxMIMO-LayerPreferenceConfig | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | maxCC-Preference | ca-BandwidthClassDL-NR ca-BandwidthClassUL-NR |
|  | maxBW-Preference maxBW-PreferenceFR2-2 | supportedBandwidthDL supportedBandwidthUL |
|  | MinSchedulingOffsetPreference/ MinSchedulingOffsetPreferenceExt | crossSlotScheduling-r16 dynamicIndicationSchedulingRestriction-r18 |
| AI/ML Related | applicabilityReportList | aiml-CSI-PredictionPerBC aiml-CSI-Prediction aiml-BM-Case1 aiml-BM-Case |