3GPP TSG RAN WG2 Meeting #131bis	R2-2506774

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	10.3.2

Source:	Xiaomi

Title:	Discussion on 6GR control plane protocol design

Document for:	Discussion and Decision

# Introduction

This contribution discusses the control plane protocol design in 6GR, based on the following objective in 6G SID [1]:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# Discussion

It is assumed that there is still an RRC protocol layer which is responsible for control plane functionalities and related access stratum (AS) signalling exchange in 6GR radio interface between the UE and network. In this section, necessary aspects to be considered for the study of 6GR RRC design are discussed with the general principles for the study proposed.

## Consideration on RRC states in 6GR

The first thing to decide is how many RRC states to define in 6GR and the corresponding UE behaviour to be operated by the UE and network for each RRC state.

Lesson learnt from 5G (1): Definition of an RRC state is characterized by the UE behaviours needed, and the difference on UE behaviours in different RRC states determines how many RRC states to define.

In 5G NR, there are three RRC states, RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. A general reason to have different RRC states is that each RRC state has its associated UE behaviours which are different with each other. This can be seen from the list of UE behaviours cited from [2] as follows.

Table 1: Functionality and UE behaviour in 5G NR for each RRC state [2]

- RRC_IDLE: - A UE specific DRX may be configured by upper layers; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast; - UE controlled mobility based on network configuration; - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - Monitors a Paging channel for CN paging using 5G-S-TMSI, except if the UE is acting as a L2 U2N Remote UE; - If configured by upper layers for MBS multicast reception, monitors a Paging channel for CN paging using TMGI; - Performs neighbouring cell measurements and cell (re-)selection; - Performs measurements on L2 U2N Relay UEs and relay (re-)selection; - Acquires system information and can send SI request (if configured); - Performs logging of available measurements together with location and time for logged measurement configured UEs; - Performs idle/inactive measurements for idle/inactive measurement configured UEs; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data. - RRC_INACTIVE: - A UE specific DRX may be configured by upper layers or by RRC layer; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for PTM transmission of MBS multicast; - UE controlled mobility based on network configuration; - The UE stores the UE Inactive AS context; - A RAN-based notification area is configured by RRC layer; - Transfer of unicast data and/or signalling to/from UE over radio bearers configured for SDT. - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI except if the UE is acting as a L2 U2N Remote UE; - While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE; - If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, monitors a Paging channel for paging using TMGI; - Performs neighbouring cell measurements and cell (re-)selection; - Performs measurements on L2 U2N Relay UEs and relay (re-)selection; - Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area; - Acquires system information and, while SDT procedure is not ongoing, can send SI request (if configured); - While SDT procedure is not ongoing, performs logging of available measurements together with location and time for logged measurement configured UEs; - While SDT procedure is not ongoing, performs idle/inactive measurements for idle/inactive measurement configured UEs; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data; - If configured for MBS multicast reception in RRC_INACTIVE, acquires multicast MCCH change notification and MBS multicast control information and data; - Transmits SRS for Positioning. - RRC_CONNECTED: - The UE stores the AS context; - Transfer of unicast data to/from UE; - Transfer of MBS multicast data to UE; - At lower layers, the UE may be configured with a UE specific DRX; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for MBS multicast; - At lower layers, the UE may be configured with a cell specific cell DTX/DRX; - For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth; - For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth; - Network controlled mobility within NR, to/from E-UTRA, and to UTRA-FDD; - Network controlled mobility (path switch) between a serving cell and a L2 U2N Relay UE, or vice versa, or between a source L2 U2N Relay UE and a target L2 U2N Relay UE; - Network controlled MP operation. - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured; - Monitors control channels associated with the shared data channel to determine if data is scheduled for it; - Provides channel quality and feedback information; - Performs neighbouring cell and/or L2 U2N relay measurements and measurement reporting; - Acquires system information; - Performs immediate MDT measurement together with available location reporting; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data.

Need of RRC_CONNECTED/RRC_IDLE like states in 6GR

It can be first seen from the above Table 1 that the UE behaviours for RRC_CONNECTED and RRC_IDLE in 5G NR are quite different. For example, RRC_CONNECTED requires a network-controlled mobility operation (i.e. handover or LTM) by the UE which is required to perform measurement and reporting, whereas RRC_IDLE only needs the UE autonomous measurements based on which cell (re)selection is performed as the mobility operation. As another example, RRC_IDLE state requires the UE to monitor paging for the network reachability, whereas a UE in RRC_CONNECTED state is consistently maintaining the RRC connection with the network and can be tracked by the network instantaneously.

Similar to above RRC_CONNECTED and RRC_IDLE in 5G NR, at least two RRC states are also needed in 6GR with the different intended UE behaviours:

- one of the states has the similar UE behaviours as in RRC_IDLE state in 5G NR, where the network does not have the UE AS context stored but the NW reachability of a UE should be ensured;

- the other state holds similarity to 5G NR RRC_CONNECTED state where the UE establishes the RRC connection with the RAN and the complete UE AS context is maintained at RAN and UE, enabling the continuous data/signalling transfer to/from the UE.

These two states correspond to the “battery efficient state (e.g. Idle state)” and the “Active state” respectively, as in 6G TPR [3] (see below citation). RAN2 needs to further study, for these two RRC states in 6GR, whether all the relevant UE behaviours in 5G NR are needed (for example, not supporting those behaviours due to non-commercialized features like MBS, SL Relay, etc., as grey highlighted in Table 1), and also consider the potential enhancements on top of the 5G NR with respect to e.g., signalling overhead, power/energy saving, resource utilization, etc. as discussed in the later sections of this paper.

Observation 1: At least two RRC states, corresponding to the “battery efficient state (e.g. Idle state)” and “Active state” specified in 6G TPR, are needed in 6G NR, with the intended UE behaviours associated with each state similar to those for RRC_CONNECTED and RRC_IDLE in 5G NR.

Need of an RRC_INACTIVE like state in 6GR

In 5G NR, the RRC_INACTIVE state was specified in order for a fast state transition to RRC_CONNECTED state, while also enabling power-efficient UE operations. Whether a similar RRC state needs to be specified in 6GR needs to be studied.

As per the 6G TPR cited above, a 20 ms CP latency is required for the UE to enter the “Active state”. The “Active state” here should refer to the RRC_CONNECTED like state, as it points to the “continuous data transfer” which certainly requires the complete UE AS context available at both UE side and RAN side. Technically speaking, it would be difficult (if possible at all) to bound this CP latency requirement within 20 ms, if the UE enters the RRC_CONNECTED state from RRC_IDLE.

To see this, a high-level evaluation on the CP latency needed for RRC IDLE to RRC CONNECTED transition in 5G NR is taken as an example as follows (based on the RRC setup procedure in [4], the related component latency values provided in [5] and related RRC processing latency in [2]):

Table 2: Evaluation on CP latency for RRC_IDLE to RRC_CONNECTED transition in 5G NR

| Procedure | Delay Components | Subtotal |
| --- | --- | --- |
| RRC connection procedure (in Uu) | RRC Setup Request: - Processing Latency [5] - UE: TUEProc-RRCULInfo - gNB: TgNBProc-RRC - Signalling propagation delay: TUE-gNB [5] | 18 ms - 19 ms |
|  | RRC Setup and RRC Setup Complete: - Processing Latency - UE: delay value for RRCSetup procedure in [2, Table 12.1-1] - gNB: TgNBProc-RRC [5] - Signalling propagation delay: TUE-gNB [5] |  |
| Initial UE context setup (in NG, incl. initial UE msg transfer) | - Processing Latency - gNB: 3* TgNBProc-NAS/LPP [5] - AMF: 3* TAMFProc [5] - Signalling propagation delay: 3* TgNB-AMF [5] | 27 ms - 48 ms |
| Initial security activation (in Uu, SMC) | - Processing Latency - UE: delay value for Initial AS security activation procedure in [2, Table 12.1-1] - gNB: TgNBProc-RRC [5] - Signalling propagation delay: TUE-gNB [5] | 8 ms - 8.5 ms |
| RRC Reconfiguration (in Uu) | - Processing Latency: - UE: TUEProc-RRCReconf [5] (which is aligned with the value in [2, Table 12.1-1]) - gNB: TgNBProc-RRC [5] - Signalling propagation delay: TUE-gNB [5] | 13 ms - 13.5 ms |
| Total Estimated CP latency |  | 66 ms ~ 89 ms |

Based on the above evaluation, at least about 60 ms (i.e. 66 ms in above example) delay is needed for the UE to transit from RRC_IDLE to RRC_CONNECTED state, and this definitely cannot satisfy the 20 ms CP latency requirement in [3], even if the processing latency can be further reduced in 6G.

Referencing the above evaluation based on 5G NR, it is envisioned that the similar situation would happen, if a UE enters the “Active state” from the “Idle state” in [3] from the scratch. To this end, besides the RRC_IDLE-like and RRC_CONNECTED-like RRC states as in Observation 1, further study on an RRC state in 6GR enabling a fast RRC state transition is needed to meet the 20 ms CP latency requirement.

Observation 2: An RRC state in 6GR is needed to meet the 20 ms control plane latency requirement according to 6G TPR conclusion.

It is intuitively undesirable to keep UE always in “Active state”, especially when the UE is having infrequent small data to transmit, because keeping UE always in “Active state” needs significant amount of signalling overhead and RS resources for radio link monitoring/maintenance, which not only is unfriendly to the power consumption to both UE and network, but results in unnecessary resource wastage (e.g. for measurements and reporting).

Lesson learnt from 5G (2): RRC_INACTIVE is a Day-1 mandatory feature that leads to extra complexity to UE and NW, but is unsuccessfully commercialized due to lack of deployment of this feature by network.

In 5G NR, the RRC_INACTIVE state was specified with the purpose to support the fast RRC state transition. Actually, the introduction of the RRC_INACTIVE state (the so called “new state” at the very beginning) was concluded at the very early stage of 5G study item [6], with the aim to not only enable the fast state transition but also a power-efficient operation of the UE with lower performance requirements (e.g. mMTC case).

The support of the RRC_INACTIVE state was specified as a mandatory UE feature from Day-1 in 5G NR. However, the commercialization of RRC_INACTIVE state of NR was not successful, with this feature not actually deployed in the real network. One of the key reasons is that operators fail to find the clear use cases/services that have to depend on the low CP latency and/or power saving gain by deploying the RRC_INACTIVE feature. On the other hand, from a technical perspective, the support of RRC INACTIVE state actually leads to extra implementation complexity at both the UE side and NW side. At the UE side, the RRC resume procedure still results in signalling overhead (not only the RRC level signalling but also the overhead for scheduling and/or random access) in the air interface, and the need of the RNA update introduces even more signalling and the complexity to UE to support the procedure. There is also the complexity for UE implementation to support differential handling of RAN triggered paging and CN triggered paging, and the small data mechanism specific to RRC INACTIVE introduced in later release introduces also complicated UP handling, in terms of e.g. TA handling, retransmission operation, etc. At the NW side, there is the complexity for network implementation on the RNA planning and UE context transfer via Xn.

Learning the lessons from 5G NR, design of an RRC state in 6GR enabling the fast RRC state transition needs to be well justified in terms of e.g. benefits, use case/scenarios, etc., and also it should support a power-efficient operation for the UE and be friendly to both UE and network implementation.

Observation 3: In 6GR, if the RRC state to support the fast state transition (e.g. similar to RRC_INACTIVE in 5G NR) is supported, it should be of power-efficient operations and be friendly to UE/network implementation.

Similar to above discussion for RRC_CONNECTED and RRC_IDLE, RAN2 needs to carefully study which UE behaviour in RRC_INACTIVE in 5G NR is needed in 6GR, and whether any enhancements are needed. Despite the 5G NR RRC_INACTIVE design, the key issue to constrain the CP latency within 20 ms, as observed from the evaluation in above Table 2, is to reduce the steps of signalling exchange as much as possible, especially saving the steps for establishment of the RAN-CN connections for the UE and UE AS contexts from the scratch. Therefore, the study of the 6GR RRC state enabling fast state transition (if regarded as necessary) should be characterized as follows from the functionality perspective:

- Enabling the fast state transition meeting at least the 20 ms CP latency requirements, with CN-RAN connection and UE AS contexts being kept at the network and UE side but not established again;

- Enabling low-power operation for UE implementation, e.g. only UE autonomous mobility, no measurement reporting, etc.

- Low-complexity for UE/NW implementation, e.g. no complicated RNA maintenance procedure, no complicated Xn/NG interface exchange, etc.

Based on the above analyses, it is proposed that RAN2 studies the RRC states supported in 6GR and the intended UE behaviour associated with each RRC state, taking the 5G NR RRC states and associated UE behaviours as the starting point and aiming to meet the 20 ms CP latency requirement:

Proposal 1: In 6GR RAN2 study on RRC states, RAN2 needs to first study what the necessary UE behaviours are to support the low power consumption, low UE/network complexity operations, considering the scenarios of “no active traffic” and “with active traffic”. The study can take 5G NR RRC states and associated UE behaviours as the starting point and target to meet the 20 ms CP latency requirement, and based on this study, RAN2 further decides how many RRC states are needed for 6GR.

## Consideration on cell management and UE AS context handling in 6GR

Lesson learnt from 5G (3): Cell definition is used in 5G NR and previous generations of 3GPP systems, but cell management in 5G NR cannot meet the new requirement in 6GR on flexible utilization of spectrum resources.

In 5G NR study item phase, cell management and cell definition were extensively discussed in RAN2. The discussions at that time were carried out from a higher layer perspective, related to e.g., mobility, beam handling, etc.

Cell management and related UE AS context handling also need be studied in RAN2 for 6GR, considering their potential impacts to RAN higher layer design. The first question to answer is whether the “Cell” concept is still needed, and how a 6GR cell is defined.

A Cell definition was specified in [7] from a terminology point of view and specified in [4] for a 5G NR cell.

TS 21.905 [7]

Cell: Radio network object that can be uniquely identified by a User Equipment from a (cell) identification that is broadcasted over a geographical area from one UTRAN Access Point. A Cell is either FDD or TDD mode

TS 38.300 [4]

Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

Based on the above citations, it is observed that in general a “Cell” refers to the geographical area for which the network provides coverage and functions as a unit of radio network to configure the UL/DL radio resources.

Therefore, conceptually a “Cell” in 5G NR can be generalized as a geographical area where the network provides UEs with the valid radio resource configurations to be used and controls UEs’ behaviour for the communication with the network within the area . Specifically, a Cell in 5G NR is characterized by the following aspects within its coverage area, as summarized in below Table 3:

Table 3: Cell characteristics in 5G NR

- identified by a Cell identifier in RAN (e.g. PCI in PHY and CGI in RAN higher layer). - deployed on a certain UL (optionally) carrier frequency and DL carrier frequency located on a specific frequency band. - Multiple cells able to be served by the same gNB. - covered/served by multiple beams/TRPs that is associated with and uniquely identified within the cell. - broadcasting common/non-UE-specific reference signal for UE to discover and synchronize to the NW, and broadcast common control information for UE initial access to the NW; - the anchor point (i.e. PCell) where a UE AS context is maintained - as UE moves within a cell, only switching of TRP/beam-level to serve the UE is needed without any handover or LTM like mobility operation. - Unit for Load balance and access control across the coverage area by the network.

In 6GR, it is still needed to have some forms of geographical “area”, where UEs can discover and synchronize to the RAN providing the coverage, acquire the associated radio resource configuration usable within the area, perform access to the network (e.g. paging, system information, initial access, etc.) and then carry out the user plane data transfer with the network with UE specific configurations. Such an “area” should also be managed for the 6G RAN to perform load balancing and access control across the areas under its coverage. As the UE moves across different such “areas”, it needs to identify in which specific “area” it is and performs the UE autonomous or network controlled mobility operations to get the associated radio resource configurations in a timely way.

Therefore, a “Cell” concept is still needed in 6G RAN. The 6G “Cell” concept could in general cover the above characteristics listed for a 5G NR Cell in Table 3; but at least the character “•	deployed on a certain UL (optionally) and DL carrier frequency located on a specific frequency band.” cannot meet the new requirements defined for 6GR as follows:

- Note 1: There are the following two requirements regarding 6GR spectrum utilization as per the RAN#109 conclusion. [8][9]

- Note 2: These two new requirements not only impact architecture and cell concept, but also impact the cell (re)selection, initial access (see the discussion in 2.3), etc.

Table 4: Requirement on spectrum utilization concluded in RAN#109 [8][9]

RP-252950 Moderator summary on spectrum utilization requirements Huawei Replaces RP-252917 conclusion: proposal 1 & 2 are endorsed

RP-252950 [9] Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

Due to the above two requirements, RAN2 should study the concept of the 6G “Cell” additionally taking into account the following two aspects:

- Whether a 6GR Cell can comprise multiple carrier frequencies across one or multiple frequency bands;

- Whether a 6G can include the flexible combinations of the DL frequencies and UL frequencies that are not necessarily located on the same frequency band.

Therefore, RAN2 needs to study the cell management with the 6GR cell concept, covering the 5G NR Cell characteristics in Table 3 and the spectrum utilization requirements in [9].

Lesson learnt from 5G (4): UE AS context management depends on the architecture, RRC state and mobility designs.

In 5G NR, the UE AS context is stored and managed in the gNB, and several features have impacts on how it is managed. For example, for RRC_INACTIVE state, the UE AS context is stored in the last serving gNB, and the latest gNB can perform a retrieve the UE AS context from the last serving gNB, when the RRC resume procedure is triggered by the UE. As another example, the UE AS context is transferred between the gNBs or between the gNB and AMF via related Xn or NG interface.

In 6GR, the UE AS context management is likely to depend on the related functionality design, at least on the design on RRC state and mobility operations. Therefore, RAN2 needs to further study how the UE AS context management w.r.t. RRC state and mobility discussions.

Proposal 2: Study the 6G Cell concept, taking into account the 5G NR Cell characteristics listed in Table 3 and new requirements on flexible spectrum utilization. Also study what UE AS context should be maintained and how it is maintained/managed in 6GR, considering the progress on relevant functionalities (e.g. architecture, RRC state, mobility operation, etc.).

## Consideration on initial access design in 6GR

In this subclause, initial access design in 6GR is discussed from two perspectives, i.e. study on a resource-efficient and energy-efficient design for initial access procedure and study on a unified signalling design during initial access

Study on a resource-efficient and energy-efficient design for initial access procedure

Lesson learnt from 5G (5): The “always-on” common signal/control information in 5G NR is inefficient from the perspective of system overhead and network energy consumption.

Generally speaking, common signal/control information needs to be broadcast by the RAN to provide necessary synchronization information, and resource configuration to be used by UEs for initial access to the network. In 5G NR, the common signal/control information is provided in the form of “always-on” signal/information, mainly concerning SSB, MIB and SIB1. More specifically, such “always-on” common signal/control information will be transmitted on each frequency intended to provide PCell access for UEs.

This way that 5G NR provides the common signal/control information is not efficient enough from signalling overhead and network energy efficiency perspective, especially in RAN with multi-carrier deployment. Specifically, for the frequency(ies) that can share the same synchronization reference (e.g. intra-frequencies/adjacent inter-frequencies, frequencies with collocated gNB deployment, etc.), such common signal/control information can be broadcast on a specific frequency which functions as the reference frequency for synchronization and MIB1/SIB1 acquisition, thus providing the SSB for synchronization and resources configurations for initial access (e.g. PRACH configuration, common search space configuration, etc.) applicable on the other frequencies. Then, for the other frequencies than the reference, such common signal/information no longer needs to be broadcast in an “always-on” manner, thus saving the signalling overhead, corresponding resource consumption and also network energy consumption. This has been reflected in RAN new requirements for 6GR on “flexible utilization of spectrum resources for DL and UL over different carriers/bands” (See Table 4).

Although, in Rel-19, NR NES WI introduced the OD-SSB and OD-SIB1 mechanisms intended to save the signalling overhead and network energy consumption for SSB and SIB1, they have their own limits: OD-SSB mechanism is applicable only for the SCells in CA and thus unable to be applied to the initial access phase towards the PCell; OD-SIB1 mechanism, if deployed, may result in degradation of the initial access performance of the legacy UEs.

Observation 4: The OD-SSB and OD-SIB1 mechanisms introduced in later releases in 5G NR may not be effective/sufficient to save the “always-on” common signal/control information and corresponding energy consumption due to their own limitations.

Lesson learnt from 5G (6): Resource configurations related to initial access (e.g. RACH resources, CSS for paging/SI, etc.) are inefficiently designed w.r.t. resource utilization and network energy consumption.

Regarding the initial access related resource configurations themselves, provided in MIB/SIB1, the related designs in current 5G NR are sub-optimal also from the resource utilization and network energy efficiency perspective:

- As an example on RACH related resource configuration, except for SUL (which actually aims at coverage extension), the UE can only use the PRACH resources and common search space (CSS) for random access on the UL carrier associated to its serving cell to initiate the initial access, but cannot use the PRACH resources/CSS for random access on another UL carrier, even if the UL carrier associated with its serving cell is suffering from access congestion but the other UL carrier is much under-loaded.

- As another example, UE has to monitor the CSS for paging/SI (e.g. other SIs than SIB1) on its serving cell/frequency for paging and SI reception. As a result, in the network deployment that supports multiple carrier frequencies as the serving frequencies for UEs to camp on, the network has to configure the CSS for paging/SI on each of these frequencies. Similar to above discussion on the “always-on” common signal/control information, if, in such a multi-carrier deployment, the CSS for paging/SI across multiple carrier frequencies were concentrated on a specific frequency(ies), e.g. the reference frequency, the resources corresponding to the CSS for paging/SI could then be saved on the other carrier frequencies and configured for other use, improving the resource utilization. Also, with paging/SI concentrated on a specific frequency(ies), network energy saving gain can also be expected by not necessarily signalling paging/SI on every frequency.

Observation 5: In 5G NR, RACH and CSS related resource configurations have the following limits in the deployment with multiple carrier frequencies:

- the initial access is unable to be offloaded among the PRACH/CSS resources on multiple frequencies, when congestion happens on a certain frequency;

- the CSS for paging/SI may need to be configured on every carrier frequency deployed as serving frequencies, but could have been concentrated in a specific frequency(ies) to improve resource utilization and network energy efficiency.

Despite the above limitations, the initial access design in 5G NR, including the RACH and CSS related procedure/configurations, cannot meet RAN#109 agreed requirements for the multiple carrier/band utilization “6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR” [9] (See above Table 4).

Observation 6: The initial access related design (related to e.g. RACH and CSS for SI/paging) in 5G NR cannot meet the 6G requirement on spectrum utilization agreed in RAN#109 “6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR”.

With above lessons learnt from 5G NR, RAN2 needs to carry out the study of the initial access mechanism, considering the below aspects from 6G Day-1:

- a signalling- and energy-efficient design for the common signal/control information, aiming at avoiding the “always-on” signal/control information whenever possible.

- a resource-efficient design that enables the flexible utilization on the resource configurations related to initial access.

Besides, This study depends on RAN1 progress for common signalling/control information design in the multiple frequency deployment (e.g. Synchronization signal design, feasibility of a reference carrier in inter-/intra-band scenarios, etc.).

Proposal 3: Study the initial access mechanism in 6GR, considering a signalling-/energy-efficient “always-on” common signal/control information design and improved resource utilization for initial access related resource configurations (e.g. PRACH resources, CSS for paging/random access/SI, etc.). Coordinate with RAN1 is needed on this study.

Study on a unified signalling design during initial access

Lesson learnt from 5G (7): In 5G NR, the differentiated message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access leads to complexity for UE implementation.

At the early stage of 5G NR, it was concluded [11][10] that there are two Msg3 message sizes supported during initial access, i.e. 56 bits and 72 bits, which result in two UL logical channels defined for Msg3 transmission, i.e. CCCH and CCCH1, respectively. The 72-bit Msg3 message size was defined specifically to include the RRCResumeRequest1, which includes the full I-RNTI that results in a Msg3 size more than 56 bits (corresponding to RRCResumeRequest/RRCSetupRequest/RRCReestablishmentRequest).

Dating back to the discussions in [11][10], the main reason for such a differential design on CCCH and CCCH1 is the concern on the coverage of Msg3 transmission during initial access phase. Some companies at that time were arguing that the coverage of 5G NR should at least not be worse than LTE, so 5G NR should take the 56-bit Msg3 size, same as LTE, as the baseline. Finally, it was agreed that the supported size of the RRC resume Request message will be indicated in the SIB1 [11], and so it is actually left to the network on whether/how to support the bigger Msg3 size, i.e. 72 bits, with the coverage of Msg3 transmission not impacted.

From the UE perspective, such differentiated Msg3 size and CCCH design is not favourable, as this increases complexity for UE implementation. Alternatively, to support a unified Msg3 size (applicable to all kinds of UL RRC signalling during initial access phase) with coverage also guaranteed is much more friendly to UE implementation. Actually, in the later Releases of 5G NR (i.e. Rel-17), some coverage enhancement mechanisms (e.g. Msg.3 repetition) were specified to enhance Msg3 coverage, and they can function as the candidate solutions that enable a bigger Msg3 size without impacting the Msg3 coverage. Then, with a bigger Msg3 size enabled from coverage perspective, a unified Msg.3 size that can accommodate all kinds of Msg.3 payload can be expected, which further enables the unified UL logical channel design used for initial access. Although these coverage enhancement designs related to initial access are not likely to be deployed in 5G NR due to the introduction at a late stage (w.r.t. the concern on the coverage of legacy UEs), they are technically feasible candidates for the support of a unified Msg3 size and UL logical channel design for the initial access, if they are taken into account from Day1 in 6GR.

Therefore, it is proposed that the study on initial access in RAN2 should aim at a uniform design on the message size and logical channel for the RRC signalling during initial access phase to ease UE implementation. This needs coordination with RAN1 on how to address the coverage issue during initial access.

Proposal 4: Study a uniform design on the message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access to ease UE implementation (considering RAN1 progress on the coverage during initial access).

## Consideration on paging and system information design in 6GR

Discussions on 6GR Paging design

Lesson learnt from 5G (8): Too-many pre-alter signal designs for paging power saving lead to complicated UE implementation and difficulty for operators to deploy.

Power saving is always a design target in each generation, and for idle state UEs, this is mainly concerned with paging monitoring. In 5G NR, i-DRX is the fundamental mechanism for UE to monitor paging message. We believe this should be inherited in 6GR, as continuous paging monitoring clearly would waste more UE power than duty cycle based monitoring. 5G NR also further optimized UE power consumption on paging monitoring, and various pre-alert signals before PO monitoring have been introduced in different releases, e.g. PEI which is based on PDCCH signal and LP-WUS which can be detected using low power receiver. On the one hand, supporting these different pre-alert signals obviously increases UE’s complexity; on the other hand, it also makes it difficult for operators to deploy, due to the reason that in the worst case network may need to upgrade and support all these functions as they do not know for each function how many idle state UEs in their network have supported. In 6GR, we need to be careful to not introduce too many pre-alert signals.

Observation 7: Too many pre-alert signals (e.g. PEI and LP-WUS) introduced in 5G NR complicate UE implementation. 6G NR should aim at a single pre-alert signal for paging reception.

Lesson learnt from 5G (9): Paging adaptation introduced in 5G NR brings trivial network energy saving gain due to introduction at a late stage.

Related to paging transmission, in early 5G NR, time-domain evenly distributed paging occasions were specified for network to distribute the paging load. In later NES WI, paging adaptation was introduce to have clustered paging occasions for better network energy saving. However, due to the existence of legacy UEs, network has to support both kinds of paging mechanism and this actually would cause even more network energy consumption. In 6GR, we do not have legacy UEs anymore, and a cleaner paging adaptation scheme should be the design target.

Observation 8: Due to legacy UEs, 5G NR NES has to introduce addition paging adaptation for NES. Two sets of paging mechanism (evenly distributed and clustered resources) actually do not save much network energy. 6GR study should aim at a cleaner paging adaptation scheme.

In 5G NR, UE power saving and network energy saving for paging reception are designed and enhanced separately in different releases, and joint power saving was not fully considered. In 6GR, we should aim at a joint power saving design for paging reception.

Discussions on 6GR System information design

Lesson learnt from 5G (10): On-demand SI mechanisms introduce in 5G NR (i.e. OD-SI/OD-SIB1) intends for NES purpose, but are unfriendly to UE power saving and/or difficult for operators to deploy.

In 5G NR early release, other SI which is not essential SI can be provided on demand. In NES WI, SIB1 can also be provided on demand. Clearly, on-demand SI and on-demand SIB1 are beneficial for network energy saving, but it also causes more UE power consumption and acquisition latency due to having to trigger RACH procedure. There should be a trade-off in between, for example: when some SI is essential for UE to have quick cell access, on-demand request and response may not be the best way and in this case network energy saving may give ways to providing shorter communication latency. One of the drawbacks of OD-SIB1 in 5G NR is that, due to the existence of legacy UEs, when OD-SIB1 is not provided, network has to bar legacy UEs, and this makes it difficult to deploy OD-SIB1 cells in coverage layers.

In any case, 6GR SI design should carefully study how to broadcast SI, taking NES, UE power consumption and SIB acquisition latency into account.

Based on the above discussions, the proposal is given as follows for paging and SI design in 6GR.

Proposal 5: 6GR should study the paging mechanism taking both UE power saving (e.g. single pre-alter signal) and network energy saving (e.g. cleaner paging adaptation mechanism) into account, and study SI broadcasting schemes, taking NES, UE power consumption and SIB acquisition latency into account.

## Consideration on UE assistance information design in 6GR

Lesson learnt from 5G (11): UAI in 5G NR is misused to include UE AS capability change related information, and the preference related info is not useful due to lack of NW behaviour on how it is used.

In 5G NR, UE assistance information (UAI) was specified, aiming at providing the RAN with related UE side information to facilitate network configurations. However, despite the non-trivial standards efforts, the UAI feature in 5G NR is not well supported due to the lack of clarified/specified NW behaviour on whether/how the received UAI is really used, which can be observed from the following two perspective:

- Some UE assistance information actually reflects the UE’s intention for a change of relevant UE AS capability, but not really a simple UE preference on related configurations and/or scheduling that may or may not be considered by the network.

For such kind of information, we propose to include it in the UE capability reporting, as in our paper [12].

- Other UE assistance information is not tightly associated with any specific UE AS capability, and is mainly related to UE preferences on network configuration parameters and scheduling.

Such information was introduced to adjust/optimize the network configurations or scheduling strategy, but may not be as useful as intended due to the ignorance of network, which further results in the lack of motivation from the UE implementation of this type of assistance information. To make such UE assistance information reported really useful, the bottom line would be to let network respect the received information, and specify this in a high-level manner.

Based on above analyse, 6GR can still study the need of a UAI framework which enables the UE to report non-capability related preference information to facilitate NW configuration, and considers to specify that network should respect/follow the received UAI reported from the UE for the adjustment of network configuration.

Proposal 6: Study the UAI framework in 6GR, with the general principle that it only includes the information irrelevant to UE AS capability change and with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration.

## Consideration on overall ASN.1 improvement in 6GR

Lesson learnt from 5G (12): 5G NR ASN.1 design for RRC configuration has shortcomings w.r.t. RRC msg processing efficiency, being unfriendly for UE implementation and failure handling with poor future proof.

In 5G NR, the dedicated configuration to UEs, after the RRC connection is established, are provided/modified/released to the UE via an RRC reconfiguration message. Specifically, the RRC reconfiguration in NR Uu interface is designed in a way that for the enhanced features introduced release after release from Rel-15, any related configurations are added into this single RRC reconfiguration message by means of extension.

However, such an “all-in-one” RRC reconfiguration framework in 5G NR may not be an optimal design with some aspects needing further improvements as follows:

- Inefficient processing of the RRC reconfiguration message.

Since everything is squeezed into one message, the UE has to decode and process all the parameters inside the RRC message, before it can really apply any of them. Especially for the fields/IEs related to the features not supported by the UE, the UE still has to first process all of them, even though they are set as “absent” with 0-value optional bits. This is certainly not an efficient way, since the UE should have been allowed to not process those fields/IEs it does not support at all.

- Unfriendly design from the perspectives of system signalling overhead and UE implementation complexity.

From system overhead point of view, even for the fields belonging to a feature that is not supported by the UE, the current RRC reconfiguration in 5G NR in most cases still needs to signal the optional bits with “0” value for each of these fields, respectively. This leads to unnecessarily big signalling overhead, as it should have been possible to signal only one optional bit associated with the feature itself, indicating all the associated fields are absent.

From UE perspective, even if it only intends to support an enhanced feature (or even just one parameter in that feature) introduced in a (rather) later release, it has to implement all the ASN.1 fields/IEs introduced in that release following the current ASN.1 structure, even though those fields/IEs it does not supported will be set to “absent” (i.e. optional bit of “0”). This obviously leads to UE implementation complexity.

- (Re)configuration failure handling with poor future-proof.

For most of the parameters, the RRC reconfiguration procedure in 5G NR today works in a way that if the UE cannot comply with a parameter(s) included in the reconfiguration message, it typically needs to perform RRC reestablishment procedure, possibly selecting another cell to continue its communications. In later releases of 5G NR, however, it was found that the inability to comply happening to some fields/parameters may not impact the basic communication operations, so below NOTE was introduced to avoid unnecessary reestablishment [2]:

NOTE 00: The UE behaviour specified in this clause does not apply to the following, and the UE ignores, i.e. does not take an action on and does not store, the fields that it does not support or does not comprehend: - The fields in ServingCellConfigCommon that are defined in Rel-16 and later. - The fields of searchSpaceMCCH and searchSpaceMTCH in PDCCH-ConfigCommon that are defined in Rel-17 and later.

Actually, such a way in 5G NR to specify reconfiguration handling is not future proof, as it needs to determine the failure handling for every field/parameter introduced in later releases case by case.

The lessons learnt from the RRC reconfiguration design in NR Uu is that it is necessary to improve the ASN.1 design for the RRC configuration signalling design. Actually, the ASN.1 design in 5G NR sidelink positioning actually set a good example on the direction for ASN.1 improvement. In particular, according to [13], NR sidelink positioning already supported parallel ASN.1 modules which encode/decode parameters associated with different feature/feature sets. In this way, the UE can efficiently handle the configuration parameters for each feature/feature set with corresponding ASN.1 module independently, and couple the application of the configuration for one feature/feature set from others. The UE can also implement only the fields of the feature/feature groups it can really support, which ease UE implementation. This can also ease the discussion on failure handling, as the failure handling will be done locally within an ASN.1 module without impacting others, and thus specify in a consistent way based on a per feature/feature group failure handling.

With a comparison between ASN.1 design for NR Uu and that used for NR sidelink positioning, the following observation is derived as the lessons learnt from 5G NR.

Observation 9: By contrast to the RRC Reconfiguration in 5G NR, the ASN.1 design for NR sidelink positioning enables parallel ASN.1 modules for different feature/feature set configurations, which can improve the processing efficiency, ease UE implementation, reduce signaling overhead, be better future proof, etc.

Based on the lessons learnt from 5G NR, it is proposed to study how to improve the RRC message structure and ASN.1 design in 6GR from Day-1, aiming at more efficient processing of the network configurations by the UE with lower UE implementation complexity. For example, a per feature/feature group ASN.1 modularization design can be considered for RRC configurations.

Proposal 7: Study simplified RRC message structure and improved ASN.1 processing in 6GR, enabling UE processing of network configuration with higher processing efficiency and lower UE implementation complexity than 5G NR RRC reconfiguration (e.g. per feature/feature set ASN.1 module).

# Conclusion

In this paper, 6GR CP protocol is discussed. As per lessons learnt from 5G NR, proposals are listed as follows.

RRC State

Lesson learnt from 5G (1): Definition of an RRC state is characterized by the UE behaviours needed, and the difference on UE behaviours in different RRC states determines how many RRC states to define.

Lesson learnt from 5G (2): RRC_INACTIVE is a Day-1 mandatory feature that leads to extra complexity to UE and NW, but is unsuccessfully commercialized due to lack of deployment of this feature by network.

Proposal 1: In 6GR RAN2 study on RRC states, RAN2 needs to first study what the necessary UE behaviours are to support the low power consumption, low UE/network complexity operations, considering the scenarios of “no active traffic” and “with active traffic”. The study can take 5G NR RRC states and associated UE behaviours as the starting point and target to meet the 20 ms CP latency requirement, and based on this study, RAN2 further decides how many RRC states are needed for 6GR.

Cell and UE AS context management

Lesson learnt from 5G (3): Cell definition is used in 5G NR and previous generations of 3GPP systems, but cell management in 5G NR cannot meet the new requirement in 6GR on flexible utilization of spectrum resources.

Lesson learnt from 5G (4): UE AS context management depends on the architecture, RRC state and mobility designs.

Proposal 2: Study the 6G Cell concept, taking into account the 5G NR Cell characteristics listed in Table 3 and new requirements on flexible spectrum utilization. Also study what UE AS context should be maintained and how it is maintained/managed in 6GR, considering the progress on relevant functionalities (e.g. architecture, RRC state, mobility operation, etc.).

Initial access design

Lesson learnt from 5G (5): The “always-on” common signal/control information in 5G NR is inefficient from the perspective of system overhead and network energy consumption.

Lesson learnt from 5G (6): Resource configurations related to initial access (e.g. RACH resources, CSS for paging/SI, etc.) are inefficiently designed w.r.t. resource utilization and network energy consumption.

Proposal 3: Study the initial access mechanism in 6GR, considering a signalling-/energy-efficient “always-on” common signal/control information design and improved resource utilization for initial access related resource configurations (e.g. PRACH resources, CSS for paging/random access/SI, etc.). Coordinate with RAN1 is needed on this study.

Lesson learnt from 5G (7): In 5G NR, the differentiated message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access during initial access leads to complexity for UE implementation.

Proposal 4: Study a uniform design on the message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access to ease UE implementation (considering RAN1 progress on the coverage during initial access).

Paging and SI design

Lesson learnt from 5G (8): Too-many pre-alter signal designs for paging power saving lead to complicated UE implementation and difficulty for operators to deploy.

Lesson learnt from 5G (9): Paging adaptation introduced in 5G NR brings trivial network energy saving gain due to introduction at a late stage.

Lesson learnt from 5G (10): On-demand SI mechanisms introduce in 5G NR (i.e. OD-SI/OD-SIB1) intends for NES purpose, but are unfriendly to UE power saving and/or difficult for operators to deploy.

Proposal 5: 6GR should study the paging mechanism taking both UE power saving (e.g. single pre-alter signal) and network energy saving (e.g. cleaner paging adaptation mechanism) into account, and study SI broadcasting schemes, taking NES, UE power consumption and SIB acquisition latency into account.

UAI

Lesson learnt from 5G (11): UAI in 5G NR is misused to include UE AS capability change related information, and the preference related info is not useful due to lack of NW behaviour on how it is used.

Proposal 6: Study the UAI framework in 6GR, with the general principle that it only includes the information irrelevant to UE AS capability change and with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration.

ASN.1 improvement

Lesson learnt from 5G (12): 5G NR ASN.1 design for RRC configuration has shortcomings w.r.t. RRC msg processing efficiency, being unfriendly for UE implementation and failure handling with poor future proof.

Proposal 7: Study simplified RRC message structure and improved ASN.1 processing in 6GR, enabling UE processing of network configuration with higher processing efficiency and lower UE implementation complexity than 5G NR RRC reconfiguration (e.g. per feature/feature set ASN.1 module).

# Reference

[1]	RP-252912 Revised SID: Study on 6G Radio

[2]	3GPP TS 38.331	NR; Radio Resource Control (RRC) protocol specification

[3]	3GPP TR 38.914	Study on 6G Scenarios and Requirements

[4]	3GPP TS 38.300	NR; NR and NG-RAN Overall Description; Stage 2

[5]	3GPP TR 38.857	Study on NR Positioning Enhancements

[6]	R2-164670 Report of 3GPP TSG RAN WG2 meeting #94

[7]	3GPP TS 21.905	Vocabulary for 3GPP Specifications

[8]	RAN#109 draft meeting report

[9]	RP-252950 Moderator’s summary on spectrum utilization requirements

[10]	R2-1811001 Report of 3GPP TSG RAN2#102 meeting, Busan, Korea

[11]	R2-1811002 Report of 3GPP TSG RAN2 NR AdHoc 1807 meeting, Montreal, Canada

[12]	R2-2506773	General Consideration on 6GR UE Capability

[13]	3GPP TS 38.355	NR; Sidelink Positioning Protocol (SLPP); Protocol specification

Note that it is not our intention that all the UE behaviors listed in the Table must be supported in 6GR Day-1. This is subjected to further discussion. For example, those unlikely supported in 6G Day 1 are highlighted in grey whereas those needing further discussion in yellow.

The minimum value of TUEProc-RRCULInfo in [5, Table 5.2.3.1.2-1] is used for an optimistic estimation for the RRC Setup Request transfer.

For the signaling propagation delays included in this table, the corresponding range defined in [5, Table 5.2.3.1.2-1] is applied.

Since the initial UE msg and initial context setup request/response involves transfer of NAS, the TgNBProc-NAS/LPP in [5, Table 5.2.3.1.2-1] is applied for gNB processing.