3GPP TSG-RAN WG2 Meeting #131bis                                                           R2-2506785

Prague, Czech Republic, Oct. 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1353 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on SONMDT UE Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE capability for “Logged MDT enhancement for Slicing” is optional without signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE capability without signalling is introduced for Logged MDT enhancement for Slicing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capability for Logged MDT enhancement for Slicing is not suported in R19 TS 38.306. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.7	MDT and SON features

Definitions for feature

Cross RAT RLF Report It is optional for UE to support the delivery of EUTRA RLF report to an NR node upon request from the network.

Logged MDT for Slicing It is optional for UE to support Logged MDT for Slicing.

Mobility history information storage It is optional for UE to support the storage of PCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9].

Radio Link Failure Report for inter-RAT MRO EUTRA It is optional for UE to support: - Inclusion of EUTRA CGI and associated TAC, if available, and otherwise to include the physical cell identity and carrier frequency of the target PCell of the failed handover as failedPCellId in RLF-Report upon request from the network as specified in TS 38.331 [9]. - Inclusion of EUTRA CGI and associated TAC as previousPCellId in RLF-Report as specified in TS 38.331 [9]. - Inclusion of eutraReconnectCellId in reconnectCellId in the RLF-Report as specified in TS 38.331 [9] upon UE has radio link failure or handover failure and successfully re-connected to an E-UTRA cell.

RACH Partitioning Information It is optional for UE to support the delivery of RACH partitioning related information via RACH report procedure, upon request from the network.

RLF report after successful fast MCG recovery It is optional for UE to support logging previousPCellId, lastHO-Type, and timeConnFailure when T316 was not running before entering the PCell in which the radio link failure was detected.

RLF Report for Fast MCG Recovery It is optional for UE to support the delivery of the Fast MCG recovery related information in the RLF-Report.

RLF Report for Inter-system HO for Voice Fallback It is optional for UE to support the delivery of an explicit indication in the RLF-report when mobility from NR due to voice fallback fails.

SCG Failure Report for CPAC It is optional for UE to support the delivery of the CPAC related parameters for MRO in SCGFailureInformation message to the network.

SCG Failure Report for MRO It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network.

SON enhancements for NR-U It is optional for UE to support the delivery of NR-U related information (FR1 only) in RA-report/SHR/RLF/SPR/SCGFailureInformation report, upon request from the network.

SON Report in SNPN It is optional for UE to support collection and delivery of SON reports in SNPN. UE is not required to support all SON reports if it supports collection and delivery of the SON reports in SNPN, it may support one or more SON report for SNPN.

SpCell ID indication It is optional for UE to support the delivery of the spCellID-r17 in the RA-Report, if the RA procedure is performed in a SCell of the MCG/SCG.

Uplink PDCP delay measurements upon MO update It is optional for UE to support not resetting the UL PDCP Packet Average Delay measurement or UL PDCP excess packet delay measurement when the associated measurement object is modified. A UE supporting this feature shall also indicate the support of at least one of ulPDCP-Delay-r16 and excessPacketDelay-r17.

CHANGE END

Annex: TP for 38.822

8.2.X	NR_ENDC_SON_MDT_Ph4

Table 8.2.X-1: Layer-2 and Layer-3 feature list for NR_ENDC_SON_MDT_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y. NR_ENDC_SON_MDT_Ph4-Core | Y-1 | RLF Report for MCG LTM | Indicates whether the UE supports RLF-Report for MCG LTM. |  | rlfReport-LTM-r19 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | Y-2 | RLF Report for CHO with candidate SCG | Indicates whether the UE supports RLF-Report for conditional handover with candidate SCG. | 37-1 | rlfReportCondHandoverWithCandSCG-r19 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | Y-3 | Geographic area scope checking for logged MDT | Indicates whether the UE supports geographic area scope checking for logged MDT. | 20-6 and 34-1 | geoAreaScopeChecking-r19 | UE-BasedPerfMeas-Parameters -r16 | No | No |  | Optional with capability signalling |
|  | Y-4 | Mobility history information storage for SCG deactivation | It is optional for UE to support the storage of the time the UE has spent in PSCell with SCG activated and the reporting in UEInformationResponse message as specified in TS 38.331 [2] |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-5 | SON enhancements for MCG LTM | It is optional for UE to support the delivery of MCG LTM related information in SHR/RACH report, upon request from the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-6 | RA Report for SDT | It is optional for UE to support the delivery of SDT related information via RACH report procedure, upon request from the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-7 | RLF Report for time/location based CHO | It is optional for UE to support the delivery of the time-/location-ased CHO related information in the RLF-Report. | 37-1 | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-8 | SON enhancements for CHO with candidate SCG | It is optional for UE to support the delivery of CHO with candidate SCG related information in SHR/SPR/SCGFailureInformation report, upon request from the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-9 | SON enhancements for CHO with candidate SCG when CHO only configuration is received | It is optional for UE to support the delivery of CHO with candidate SCG related information when CHO only configuration is received in RLF/SHR report, upon request from the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-10 | SCG Failure Report for Subsequent CPAC | It is optional for UE to support the delivery of the Subsequent CPAC related parameters for MRO in SCGFailureInformation message to the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | Y-11 | Logged MDT for Slicing | It is optional for UE to support Logged MDT for Slicing. |  | N/A | N/A | No | No |  | Optional without capability signalling |