3GPP TSG-RAN WG2 Meeting #131bis	R2-2506797

Prague, Czech Republic, October 13th – 17th, 2025

Source:	vivo

Title:	Considerations on 6GR general aspects

Agenda Item:	10.2

Document for:	Discussion and Decision

- Introduction

In RAN#109, Study on 6G Radio was agreed in [1]. Main RAN2 leading items are listed in below. In this contribution, we will discuss 6G General aspects.

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

- Discussion

As NR technologies matured and numerous networks are deployed all over the world, further improvement of specification does not benefit the eco-system. Hence, a new generation of technologies is envisioned with 6GR providing superior performance and various services. However, while developing 6GR air interface standards, we should avoid making same mistakes made in NR while taking the better ingredients of it as baseline. In this section, we provide high level views in sub-sections below covering several aspects including basic design principles and guidelines, deployment scenarios and architectures and UE capability.

We also provide our details views on those in respective contributions for “Considerations on 6GR user plane” [2], “Considerations on 6GR control plane” [3], “Considerations on 6GR AI framework” [4], “Considerations on 6G data collection and data transfer” [5], and “Considerations on 6GR mobility” [6].

  - Principles and guidelines in 6G

6GR air interface shall significantly provide better performance than NR in terms of spectral efficiency, peak data rate, overall improved user experience etc. Besides meeting ITU requirements for IMT-2030, fulfilling the KPIs, 6GR system shall address the real deployment issues both from network side as well as from device side. It shall support enhanced mobile broadband services as high priority while also meeting the vertical needs. It shall support very high speed (e.g., high-speed train, high-speed motorways) scenario while better user experience in urban mobility, seamless mobility in mid to high-speed scenario is also important. It shall also support new services such as sensing, AI and sensing etc. 6GR air interface shall efficiently support various device types with different form factors, e.g., smartphones, wearables (smart watch, XR), FWA etc. Energy efficient system design is yet another important aspect to consider in development of 6GR air interface.

On the other hand, lessons should be learnt from enormous NR specifications and mistakes should be avoided! NR system was designed with unnecessary flexibilities, e.g., with too many options from the very first release, which in other words is “too complex”. And, there are too many optional features together with multiple options, i.e., too many optional features give huge UE capability signaling. Furthermore, too many sub-features with unnecessary decomposition of granularity create very difficult choices for development/planning of devices and network deployment, and leading to impediment in quick deployment as well as market fragmentation.

Besides, lessons from 5G show that multiple features were introduced for the same target, e.g., UE power saving, each targeting different scenarios. However, in practice, operators faced significant confusion on: which features should be deployed, and which combinations of features could bring actual gains. This complexity limited the commercial deployment of several standardized features. For 6G Day-1 system design, it would be beneficial to consolidate features serving the same purpose into an integrated framework, and evaluate their combined gains. This approach will provide clearer deployment guidance to operators and increase the likelihood of commercial adoption.

With above in mind, our views on design of 6GR air interface are that it shall be sufficiently better than NR and simple while supporting verities of device types and energy efficient. Hence, we have following proposal.

- Proposal 1: Consider following aspects in study of 6GR air interface design:

- Not to support features which are not commercialized in NR which is introduced from early NR stage or make the non-commercialized R15 mandatory features as optional features (e.g., RRC inactive state).

- Avoid multiple options for the same purpose/functionality, and make simplification of specifications.

- Minimize unnecessary decomposition of features granularity, define a set of mandatory features for a certain device type or feature profiles.

- Consolidate the features with the same purpose into a unified framework, and jointly evaluate their deployment gains, e.g., for power saving features.

  - deployment scenarios and architectures

    - RAN protocol architecture

In the NR protocol stack, we largely retained LTE's protocol stack architecture while implementing evolutionary enhancements to accommodate emerging service requirements, e.g., introduces a new L2 sublayer SDAP to carry out new QoS model. For 6GR protocol stack design, we propose adopting a similar approach of architectural inheritance coupled with demand-driven enhancements.

- Inheritance

The NR protocol stack consists of control plane protocol stack and user plane protocol stack, as illustrated below:

Figure 1：User Plane and Control Plane Protocol Stack in NR

The NR protocol stack architecture was designed to meet the diverse data transmission and management requirements of communication services, demonstrating robust support for high-performance connectivity while ensuring extensibility. Our analysis reveals no motivation for removing or relocating any protocol layer, though we propose functional enhancements for the Control Plane and User Plane for 6G[2][3]. Consequently, we think that 6GR should reuse the NR protocol architecture to perform user data delivery and signaling transfer for communication-centric services.

- Potential enhancement

In addition to conventional communication-centric data, non-traditional data types, which may lead to new requirement to protocol architecture enhancement, is expected to be transferred in 6G. The data types includes the following：

- 6G operational data:  it is the data that is controlled by the 6G system and can be generated or collected by the 6G system. To better serve the users and manage the network, the 3GPP system need to collect 6G system data and transfer the data between different entities (e.g., UE, RAN node, CN nodes).

- AI related data, including AI model, collected datasets for AI training.

- Sensing-oriented data

The processing and transfer requirements of the new emerging data fundamentally differ from traditional communication data in several aspects: Destination diversity (targeting to different network nodes), differentiated QoS requirements, varied network visibility needs and user privacy considerations. These requirements exceed the current capabilities of NR's Control Plane and User Plane. With the above observation, we propose investigating a unified data collection and transfer framework to handle non-communication-centric data, this framework may be characterized by new radio bearer type in air interface, aiming to address the limitations of existing SRBs and DRBs. And the framework is expected to be flexible and extensible to handle new data types to emerge in the 6G era. More details can be found in our company paper [5].

Proposal 2: The 6GR protocol stack shall builds upon NR's foundational architecture while introducing targeted enhancements to support non-communication-centric data processing and transfer, including 6G operational data, AI/ML related traffic, Sensing-oriented data and new data types to emerge in the 6G era.

    - Single cell with multi-carriers (SCMC)

Since late releases of NR, network energy savings (NES) have been intensely studied, including time/frequency/power/spatial-domain techniques. One focus among the techniques is to reduce signalling transmission from time-domain’s perspective, such as the NES cell transmitting SIB1 in an on-demand manner. In 6G, one possible evolvement for cell modelling is that one single cell can aggregate multiple carriers (Fig. 2), so that NES gain can be achieved by transmitting SIB only on one carrier (e.g. anchor carrier) to reduce independent SIB transmission on multiple carriers. For discussion reference, we can name such a cell model as single cell with multi-carriers (SCMC).

Figure 2: Example of SCMC

SCMC is not only beneficial for NES, but also helps the operators to utilize some fragmented spectrums which are re-farmed from 2G/3G with small bandwidth in sub-1GHz low bands. Considering the bandwidth of the fragmented spectrum is limited, the carrier in the fragmented spectrum can rely on other carriers in the SCMC for SSB transmission.

Figure 3: Different bands/band groups in SCMC

Figure. 3 illustrates some examples of possible bands or band groups deployed in SCMC. The frequency location of the bands/band groups may cover from low bands to ultra-high bands. Traditional single cell with single carrier deployment in LTE/NR mandates the cells deployed with high bands or ultra high bands to transmit SSB/SIB of their own for UEs camping in IDLE/INACTIVE mode. In comparison, an anchor carrier in low bands can be set in SCMC to provide necessary SSB/SIB. This is beneficial for UE’s power saving as the UE may monitor SSB/SIB only on anchor carrier for cell camping in a wide area within the cell coverage.

Observation 1: The cell model of single cell with multi-carriers (SCMC) is not only enabling the operators to utilize fragmented spectrums with potential NES gain (e.g., SIB is transmitted only on anchor carrier), but also beneficial for the UE’s power saving to camp on SCMC when the anchor carrier is deployed in low bands.

Figure 4: carriers with similar or diverse coverage due to the band location and signal power control

For further study of SCMC, we think the deployment scenario needs to be discussed first. For example, the carriers within 6G SCMC may be deployed within the same band, or in different bands from low-band (<1GHz) to mid/high-band, and thus the coverage of the carriers may be similar as Fig. 4-a (e.g. all DL CCs are within the same band or in low-bands) or diverse as Fig. 4-b (e.g. one DL CC on low-band and another CC on mid/high-band). However, field deployment of SCMC may need to consider inter-site interference control. With UL/DL signal power control for different bands, even the coverage of the low band may also ‘shrink’ to be similar with the mid-band, as Fig. 4-c shows.

Figure 5: SCMC with co-located or non-co-located deployment

Another deployment assumption is with regard to co-location relationship among the carriers, i.e. all the carriers within SCMC may be co-located (Fig. 5-a) or non-co-located (Fig. 5-b) deployed.

For co-located deployment, all the carriers may share a same SSB for UE’s time/frequency tracking, if the bands of the carriers are within a certain frequency range or in a band group, e.g., the 3 DL carriers belongs to the bands of 700/800/900MHz separately (Fig. 4-a). Note that if the DL carriers belong to bands with distant band range, e.g. one carrier is < 1GHz while the others are > 1GHz (Fig. 4-b), the UEs served on carriers of the bands > 1GHz may have to rely on the independent SSB of the carriers for time/frequency tracking.

For non-co-located deployment, the carriers on different sites also need independent SSBs for synchronization, i.e. they belong to different TAG. If the network supports ideal backhaul, cross-carrier scheduling among carriers of different sites can be supported for CONNECTED mode operation. Otherwise, scheduling may be independent in different sites.

In sum, we propose RAN2 to initiate the discussion on how to characterize the deployment scenarios of 6G SCMC. From our point of view, the DL/UL carriers of SCMC may be deployed within the same band or in different bands with potential power control strategy, resulting in similar or diverse coverage among the DL/UL carriers. This may affect how we further study the functional design for 6G SCMC. Besides, the carriers of SCMC may be co-located or non-co-located deployed, which may have impact on the scheduling mechanism.

Proposal 3: 6GR shall study single cell with multi-carriers (SCMC) to aggregate multiple carriers in the same or different bands as a single cell, with the assumption of same/diverse coverage and co-located/non-co-located deployment among the carriers.

    - eMBB and 6G IoT

In the era of 6G, the mobile system will provide its services to a myriad of devices, from low-end IoT devices to high-end advanced eMBB devices. The design of 6GR should learn from the lessons of LTE & NR systems, wherein several IoT device types (e.g., e.g., NB-IoT, (e)MTC, (e)Redcap) were introduced in later releases, causing too many different technical paths serving similar service and thus resulting in serious dilemma in deployment. Therefore, clear blueprint for 6G IoT should be laid out in the first release to avoid fragmented markets as well as compatibility issues in later releases.

Observation 2: Developing single 6G RAT to serve diverse device types including eMBB and 6G IoT devices is considered to be necessary for future IoT markets.

Moreover, according to RAN1#122 meeting, the following agreement has been achieved.

RAN1#122 Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any

Regarding the highlighted yellow text as above, we think that common features that are required for both eMBB and 6G IoT devices should be part of the key aspects. For example, IoT technologies for reduced power consumption and enhanced coverage are also critical for eMBB devices to ensure optimal user experience in specific scenarios. In detail, to support eMBB and 6G IoT harmonization in single 6G RAT, at least the following aspects needs to be considered:

- Power saving features: the 6G radio protocol design shall support technologies to improve energy efficiency that are mutually required by eMBB and 6G IoT devices. Power saving features e.g., eDRX with very long cycle, LP-WUS, reduced UE capability update (e.g., to single Rx, reduced MIMO layers) that are introduced in 4G/5G may still be leveraged in the 6G network. Moreover, it is expected to introduce common 6GR design for power saving features that are commonly applicable to eMBB and 6G IoT devices with minimal differences.

- Coverage enhancement features: the 6G radio protocol design shall support coverage native design for ensuring seamless service continuity across diverse use cases/scenarios. Extended coverage is yet another common 6G Radio design aspect applicable for both consumer eMBB and IoT applications. Moreover, RAN2 study shall also focus on developing efficient mechanisms to enable resource sharing for common control channels (e.g., for carrying system information) and supporting extended coverage for both initial access and RRC connected mode.

Assume that single 6G RAT is deployed to serve both eMBB and 6G IoT, network adaptation can be used to achieve energy saving, e.g.  during night time when eMBB data volume is low, operator can turn off the capacity layer BS and keep the coverage layer BS active to provide basic services for both 6G IoT and eMBB devices, as shown in right half of Figure 6.

Figure 6:  Network energy saving if coverage enhancement features supported for all devices

Proposal 4: 6GR shall study a scalable design that is able to efficiently support the different features e.g., power saving, coverage enhancement that are commonly applicable to eMBB and 6G IoT devices.

    - 6G NTN

NTNs are expected to be an integral part of the system design for 6G—a requirement explicitly mandated by the objective of the 6G SID, as they play a pivotal role in delivering 6G’s core goal of ubiquitous connectivity.

Notably, the initial design of NR was centered entirely on Terrestrial Networks (TNs), with no consideration for the unique characteristics of NTNs. The NR NTNs were introduced as an important deployment scenario in later releases of NR (i.e., since NR Rel-17), and the NR architecture inherently lacked native support for NR NTNs. At the time, to avoid disrupting the existing TN specification and implementation, the design of NR NTNs was ultimately decoupled from the "mainstream" NR framework and developed as a separate branch. This fragmented approach created significant barriers to the practical commercial deployment of NR NTNs. Specifically, since NR chips were not designed with NTN capabilities at the very beginning (i.e., NR day 1), chip manufacturers have to develop specialized NTN-enabled chips, while device manufacturers were forced to integrate these external "add-on chips" into terminals to enable NTN functionality. This extra hardware requirement directly drove up the production cost of end-user devices. Meanwhile, network operators faced additional burdens: existing network equipment (such as base stations and core network nodes) required hardware upgrades and software reconfiguration to adapt to NR NTN feature. These cumulative cost pressures significantly hindered the commercialization of NR NTNs. Beyond the cost issues, the “add-on” nature of NTN feature in NR system also led to the fragmented protocol logic—a problem that compounded technical complexity. This manifests in protocols patching (e.g., introducing NTN-specific signaling to extend parameter ranges), duplicate implementations (e.g., coexisting “TN" vs. "NTN" versions of the same function), and distinguished function implementations (e.g., specified staring point of L2 timer for TN vs. UE adjusted starting point of L2 timer for NTN).

Building on the critical lessons from NR design, the design of 6G Radio Access (6GR) must prioritize a harmonized TN-NTN architecture. By unifying the design of terrestrial and non-terrestrial networks from the outset, 6G can eliminate the fragmentation that plagued NR NTNs. A review of NR NTN design reveals that the key point of NR’s TN-NTN divergence is the introduction of Ephemeris (working together with the GNSS) for NTN. Due to the large propagation delay and movement of the satellite, the time synchronization, scheduling, and mobility management become crucial issues for NR NTN. To address these issues, Ephemeris was adopted: For time synchronization, the Ephemeris was introduced to support UE pre-compensation. For scheduling issue, UE adjustment-based L2 timer handling based on ephemeris is introduced. For example, UE determines the starting point of L2 timer (e.g., RAR window, HARQ RTT timer) according to the UE-gNB RTT calculated from ephemeris and UE location. For movement management, location-based mechanism is used. Although these approaches are beneficial to minimize the impacts to the NR specification, it leaves a lot of complexities to the implementation, such as a redesign and development of the UE (e.g., L2 handling). Consequently, it not only results in significant design and development efforts, but also requires a lot of testing efforts, not to say the high cost of the product. Therefore, instead of introducing the NTN specific feature such as Ephemeris, a unified radio design for TN and NTN should be adopted for 6GR. For example, instead of an Ephemeris-based solution, a pre-compensation scheme based on network indication, e.g., similar to the existing TA-command based adjustment, should be used for time synchronization. For scheduling issues caused by large propagation delay, instead of a UE-adjusted starting point of L2 timer (e.g., RAR window, HARQ RTT timer), an offset-based scheme guided by NW indication can be used. Such a unified 6GR design is beneficial for easier implementation and deployment of 6G NTN since NTN-specific hardware, functionality, requirement or testing would not be required.

Observation 3: Due to the lacked native support NTN in NR system, the design of NR NTN is forked out as a separate branch away from the “main stream” of NR, which caused fragmented protocol logic and difficulties for the deployment of NR NTN.

Observation 4: The key point of NR’s TN-NTN divergence is the Ephemeris based framework for NTN, which results in extensive NTN specific procedures.

Proposal 5: 6GR shall strive for as much commonality for both NTN and TN operations, at least, ephemeris-free based framework.

  - UE capability

In LTE, UE capability information can be indicated via UE category, which defines a combined uplink and/or downlink capability, e.g., the maximum number of transport block bits within a TTI, the maximum number of supported layers for spatial multiplexing. To enhance signaling flexibility and forward compatibility, the UE capabilities in NR do not rely on UE categories, i.e., UE categories associated with fixed peak data rates are only defined for marketing purposes and not signalled to the network.

However, the excessive flexibility in NR results in significant signaling overhead. As a consequence, the UECapabilityInformation message may exceed the maximum PDCP SDU size and segmentation will be performed. This may introduce additional latency during the initial connection establishment procedure. Meanwhile, the unnecessary decomposition of features into overly granular sub-features complicates device development, network planning, and deployment, and leads to difficulties in quick deployment as well as market fragmentation.

Furthermore, due to excessive capability flexibility, some capability-related mechanisms in NR are impacted:

- Feature Set combination: A UE may indicate multiple Feature Sets per band (also known as feature sets per band) to advertise different alternative features. Since feature sets are defined per band and each Feature Set is associated with both uplink and downlink capabilities, excessive capability flexibility makes it difficult for Feature Set combinations to be reused across multiple band combinations.

- UE radio capability signalling optimization (RACS): RACS operates by assigning a UE Radio Capability ID to represent a set of UE radio capabilities. As the radio capability ID is per UE, excessive capability flexibility makes it nearly impossible for the same UE Radio Capability ID to be reused across different UE types. This results in increased storage overhead for the UCMF (UE Radio Capability Management Function) to store the mappings between UE Radio Capability IDs and corresponding UE radio capability information.

Observation 5: The following issues are observed related to UE capability in NR:

- Excessive UE capability flexibility results in significant signaling overhead of UE capability information report.

- Feature sets per band are associated with both uplink and downlink capabilities, makes it difficult for Feature Set combinations to be reused across multiple band combinations.

- UE Radio Capability ID in RACS is a per UE identifier, makes it nearly impossible for the same UE Radio Capability ID to be reused across different UE types.

Figure 7: Feature set combinations with decoupled UL/DL feature sets per band group

6G should learn from the lessons of LTE and NR by reducing the signaling overhead of UE capability with reasonable UE flexibility. The following approaches could be considered:

- Reuse filtering and fallback mechanisms in NR: The gNB can request the UE to provide capabilities for a restricted set of bands. In response, the UE may omit a subset of the requested band combinations in accordance with UE capability fallback behavior.

- Reduce the number of options for features and parameters, e.g., different device types may associate with different mandatory features and minimum capabilities.

- Adjust the mapping of Feature Set combination, including the introduction of Feature Sets per band group or even feature set across all the bands, and decoupling of uplink and downlink Feature Sets, as illustrated in Figure 7.

- Group sets of features together to minimize signaling overhead. Unlike the RACS in NR, each group can be associated with a separate ID. If multiple UEs share certain capability groups, the same ID can be reused. Additionally, if a UE's capabilities change (e.g., due to RF capability modifications), only the IDs corresponding to the changed capabilities need to be updated.

Proposal 6: The UE capability information in 6G should consider reducing signaling overhead with reasonable flexibility. The following approaches could be considered:

- Reuse filtering and fallback mechanisms in NR.

- Reduce the number of options for features and parameters.

- Adjust the mapping of Feature Set combination, e.g., introduce Feature Sets per band group or across all the bands, and decouple uplink and downlink Feature Sets.

- Group sets of features together to minimize signaling overhead, with each group associated with a separate ID.

On the other hand, NR supports the reporting of temporary capability restrictions, e.g., due to hardware sharing, IDC problem, or overheating. These temporary restrictions are indicated to the gNB via the UEAssistanceInformation message rather than through UE capability update. However, since temporary capability restrictions are reported as UE preferences, how the network treats them for further configuration is up to network implementation.

Furthermore, with the ongoing evolution of UE types and form factors, such as the growing popularity of foldable phones, the RF capabilities (e.g., MIMO layers) may change depending on the device’s current state. In such cases, changes in UE capability should not be reported merely as preferences or assistance information, but rather as actual capability updates. The network should respect the updated capability parameters when configuring and scheduling the UE.

Therefore, we propose a unified UE capability update framework for the above various dynamic UE capability update cases.

Proposal 7: The UE capability framework in 6G should support dynamic and efficient UE capability update, including the following scenarios, e.g., factor switching (foldable phone and MUSIM) and exceptional UE issues (IDC problem, overheating, low power).

- Conclusion

In this paper, we discuss the general aspects on 6G radio. we have the following observations and proposals:

- Principles and guidelines

- Proposal 1: Consider following aspects in study of 6GR air interface design:

- Not to support features which are not commercialized in NR which is introduced from early NR stage or make the non-commercialized R15 mandatory features as optional features (e.g., RRC inactive state).

- Avoid multiple options for the same purpose/functionality, and make simplification of specifications.

- Minimize unnecessary decomposition of features granularity, define a set of mandatory features for a certain device type or feature profiles.

- Consolidate the features with the same purpose into a unified framework, and jointly evaluate their deployment gains, e.g., for power saving features.

- RAN protocol architecture

Proposal 2: The 6GR protocol stack shall builds upon NR's foundational architecture while introducing targeted enhancements to support non-communication-centric data processing and transfer, including 6G operational data, AI/ML related traffic, Sensing-oriented data and new data types to emerge in the 6G era.

- deployment scenarios

Observation 1: The cell model of single cell with multi-carriers (SCMC) is not only enabling the operators to utilize fragmented spectrums with potential NES gain (e.g., SIB is transmitted only on anchor carrier), but also beneficial for the UE’s power saving to camp on SCMC when the anchor carrier is deployed in low bands.

Observation 2: Developing single 6G RAT to serve diverse device types including eMBB and 6G IoT devices is considered to be necessary for future IoT markets.

Observation 3: Due to the lacked native support NTN in NR system, the design of NR NTN is forked out as a separate branch away from the “main stream” of NR, which caused fragmented protocol logic and difficulties for the deployment of NR NTN.

Observation 4: The key point of NR’s TN-NTN divergence is the Ephemeris based framework for NTN, which results in extensive NTN specific procedures.

Proposal 3: 6GR shall study single cell with multi-carriers (SCMC) to aggregate multiple carriers in the same or different bands as a single cell, with the assumption of same/diverse coverage and co-located/non-co-located deployment among the carriers.

Proposal 4: 6GR shall study a scalable design that is able to efficiently support the different features e.g., power saving, coverage enhancement that are commonly applicable to eMBB and 6G IoT devices.

Proposal 5: 6GR shall strive for as much commonality for both NTN and TN operations, at least, ephemeris-free based framework.

- UE capability

Observation 5: The following issues are observed related to UE capability in NR:

- Excessive UE capability flexibility results in significant signaling overhead of UE capability information report.

- Feature sets per band are associated with both uplink and downlink capabilities, makes it difficult for Feature Set combinations to be reused across multiple band combinations.

- UE Radio Capability ID in RACS is a per UE identifier, makes it nearly impossible for the same UE Radio Capability ID to be reused across different UE types.

Proposal 6: The UE capability information in 6G should consider reducing signaling overhead with reasonable flexibility. The following approaches could be considered:

- Reuse filtering and fallback mechanisms in NR.

- Reduce the number of options for features and parameters.

- Adjust the mapping of Feature Set combination, e.g., introduce Feature Sets per band group or across all the bands, and decouple uplink and downlink Feature Sets.

- Group sets of features together to minimize signaling overhead, with each group associated with a separate ID.

Proposal 7: The UE capability framework in 6G should support dynamic and efficient UE capability update, including the following scenarios, e.g., factor switching (foldable phone and MUSIM) and exceptional UE issues (IDC problem, overheating, low power).

- Reference

- RP-252912	New SID: Study on 6G Radio.

- R2-2506798 Considerations on 6GR user plane.

- R2-2506799 Considerations on 6GR control plane.

- R2-2506800 Considerations on 6GR AI framework.

- R2-2506801 Considerations on 6G data collection and data transfer.

- R2-2506802 Considerations on 6GR mobility.