3GPP TSG-RAN WG2 Meeting #131bis	R2-2506803

Prague, Czechia, 13th – 17th October, 2025

Agenda Item:	8.13.3

Source:	OPPO

Title:	User Plane correction for R19 Multi-hop U2N Relay

Document for:	Discussion, Decision

Introduction

This paper is to discuss the SRAP open issues of Multi-hop U2N Relay.

Discussion

SRAP-6 (OPPO) indirectly connected remote UE’s SRB1 configuration

| SRAP-6 (OPPO) | In clause 5.2.2.2, based on current procedure, for SRB1 of directly and indirectly Remote UE, it assumed default configuration (SL-RLC1) can be used. But there is no agreement on that. | Considering RAN2 agrees the configuration of indirectly connected remote UE’s SRB0 is dedicated configuration, seems it is reasonable to use dedicated configuration for SRB1 as well instead of use default SL-RLC1 to differentiate the SRB1 from Child UE and other remote UEs. - As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay: - At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0); - At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message. RAN2 to discuss using dedicated RLC channel configuration for SRB1 of indirectly connected remote UE. If it can be agreed, change as follows: | Considering this issue is not discussed in RAN2, contribution input on the open issue is recommended. |
| --- | --- | --- | --- |

For SRB1 in single-hop U2N Relay, the RLC channel configuration may be omitted, as the default SL-RLC1 can be utilized. However, for multi-hop U2N Relay, there has been no discussion on whether the default RLC channel configuration for SRB1 can similarly be reused.

There is currently no discussion on whether or how the default SL-RLC1 is applied in multi-hop U2N Relay scenarios.

RAN2 has discussed the usage of SL-RLC0, and the following agreement was made:

- As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay: - At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0); - At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

For SRB0, SL-RLC0 is only used on the link between the remote UE and the first relay UE for the Child UE, and dedicated configured RLC channel will be used for the indirectly connected remote UEs.

SL-SRB0 is used only for directly connected remote UE. Dedicated configuration is used for the SRB0 of indirectly connected remote UE(s).

Similar to SRB0, SL-RLC1 can be used for the directly connected remote UE as in single hop case, i.e., on the PC5 link between the remote UE and the first relay UE.

For the indirectly connected remote UE, i.e., on the PC5 link between the intermediate relay UEs and Uu link, if default PC5 RLC channel is used, there is additional impact at each relay UE to check whether the bearer is SRB1 and the egress RLC channel is not configured. The specification impact is as follows:

#### 5.3.3.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList matches the UE ID field in SRAP Data PDU: - If the SRAP Data PDU is for SRB0: - Determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3]. - Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]: - If the SRB1 is not from the Child UE and sl-EgressRLC-Channel-DL is absent in sl-SRAP-ConfigRelay-ToAddModList, determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3] - Else if determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3]. - Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1): - Determine the egress Uu/PC5 Relay RLC channel corresponding to sl-EgressRLC-ChannelUu or sl-EgressRLC-Channel-UL configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

Considering from the Relay UE point of view, there is no need for the special handling of an indirectly connected SRB1, i.e., all the RBs are same for the relay UE. It is preferred to align the design with SRB0, and use dedicated RLC channel configuration for the indirectly connected remote UE. The following TP1 to remove the “or sl-EgressRLC-Channel-DL in sl-SRAP-ConfigRelay-ToAddModList is absent” in clause 5.2.2.2 is needed if dedicated configuration is agreed:

#### 5.2.2.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - If the SRAP Data PDU is for SRB0 of the Child UE (for single-hop U2N Relay case, the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received; or for multi-hop U2N Relay case, the UE ID field matches the sl-LocalIdentity configured in sl-SRAP-ConfigRelay, BEARER ID field is 0, and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry either containing the sl-EgressRLC-Channel-UL which matches the LCID of the PC5 Relay RLC Channel or the sl-EgressRLC-Channel-Uu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received): - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3]. - Else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel or sl-EgressRLC-Channel-UL which matches the LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1): - If the SRAP Data PDU is for SRB1 but the corresponding sl-EgressRLC-ChannelPC5 is absent in sl-SRAP-ConfigRelay: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3]. - Else: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 or sl-EgressRLC-Channel-DL configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

(SRAP-6) For the DL and UL SRB1 of remote UE in multi-hop U2N Relay:

      - At the link between remote UE and the first relay UE, the default PC5 RLC channel (i.e., SL-RLC1) can be used as in R17 single hop U2N Relay;

      - At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

Conclusions

We have the following observations:

Observation 1	There is currently no discussion on whether or how the default SL-RLC1 is applied in multi-hop U2N Relay scenarios.

Observation 2	SL-SRB0 is used only for directly connected remote UE. Dedicated configuration is used for the SRB0 of indirectly connected remote UE(s).

We have the following proposals:

Proposal 1	(SRAP-6) For the DL and UL SRB1 of remote UE in multi-hop U2N Relay:

-	At the link between remote UE and the first relay UE, the default PC5 RLC channel (i.e., SL-RLC1) can be used as in R17 single hop U2N Relay;

-	At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message.

References

[1] R2-2506566 Introduction of NR sidelink multi-hop relay in TS 38.351