3GPP TSG-RAN WG2 Meeting #131bis                                                  R2-2506822

Prague, Czech Republic, 13th – 17th Oct, 2025

Source:              CATT

Title:	             Remaining Issues on Random Access

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

According to the email discussion [1], there are two remaining issues needs for further discussion with tdoc contributions, listed below:

- MAC-2: How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured." - MAC-3: Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

In this contribution, the above two open issues will be further discussed.

Discussion

[MAC-2] Clarification on preambleReceivedTargetPower

Issue description

In RAN1#120bis meeting, the following agreement was reached:

Agreement For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols.

Based on the above agreement, for RACH configuration option 2, if Msg3 PUSCH is transmitted in SBFD symbols, preambleReceivedTargetPower configured for additional RO is used; otherwise, preambleReceivedTargetPower configured for legacy RO is used.

But in the MAC running CR [2], the following is captured:

### 5.1.4 Random Access Response reception …… 5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]: 6> if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else: 7> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]): 6> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

For the last green part, it is unclear the preambleReceivedTargetPower refers to the preambleReceivedTargetPower included in sbfd-RACH-DualConfig or not. In fact, according to RAN1 agreement, the preambleReceivedTargetPower here does not refer to the preambleReceivedTargetPower included in sbfd-RACH-DualConfig. This needs to be clarified.

Proposed solution

During the email discussion [1], there are two options for this clarification:

- Option 1: Negative statement (e.g., clarify it refers to preambleReceivedTargetPower not included in sbfd-RACH-DualConfig)

- Option 2: Positive statement (e.g., clarify it refers to preambleReceivedTargetPower included in xxx)

The intention of Option 2 is to simplify the UE implementation and UE can directly determine which preambleReceivedTargetPower should be used. For Option 2, the main issue is how to specify the xxx. As commented in the email discussion [1], it is uneasy to specify xxx:

- If the UE selects the set of Random Access resources which is not associated with any feature, the UE should use preambleReceivedTargetPower in rach-ConfigCommon (without suffix) which is directly included in BWP-UplinkCommon IE.

- If the UE selects the set of Random Access resources associated with feature(s), the UE may need to use preambleReceivedTargetPower in rach-ConfigCommon-17 for the selected feature combination, which is included in additionalRACH-Config-r17 IE.

But in our understanding, from UE perspective, once UE select the feature/featurecombination, it is clear which xxx is used. Hence, there is no big problem to use rach-ConfigCommon as xxx. From this perspective, Option 2 is slightly preferred.

[MAC-2] Proposal 1: Positive statement is slightly preferred, that is to clarify that for RACH configuration option 2, if Msg3 PUSCH transmission is performed in non-SBFD symbols, preambleReceivedTargetPower included in rach-ConfigCommon can be used.

[MAC-3] UE transmit power continuity during RO type switching

Issue description

During RAN2#131, for RO type switching, power ramping step offset compensation between different RO types was agreed. But in the email discussion [1], one company further raised that preambleTargetReceivedPower also needs to be compensated. The key reason for this compensation is that UE should not change its transmit power by a value higher than 6dB between msg1 transmissions because the maximum powerRampingStep is 6dB.

Proposed solution

But in our understanding, the power ramping step offset compensation between different RO types is inherited from 2-step RA/4-step RA fallback.  For the 2-step RA and 4-step RA, the preambleTargetReceivedPower were also configured separately, but no compensation was introduced to compensate the difference between the preambleTargetReceivedPower of the two RA types. Hence, the same principle should be used for RO type switch, and no spec impact is needed.

[MAC-3] Proposal 2: No need to compensate the preambleTargetReceivedPower difference in case of RO type switch.

Conclusion

According to the analysis in section 2, it is proposed:

[MAC-2] Proposal 1: Positive statement is slightly preferred, that is to clarify that for RACH configuration option 2, if Msg3 PUSCH transmission is performed in non-SBFD symbols, preambleReceivedTargetPower included in rach-ConfigCommon can be used.

[MAC-3] Proposal 2: No need to compensate the preambleTargetReceivedPower difference in case of RO type switch.

Reference

- R2-25xxxx List of Rel-19 SBFD MAC open issues for maintenance Samsung

- R2-2506606 Introduction of Rel-19 Evolution of NR duplex operation(SBFD) for MAC spec Samsung

- R2-2506605 Corrections for WI Evolution of NR duplex operation (SBFD)  Huawei, HiSilicon