3GPP TSG-RAN WG2 Meeting # 131	R2-2506826

# Bangalore, India, 25th – 29th August 2025

Agenda item:	8.19.2

Source:	Google

Title:	Support for MINT in EPS (MINT_Ph2)

Document for:	Discussion & Decision

- Introduction

CT1 sent a LS in R2-2506707/C1-25567 to RAN2 as below. This paper discusses how to support MINT in EPS.

CT1 has started the normative work for the Rel-19 Work Item on MINT_Ph2(Minimization of Service Interruption Phase 2). This feature is based on the conclusions of the FS_MINT_Ph2 (Study on MINT support in EPS for 5G-only national roaming UE) as described in TR 24.812. CT1 concluded that the broadcast information, which was specified for NG-RAN as part of Rel-17 MINT also needs to be broadcasted via E-UTRAN connected to EPC as below: a) disaster related indication; or b) "list of one or more PLMN(s) with disaster condition for which disaster roaming is offered by the available PLMN" where each PLMN with disaster condition is identified by its PLMN ID. The list will need to be able to hold at least the same amount of PLMN IDs as number of PLMNs which can share an E-UTRAN cell. (a) or (b) is used depending on the decision of the available PLMN. In addition, CT1 concluded that E-UTRAN cell broadcasts access-barring parameters for access barring for disaster roaming services. CT1 also agreed that UE NAS is able to indicate to UE AS that establishment of the NAS signalling connection is for a UE attaching or attached for disaster roaming services, as described in C1-255284. CT1 would like to request RAN2 to work on system information broadcasted by E-UTRAN to support MINT_Ph2 feature.

- Discussion

## 2.1 	Configurations of disaster roaming information for E-UTRA/EPC

SIB30 is defined to include configurations of disaster roaming information for E-UTRA/5GC as shown below.

#### – SystemInformationBlockType30 The IE SystemInformationBlockType30 contains configurations of disaster roaming information. SystemInformationBlockType30 information element -- ASN1START SystemInformationBlockType30-r17 ::= SEQUENCE { commonPLMNsWithDisasterCondition-r17 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Identity OPTIONAL, -- Need OR applicableDisasterInfoList-r17 SEQUENCE (SIZE (1..maxPLMN-r11)) OF ApplicableDisasterInfo-r17 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ... } ApplicableDisasterInfo-r17 ::= CHOICE { noDisasterRoaming-r17 NULL, disasterRelatedIndication-r17 NULL, commonPLMNs-r17 NULL, dedicatedPLMNs-r17 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Identity } -- ASN1STOP

For signaling, the disaster roaming information configurations used for E-UTRA/5GC can be adapted for E-UTRA/EPC. The most direct approach would be to reuse SIB30 and its existing configurations to support MINT in E-UTRA/EPC.

However, a challenge arises when an eNB connects to both a 5GC and an EPC. The network might require distinct configurations for E-UTRA/EPC and E-UTRA/5GC disaster roaming. This would necessitate two separate sets of configurations.

### Options for Supporting MINT in E-UTRA/EPC

- Option 1: Reuse SIB30 with existing configurations, i.e., the existing configurations are shared between E-UTRA/EPC and E-UTRA/5GC

  - This is the simplest option. It reuses the current SIB30 and its existing configurations for MINT support in E-UTRA/EPC.

  - Pro: High simplicity and minimal implementation effort.

  - Con: It doesn't allow for different configurations for E-UTRA/EPC and E-UTRA/5GC.

- Option 2: Reuse SIB30 with R19 configurations

  - This option also reuses SIB30 but introduces new Release 19 configurations specifically for E-UTRA/EPC disaster roaming. This would allow the network to support different configurations for the two core networks.

  - Pro: Provides flexibility by allowing separate configurations while leveraging an existing SIB.

  - Con: Requires modifications to SIB30.

- Option 3: Define a New SIB

  - This is the most complex option. A new SIB would be defined exclusively for MINT support in E-UTRA/EPC, containing the new R19 configurations.

  - Pro: Offers a clean separation of configurations.

  - Con: Requires a new SIB. It necessitates changes to SIB1 to schedule the new SIB, making it more involved than Option 2.

Considering simplicity and implementation effort, Option 1 is the most straightforward, though it has functional limitations. Option 3 is the most complex due to the need for changes in SIB1.

Proposal 1: 	Discuss which of these options is the most suitable approach for providing configuration of disaster roaming information.

## 2.2 	Access barring for disaster roaming

In NR and E-UTRA/5GC, all User Equipment (UEs) engaging in disaster roaming utilize Access Identity 3 for cell access, irrespective of the traffic type (signaling or services). This standardized approach leverages the Unified Access Control (UAC) framework to manage access from the disaster roaming UEs.

However, as confirmed by the CT1 agreed CR in C1-255284, this mechanism does not extend to the E-UTRA/EPC domain. The key reason for this divergence is that the E-UTRA/EPC architecture fundamentally does not support UAC. Consequently, the procedure for access barring related to disaster roaming in E-UTRA/EPC is handled distinctly from the unified approach employed in NR and E-UTRA/5GC.

The following is quoted from the CT1 agreed CR.

#### 5.3.1.1 Establishment of the NAS signalling connection When the UE is in EMM-IDLE mode without suspend indication and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection. In this request to the lower layer the NAS shall provide to the lower layer the RRC establishment cause and the call type as specified in annex D of this specification and, for the case specified in clause 5.6.1.2.2, shall also provide the initial NAS message, otherwise NAS may also provide the initial NAS message. <unrelated text omitted> If a UE attaching or attached for disaster roaming services in EPS performs an attach procedure, tracking area updating procedure, or service request procedure, the UE NAS shall indicate to the lower layers that the establishment of the NAS signalling connection is for a UE attaching or attached for disaster roaming services.

Based on the CT1 agreed CR as shown above, when the NAS layer of the disaster roaming UE initates a NAS signalling connection establishment, it indicates the following items to the RRC layer, which should be considered in acces barring check.

- RRC estabislihemt cause

- Call type, and

- Indication of establishment of the NAS signalling connection for a UE attaching or attached for disaster roaming services

In E-UTRA/EPC, different access control mechanisms are specified as below. The non-disaster roaming UE performs access barring check with the RRC establishment cause and call type provided by the NAS layer.

- Access class barring (ACB) for emergency call, mobile originated signalling, and mobile originated data

- Service Specific Access Control (SSAC) for MMTEL voice and video

- Skip barring for MMTEL voice and MMTEL video

- Skip barring for SMS

- Access barring for CSFB

- Extended access barring (EAB)

- Application specific Congestion control for Data Communication (ACDC)

Below, we will discuss:

- Which access barring mechanisms and related parameters should be supported for disaster roaming UEs.

- The access barring parameters should be designed as per PLMN access barring parameters or common access barring parameters for all PLMNs.

- Access barring check for disaster roaming UEs

- Which SIB to include the access barring parameters for disaster roaming UEs

### 2.2.1 	Access barring parameters for disaster roaming

The existing RRC access barring parameters for ACB, SSAC, access barring for CSFB, EAB, and ACDC are listed below.

| Access control mechanism | Barring parameters |
| --- | --- |
| ACB for emergency call, mobile originated signalling, and mobile originated data | ACB parameters in SIB2: |
| Skip barring for MMTEL voice and MMTEL video | MMTEL barring skip parameters in SIB2: |
| Skip barring for SMS | SMS arring skip parameters in SIB2: |
| SSAC for MMTEL voice and video services | SSAC barring parameters in SIB2: |
| ACB for CSFB | ACB parameter for CSFB in SIB2: |
| EAB | EAB parameters (in SIB14): |
| ACDC | ACDC baring parameters in SIB2: |

ACB is the basic barring mechanisms used for access control for emergency call, mobile originated signalling, and mobile originated data. Therefore, the ACB parameters should be supported for access control for disaster roaming UEs.

Proposal 2: 	Support the ACB parameters (i.e., for access control for emergency call, mobile originated signalling, and mobile originated data) for disaster roaming UEs.

In a disaster area, a network providing disaster roaming serivcs may want to have different control access for mobile originated data from MMTEL voice or video calls. For example, in a disaster area, the network may prioritize access for MMTEL voice calls over the moibe originated data calls (i.e., non-MMTEL calls). Thus, disaster roaming users in the area can report their status to their faimilies via MMTEL voice calls. Similarly, in a disaster area, the network may prioritize access for SMS over the moibe originated data calls. Thus, disaster roaming users in the area can report their status to their faimilies via SMS.

Proposal 3: 	Support the SSAC parameters (i.e., for MMTEL voice and MMTEL video) for disaster roaming UEs.

Proposal 4: 	Support barring skip parameters for MMTEL voice, MMTEL video, and SMS for disaster roaming UEs.

In our understanding, the disaster roaming feature does not apply to 3G. From the NAS specification point of view, it is not clear whether a disaster romaing UE can peform a combined attach for LTE and CS. It is also not clear whether the network accepts the combined attach for a disaster roaming UE.

Proposal 5: 	RAN2 should disucss the following options:

Option 1: 		Don’t support the ACB parameter for CSFB for disaster roaming UEs.

Ootion 2: 		Support the ACB parameter for CSFB for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether CSFB is applicable to a disaster roaming UE. RAN2 decides whether to support the ACB parameter for CSFB for disaster roaming UEs based on CT1/SA1 reply.

EAB is a mechanism for the operator(s) to control Mobile Originating access attempts from MTC UEs that are configured for EAB in order to prevent overload of the access network and/or the core network. In our view, it is unlikely that MTC UEs support disaster roaming. On the other hand, if MTC UEs supports disaster romaing, the network can reject disaster roaming MTC UEs during NAS attach procedures. Such rejections cause a lot of signaling overhead. RAN2 should disucss the following options.

Proposal 6: 	Disucss the following options:

Option 1: 		Don’t support the EAB parameters for disaster roaming UEs.

Option 2: 		Support the EAB parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether EAB is applicable to a disaster roaming UE. RAN2 decides whether to support the EAP parameters for disaster roaming UEs based on CT1/SA1 reply.

ACDC is an access control mechanism for the operator to allow/prevent new access attempts from particular, operator-identified applications. Different operators may have their specific applications not known by each other. If the ACDC barring is supported for disaster roaming UEs, it is questionable how the network sets the ACDC barring parameters for disaster roaming UEs. RAN2 should disucss the following options:

Proposal 7: 	Disucss the following options:

Option 1: 		Don’t support the ACDC parameters for disaster roaming UEs.

Option 2: 		Support the ACDC parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether ACDC is applicable to a disaster roaming UE. RAN2 decides whether to support the EAB parameters for disaster roaming UEs based on CT1/SA1 reply.

For non-disaster roaming, access barring parameters are included in SIB2. Access barring parameters for disaster roaming can only be configured when the disaster roaming information is broadcast. Therefore, we prefer to include the access barring parameters for disaster roaming and the disaster roaming information in the same SIB.

Proposal 8: 	Access barring parameters for disaster roaming are included with the disaster roaming information in the same SIB.

### 2.2.2 	Disaster roaming specific access barring parameters per PLMN or common for all PLMNs

A network may provide disaster roaming services for other networks based on their agreements. The agreements may be different between the network providing disaster roaming services and the other networks. The network may want to configure different sets of access barring parameters for different networks with disaster conditions based on their agreements. Therefore, RAN2 should support per-PLMN disaster roaming specific access barring parameters.

Proposal 9: 	Support per-PLMN disaster roaming access barring parameters.

## 2.3 	Access barring check for disaster roaming

If the disaster roaming specific access barring parameters follow the legacy design as described above, the access barring check in the RRC specification can be reused.

Proposal 10: 	Reuse the existing access barring check for disaster roaming specific access barring

The disaster roaming specific access barring parameters are optional. If disaster roaming specific access barring parameters are not broadcasted, disaster roaming UEs should use the legacy access barring parameters (if broadacst) for access barring check.

Proposal 11: 	If the disaster roaming specific access barring parameters are not broadcast on the cell, disaster roaming UEs should use the legacy access barring parameters (if broadacst) for access barring check.

# Conclusion

We provide the following proposals to support MINT for E-UTRA/EPC. We also provide CRs based on our preferred options. We will update the CRs based on the RAN2 agreements.

Proposal 1: 	Discuss which of these options is the most suitable approach for providing configuration of disaster roaming information.

Option 1: 		Reuse SIB30 with existing configurations, i.e., the existing configurations are shared between E-UTRA/EPC and E-UTRA/5GC.

Option 2:		Reuse SIB30 with R19 configurations.

Option 3:		Define a new SIB.

Proposal 2: 	Support the ACB parameters (i.e., for access control for emergency call, mobile originated signalling, and mobile originated data) for disaster roaming UEs.

Proposal 3: 	Support the SSAC parameters (i.e., for MMTEL voice and MMTEL video) for disaster roaming UEs.

Proposal 4: 	Support barring skip parameters for MMTEL voice, MMTEL video, and SMS for disaster roaming UEs.

Proposal 5: 	Disucss the following options:

Option 1: 		Don’t support the ACB parameter for CSFB for disaster roaming UEs.

Ootion 2: 		Support the ACB parameter for CSFB for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether CSFB is applicable to a disaster roaming UE. RAN2 decides whether to support the ACB parameter for CSFB for disaster roaming UEs based on CT1/SA1 reply.

Proposal 6: 	Disucss the following options:

Option 1: 		Don’t support the EAB parameters for disaster roaming UEs.

Option 2: 		Support the EAB parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether EAB is applicable to a disaster roaming UE. RAN2 decides whether to support the EAP parameters for disaster roaming UEs based on CT1/SA1 reply.

Proposal 7: 	Disucss the following options:

Option 1: 		Don’t support the ACDC parameters for disaster roaming UEs.

Option 2: 		Support the ACDC parameters for disaster roaming UEs.

Option 3: 		Send an LS to CT1/SA1 to ask whether ACDC is applicable to a disaster roaming UE. RAN2 decides whether to support the EAP parameters for disaster roaming UEs based on CT1/SA1 reply.

Proposal 8: 	Access barring parameters for disaster roaming are included with the disaster roaming information in the same SIB.

Proposal 9: 	Support per-PLMN disaster roaming access barring parameters.

Proposal 10: 	Reuse the existing access barring check for disaster roaming specific access barring

Proposal 11: 	If the disaster roaming specific access barring parameters are not broadcast on the cell, disaster roaming UEs should use the legacy access barring parameters (if broadacst) for access barring check.

# Reference

[1] 24.812 v19.1.0 Study on MINT support in EPS for 5G-only national roaming UE

[2] CP-251282 New WID on CT aspects of MINT support in EPS for 5G-only national roaming UE

[3] C1-252195 Conclusions for Key Issue #7, agreed CR to 24.812,  Google, InterDigital, Nokia, China Telecom

[4] R2-2506827 Introduction of MINT for EPS, 36.331 CR

[5] R2-2506829 Introduction of MINT for EPS, 36.300 CR

[6] R2-2506830 Introduction of MINT for EPS, 36.306 CR