3GPP TSG-RAN WG2 Meeting #131bis	R2-256829

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1924 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MINT in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CT1 has agreed to support MINT in EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifiy MINT is an optional feature for EPS without AS capability indications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Disaster roaming for EPS is not supported in 36.306. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 5155r1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.300 CR 1431r1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2506193 |  |  |  |  |  |  |  |  |

## 6.8	Other features

### 6.8.1	System Information Block Type 16

It is optional for UE, including UEs of any ue- Category-NB, to support the reception of SystemInformationBlockType16 as specified in TS 36.331 [5].

### 6.8.2	QCI1 indication in Radio Link Failure Report

It is optional for the UE to include drb-EstablishedWithQCI-1 in RLF-Report as specified in TS 36.331 [5].

### 6.8.3	Enhanced random access power control

It is optional for UE to support enhanced random access power control for FDD as specified in TS 36.321 [4] and TS 36.213 [22], clauses 16.2.1.1.1 and 16.3.1. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.8.4	MO-EDT for Control Plane CIoT EPS Optimization

It is optional for UE to support MO-EDT for Control Plane CIoT EPS optimizations as specified in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.5	Void

### 6.8.6	Enhanced PHR

It is optional for UE to support enhanced PHR in MSG3 for FDD, as defined in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.8.7	void

### 6.8.8	Resynchronization Signals

It is optional for UE to support resynchronization signals, as defined in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.9	Measurement gaps for higher UE velocity

It is optional for UE to support measurement gaps for higher UE velocity, as defined in TS 36.331 [5] and TS 36.133[16]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.10	MT-EDT for Control Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports 'MT-EDT for Control Plane CIoT EPS Optimisation' it shall support 'MO-EDT for Control Plane CIoT EPS Optimisation' as described in clause 6.8.4. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.11	MT-EDT for User Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for User Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports 'MT-EDT for User Plane CIoT EPS Optimisation' it shall support earlyData-UP-r15 as described in clause 4.3.8.7. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.12	Void

### 6.8.13	Reduced MIB/SIB1-BR acquisition time

It is optional for UE to support reduced MIB/SIB1-BR acquisition time requirements as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeB-r13.

### 6.8.14	High speed dedicated network features

It is optional for UE to support HSDN cell reselection handling in RRC_IDLE and RRC_INACTIVE (if the UE supports eutra-5GC-r15) as specified in TS 36.304 [14] and TS 36.331 [5].

### 6.8.15	Carrier specific NRSRP thresholds for NPRACH resource selection

It is optional for UE to support carrier specific NRSRP thresholds for NPRACH resource selection as specified in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB and multiCarrier-NPRACH-r14 or multiCarrierPagingTDD-r15.

### 6.8.16	Protection against improper reselection to GERAN/UTRAN

It is optional for UE to support protection against improper reselection to GERAN/UTRAN as specified in TS 36.304 [14].

### 6.8.17	Inter-RAT cell reselection of an NR mobile IAB cell

It is optional for UE to support inter-RAT cell reselection priority handling of an NR mobile IAB cell in RRC_IDLE and RRC_INACTIVE (if the UE supports eutra-5GC-r15) as specified in TS 36.304 [14] and TS 36.331 [5].

### 6.8.18	Inter-RAT measurement on an NR NTN cell

It is optional for UE in RRC_IDLE, or in RRC_INACTIVE (if the UE supports eutra-5GC-r15) to support inter-RAT measurement for cell reselection from an E-UTRA terrestrial network cell to an NR NTN cell as specified in TS 36.304 [14] and TS 36.331 [5].

### 6.8.x	Minimization of service interruption

It is optional for UE to support minimization of service interruption including reporting to NAS of disaster roaming information for available PLMNs and Access Barring check, as specified in TS 24.301 [28] and TS 36.331 [5].