3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2506833

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.8.2

Source:	vivo

Title:	Discussion on RIL V206

Document for:	Decision

# Introduction

In this contribution, we would like to further discuss the RIL issues [V206]. Specifically, we will provide our understanding of how to capture the following agreement achieved in the previous RAN2 meeting [1].

RAN2#131 Agreement RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area, with legacy behavior applicable (FFS whether we capture this in the spec).

# Discussion

In the latest RRC spec, the mapping between an MBS broadcast session (TMGI) and ISA ID(s) is achieved by extending the MBS-SessionInfoList. Specifically, a broadcast service (identified by TMGI) can be associated with an ISA ID list, which may be empty or include one or more ISA IDs. The signalling details are provided below,

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoListExt-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoExt-r19

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfoExt-r19 ::=       SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-AreaPerSessionPerArea-r19)) OF MBS-IntendedAreaID-r19 OPTIONAL, -- Need R

...

}

Figure 1: Mapping between broadcast service and ISA list

Take the above configuration as an example. For the broadcast service 3, which is not assocaited with any explicit area ID(s), from the Rel-19 UE point of view, if we don’t capture the agreement mentioned above, it is not clear whether the service is not allowed to receive by the R19 UE due to no allowed ISA is indicated or the the UE is allowed to receive it within the whole cell area. There exists UE implementation ambiguity.

During the ASN.1 review phase, the RRC rapporteur commented that further explanation was not needed for this, as the UE applied legacy mechanisms if the ISA (ID) is not provided for a certain service. Usually, it is only the new functionality that needs to be clarified in new parameters. In our understanding, it is not sufficient to only clarify new functionality through new parameters in the field description. For a new configuration, even if it is not indicated, the expected legacy behavior of the UE must still be explicitly specified. This is critical because without such clarification, UEs supporting new functionalities would lack clear guidance on whether to fall back to legacy behavior, leading to potential interoperability issues. Moreover, such explicit capture of legacy behavior in the absence of a configuration has established precedents in existing specifications. For example:

- When channelAccessMode2 is not configured, the UE is specified to follow legacy behavior by not performing channel access procedure.

- When rrc-SSB-Subset or sdt-SSB-Subset is not configured, the UE assumes the SSB set includes all actually transmitted SSBs indicated by ssb-PositionsInBurst, the same as R15 behavior.

- When connectedReporting is not configured, the UE is specified to follow legacy behavior by not reporting a preference to stay in RRC_CONNECTED state.

These precedents demonstrate that clarifying UEs’ operation of legacy behavior when a configuration is absent is essential for ensuring consistent UE behavior. Therefore, we propose to capture the aforementioned agreement as follows,

- Change the Need code for mbs-SessionAreaList-r19 from Need R to Need S;

- Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent.

Proposal 1: (RIL V206) Change the need code for mbs-SessionAreaList-r19 from Need R to Need S.

Proposal 2: (RIL V206) Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent.

The corresponding TP is given in the Annex for your further reference.

# 3 Conclusion

In this contribution, we have discussed the RIL V206 issue on MBS broadcast in Rel-19 NTN. And we make the following proposals,

Proposal 1: (RIL V206) Change the need code for mbs-SessionAreaList-r19 from Need R to Need S.

Proposal 2: (RIL V206) Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent.

# 4 Reference

[1] R2-2506702, RAN2#131 Meeting Report.

# 5 Annex

#### –	MBS-SessionInfoList

The IE MBS-SessionInfoList provides the list of ongoing MBS broadcast sessions transmitted via broadcast MRB and, for each MBS broadcast session, the associated G-RNTI and scheduling information.

MBS-SessionInfoList information element

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoList-v19xy ::=    SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-v19xy

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfo-v19xy ::=        SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-AreaPerSession-r19)) OF MBS-IntendedAreaID-r19 OPTIONAL  -- Need S

}

DRX-ConfigPTM-Index-r17 ::=          INTEGER (0..maxNrofDRX-ConfigPTM-1-r17)

PDSCH-ConfigIndex-r17  ::=           INTEGER (0..maxNrofPDSCH-ConfigPTM-1-r17)

MTCH-SSB-MappingWindowIndex-r17  ::= INTEGER (0..maxNrofMTCH-SSB-MappingWindow-1-r17)

MRB-ListBroadcast-r17 ::=            SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17

MRB-InfoBroadcast-r17 ::=            SEQUENCE {

pdcp-Config-r17                      MRB-PDCP-ConfigBroadcast-r17,

rlc-Config-r17                       MRB-RLC-ConfigBroadcast-r17,

...

}

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S

headerCompression-r17                CHOICE {

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}    OPTIONAL -- Need S

}

MRB-RLC-ConfigBroadcast-r17 ::=      SEQUENCE {

logicalChannelIdentity-r17           LogicalChannelIdentity,

sn-FieldLength-r17                   ENUMERATED {size6}                       OPTIONAL, -- Need S

t-Reassembly-r17                     T-Reassembly                             OPTIONAL  -- Need S

}

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

MBS-SessionInfoList field descriptions

g-RNTI G-RNTI used to scramble the scheduling and transmission of MTCH.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in downlink.

mbs-SessionId Indicates an identifier of the MBS session provided by the MTCH.

mbs-SessionAreaList Indicates the list of intended service areas associated with an MBS broadcast session in an NTN cell. If this field is absent, then the UE considers the associated service can be received within the entire cell area.

mrb-listBroadcast A list of broadcast MRBs to which the associated broadcast MBS session is mapped to.

mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. The field is absent when mbs-NeighbourCellList is absent or an empty mbs-NeighbourCellList is signalled. If this field is absent when mbs-NeighbourCellList is absent or a non-empty mbs-NeighbourCellList is signalled, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. If this field is absent and an empty mbs-NeighbourCellList is signalled, then the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell.

mtch-schedulingInfo Indicates the index of DRX configuration entry in drx-ConfigPTM-List that is used for scheduling the MTCH. The value 0 corresponds to the first entry in drx-ConfigPTM-List, the value 1 corresponds to the second entry in drx-ConfigPTM-List and so on. In case mtch-schedulingInfo is absent for a G-RNTI (i.e. no PTM DRX), the UE shall monitor for PDCCH scrambled with G-RNTI in any slot according to the search space configured for MTCH [see TS 38.213 [13], clause 10.1].

mtch-SSB-MappingWindowIndex Indicates the index of MTCH-SSB-MappingWindowCycleOffset configuration entry in MTCH-SSB-MappingWindowList. The value 0 corresponds to the first entry in MTCH-SSB-MappingWindowList, the value 1 corresponds to the second entry in MTCH-SSB-MappingWindowList and so on. This field is set to the same value for all MBS sessions mapped to the same G-RNTI.

pdcp-SN-SizeDL Indicates that PDCP sequence number size of 12 bits is used, as specified in TS 38.323 [5]. When the field is absent the UE applies the value as specified in 9.1.1.7.

pdschConfigIndex Indicates the index of PDSCH configuration entry in pdschConfigList for MTCH. Value 0 corresponds to the first entry in pdschConfigList, the value 1 corresponds to the second entry in pdschConfigList and so on. When the field is absent the UE applies the first entry in pdschConfigList for MTCH.

sn-FieldLength Indicates that the RLC SN field size of 6 bits is used, see TS 38.322 [4]. When the field is absent the UE applies the value as specified in 9.1.1.7.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. When the field is absent the UE applies the value in specified in 9.1.1.7.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms1 corresponds to 1 ms, value ms10 corresponds to 10 ms, and so on. When the field is absent the UE applies the value as specified in 9.1.1.7.

| Conditional Presence | Explanation |
| --- | --- |
| MTCH-Mapping | The field is mandatory present if the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 is more than 1, and searchspaceMTCH is not set to zero (including the case where searchSpaceMTCH is absent and searchSpaceMCCH is not set to zero). Otherwise, it is absent, Need R. |