3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2506838

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Discussion on RIL V215

Document for:	Decision

# Introduction

In this contribution, we would like to further discuss the RIL issue V215 proposed in [1].

# V215 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. V215; IoTNTN; 2; Removal of p0-UE-PUSCH-r19; Yes, R2-2506838; vivo (Stephen); v008; ToDo. [Description]: The field p0-UE-PUSCH-r19 is totally not used in CB-Msg3 PUSCH power control, as per the P0 formula from MAC - the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep; Note that the p0-UE-PUSCH for PUR is used as a power offset to calculate the initial received target power for PUR based on preambleInitialReceivedTargetPower. As we had introduced cb-Msg3-InitialReceivedTargetPower-r19, there is no need to use p0-UE-PUSCH and preambleInitialReceivedTargetPower for CB-Msg3 PUSCH. [Proposed Change]: Removal of p0-UE-PUSCH-r19 from both CB-Msg3-ConfigSIB and cb-Msg3-ConfigSIB-NB [Comments]: Rapporteur’s comments: I have some sympathy with this proposal. Since the proponent is going to prepare a contribution, we can discuss during the next meeting.

# Discussion

As per 36.213, the UE transmit power  for PUSCH transmission in subframe/slot/subslot i for the serving cell is given by

[dBm]

where is a parameter composed of the sum of a component  provided from higher layers for j=0, 1 and 3 and a component  provided by higher layers for j=0, 1 and 3 for serving cell . For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1, for PUSCH (re)transmissions corresponding to the random access response grant then j=2 and for BL/CE UE PUSCH (re)transmission using preconfigured uplink resource then j=3.  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell .

From the above, we can know that the  for PUR transmission is calculated based on the parameters preambleInitialReceivedTargetPower,, and p0-UE-PUSCH included in PUR-Config-r16. In our understanding, p0-UE-PUSCH is configured as a power offset to adjust the target P0 for PUR transmission, based on the link performance gap between the preamble and PUR PUSCH. A proper configuration of p0-UE-PUSCH helps guarantee the transmission reliability of PUR PUSCH.

Observation 1: As there is a dependency of power control for PUSCH and preamble, p0-UE-PUSCH configured for PUR transmission is utilized as a power offset to adjust the target P0 for PUR transmission, based on the link performance gap between the preamble and PUR PUSCH.

For CB-Msg3-EDT PUSCH, the power control procedure differs fundamentally as per the following text from MAC spec,

-	the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

Specifically, with the introduction of cb-Msg3-InitialReceivedTargetPower-r19, a dedicated parameter explicitly configuring the target P0 for CB-Msg3-EDT PUSCH transmission, independent configuration of the target P0 specific to CB-Msg3-EDT PUSCH is enabled. This parameter thereby decouples the power control for CB-Msg3-EDT PUSCH entirely from preambleInitialReceivedTargetPower. Accordingly, the technical justification for retaining the power offset p0-UE-PUSCH in this scenario is eliminated. Retention of p0-UE-PUSCH as specified in the current document would introduce redundant parameters and result in unnecessary signaling overhead. Furthermore, its retention may introduce ambiguity for RAN1 when drafting the UE behavior of power control for CB-Msg3-EDT PUSCH.

Observation 2: The parameter cb-Msg3-InitialReceivedTargetPower-r19, a dedicated parameter explicitly configuring the target P0 for CB-Msg3-EDT PUSCH transmission, decouples the power control for CB-Msg3-EDT PUSCH entirely from preambleInitialReceivedTargetPower.

Therefore, we propose removing p0-UE-PUSCH from the CB-Msg3-EDT configuration, as its role is fully superseded by the parameter cb-Msg3-InitialReceivedTargetPower-r19.

Proposal: (RIL V215) RAN2 agrees to remove the parameter p0-UE-PUSCH-r19 from cb-Msg3-ConfigSIB(-NB).

# 3 Conclusion

In this contribution, we have discussed the remaining issues on CB-Msg3. And we made the following observations and proposal,

Observation 1: As there is a dependency of power control for PUSCH and preamble, p0-UE-PUSCH configured for PUR transmission is utilized as a power offset to adjust the target P0 for PUR transmission, based on the link performance gap between the preamble and PUR PUSCH.

Observation 2: The parameter cb-Msg3-InitialReceivedTargetPower-r19, a dedicated parameter explicitly configuring the target P0 for CB-Msg3-EDT PUSCH transmission, decouples the power control for CB-Msg3-EDT PUSCH entirely from preambleInitialReceivedTargetPower.

Proposal: (RIL V215) RAN2 agrees to remove the parameter p0-UE-PUSCH-r19 from cb-Msg3-ConfigSIB(-NB).

# 4 Reference

[1] R2-2507060, RIL status on IoT NTN Ph3, Huawei, HiSilicon.