3GPP TSG-RAN WG2 Meeting #131bis                                                   R2-2506841

Prague, Czech Republic, Oct. 13th-17th 2025

Source:              CATT

Title:	             Leftover Issues on User Plane

Agenda Item:	8.7.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss leftover issues on user plane as follows:

- [MAC-D2] The priority of LCH for UL grant priority

- [RLC-X01] whether to remove the RLC calculation of the non-delay reporting data

- [RLC-V01] whether indicate the SN of obsolete RLC SDUs

- [RLC-N02] The NOTE in clause 5.3.3.3

- Further clarify the agreement on MAC-13

- The logical channel selection for additional priority determination

Our views are also given in the conclusion section.

Discussion

[MAC-D2] The priority of LCH for UL grant priority

TS 38.321 For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

For above part procedure, the issue [MAC-D2] proposed that the meaning of highest priority for a logical channel in UL grant priority is not clear in yellow part, in our view it is better to aligned to the description on UL grant priority in green part to avoid the unclear understanding that the highest priority is always the additional priority. The revision is as below:

In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority among the applied priorities of the logical channel in the LCP procedure(see clause 5.4.3.1.3), in which the logcial channle are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU .

Proposal 1: [MAC-D2] Revise the description of logical channel priority for UL grant priority as follows:

In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority among the applied priorities of the logical channel in the LCP procedure(see clause 5.4.3.1.3), in which the logcial channle are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU .

[RLC-X01] whether to remove the RLC calculation of the non-delay reporting data

In [POST131][507][XR] email discussion, one open issue is proposed to discuss whether to remove the RLC calculation of the non-delay reporting data.

Based on the logic in last meeting, the PDCP entity has a full picture of the information of the order and remaining time of each PDCP data SDU and PDU, so it could determine whether the PDCP data SDU/PDU(i.e., RLC SDU data) is delay/non-delay reporting data associated to each reporting threshold in PDCP layer and RLC layer. Then, RAN2 agreed that RLC layer is not necessary to compute whether the RLC SDU or segment is delay reporting or non-delay reporting data, as the PDCP will give an indication associated with the i:th DSR-ReportingThreshold for the PDCP Data PDU to lower layers.  RLC entity will compute the data volume based on its status and the data indication from PDCP layer. However, the email discussion proposed an issue on the implementation on RLC handling, e.g. whether PDCP and RLC have the same packet placement order, and whether PDCP and RLC share single IP packet copy in physical memory but maintaining respective protocol headers at UE side. In our view, the reasonable implementation should be SDU handling in RLC layer follows the same order as in PDCP layer, then, RLC layer will not compute the non-delay reporting RLC data SDU by itself.

Proposal 2: [RLC-X1] RLC layer follows the same order as in PDCP layer for SDU handling， remove the RLC calculation of the non-delay reporting data.

[RLC-V01] whether indicate the SN of obsolete RLC SDUs

In [POST131][507][XR] email discussion, one open issue is proposed to discuss whether to the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

We think the statement is true that “the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery.”  However, this is not new issue introduced by unnecessary retransmission avoidance in RLC. And this indication may also impact PDCP specification. Considering the timeline, we can keep this topic and continue the discussion in Release 20. Therefore we propose the receiving RLC entity does not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

Proposal 3: [RLC-V01] The receiving RLC entity does not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

[RLC-N02] The NOTE in clause 5.3.3.3

TS 38.322 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Since above Note is in TS 38.322 clause 5.3.3.3 which specifies the condition to stop and reset t-PollRetransmit. And we think it may miss the scenario for negatively acknowledged to delete “or negatively” from the Note. We prefer to keep the current Note.

Proposal 4: Keep the current Note in clause 5.3.3.3 that “NOTE:	When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.”.

Further clarify the agreement on MAC-13

In RAN2#130 meeting, the agreement on MAC-13 is as follows:

(MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority.

In current MAC running CR R2-2506349, if pdu-SetDiscard is configured, the PDU Set remaining time of the evaluated PDCP SDU is always used to evaluate with priorityAdjustmentThreshold, listed below:

1> if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission: 2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or 2> if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold: 3> consider this PDCP SDU being priority adjustable.

There’re two options to further clarify the agreement on MAC-13:

- Option 1: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in current LCP procedure with adjusted priority.

- Option 2: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in the current and future LCP procedure which handles PDCP SDUs of this PDU Set with adjusted priority.

Observation 1: There’re two options to further clarify the agreement on MAC-13:

- Option 1: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in current LCP procedure with adjusted priority.

- Option 2: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in current and future LCP procedure that handles PDCP SDUs belonging to this PDU Set with adjusted priority.

The following case explains the difference between the two options.

For example, we can look at below figure 1. PDCP SDUs(1-10) belongs to a PDU Set, and priorityAdjustmentThreshold is 10ms. At time T1, assume the remaining time of PDCP SDU 1 is 8ms and the remaining time of PDCP SDU 2 is 9 ms, which is less than priorityAdjustmentThreshold. The remaining time of PDCP SDU 1 is the shortest remaining time till discardTimer expiry among the remaining time of the PDCP SDUs belonging to the PDU Set. The PDU Set remaining time of the PDCP SDU is 8ms and the PDCP SDU1/2/3/4/5/6/7/8/9/10 is priority adjustable but PDCP SDU1/2/3/4/5(the green ones) is transmitted. At time T2, it is assumed that PDCP SDU1/2/3/4/5 was successfully received, and the transmitting PDCP entity received a PDCP status report and discarded PDCP SDU1/2/3/4/5 in buffer. If the logical channel is selected for UL grant, and assume the remaining time of the PDCP SDU6 is 11ms. There’re two different LCP results based on option 1 and option 2:

- In case option 1, PDCP SDU6/7/8/9/10 is not considered being priority adjustable.

- In case option 2, PDCP SDU6/7/8/9/10 is considered being priority adjustable.

Therefore we propose RAN2 to discuss and further clarify the agreement on MAC-13 based on option 1 and option 2.

Proposal 5: RAN2 discuss and further clarify the agreement on MAC-13 based on option 1 and option 2.

If option 2 is preferred, follow the current MAC running CR, the remaining time of PDCP SDU6 is the shortest remaining time till discardTimer expiry among the remaining time of the PDCP SDUs belonging to the PDU Set. The PDU Set remaining time is 11ms and PDCP SDU 6 would be evaluated not to adjust priority. The result of LCP procedure based on current MAC running CR is conflict with option 2.

Figure 1 LCP procedure if pdu-SetDiscard is configured

Observation 2:The LCP procedure based on current MAC specification does not support option 2.

In our understanding, to solve this issue, one place in MAC running CR need to be fixed:

-	Correct the procedural text in subclause 5.4.3.1.3 so that if pdu-SetDiscard is configured, consider a PDCP SDU to be priority adjustable without evaluating with the PDU Set remaining time of this PDCP SDU again in case the PDU Set of this PDCP SDU has PDCP SDU(s) which have been considered being priority adjustable before.

Proposal 6: If option 2 is preferred, RAN2 fix the logical channel priority adjustment evaluation procedure in MAC:

-	Correct the procedural text in subclause 5.4.3.1.3 so that if pdu-SetDiscard is configured, consider a PDCP SDU to be priority adjustable without evaluating with the PDU Set remaining time of this PDCP SDU again in case the PDU Set of this PDCP SDU has PDCP SDU(s) which have been considered being priority adjustable before.

The corresponding TP for Proposal 6 are attached in section 4.1.

The logical channel selection for additional priority determination

The allocation of resources section in LCP is as follows:

TS 38.321 The MAC entity shall, when a new transmission is performed: 1> if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission: 2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or 2> if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold: 3> consider this PDCP SDU being priority adjustable. 1> allocate resources to the logical channels as follows: 2> if a logical channel has a priority adjustable PDCP SDU: 3> apply additionalPriority of this logical channel; 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain: 3> if a logical channel has applied additionalPriority in the above allocation of resources and does not have any priority adjustable PDCP SDUs left: 4> apply priority of this logical channel; 3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels with equal applied priority should be served equally. NOTE 1: The value of Bj can be negative.

Based on above spec on additional priority of LCP, it can be seen that additional priority could be determined when new transmission is applied and the reaming time could be computed based on the time point at PUSCH transmission, we are fine to this procedure.

However, according to current procedure, the additional priority determination is applied to all configured logic channels, no matter it is the selected logical channel from the 5.4.3.1.2 as described in resource allocation section or not. In fact, only the data from the selected logical channel will be multiplexed in resource allocation part, the additional priority determination for not selected logical channel seems not necessary, it is suggested to restrict the additional priority determination to the selected logical channel to reduce the UE computing complexity. Otherwise, as seen in the figure 2, the UE needs to compute the remaining time of each PDCP SDU or shortest reaming time of each PDU set for each configured logical channel, it will increase the UE computing complexity.

Figure 2 The selected logical channel(s)

Observation 3: The additional priority determination for not selected logical channel seems not necessary and increases the UE computing complexity.

Furthermore, besides the LCP procedure, the additional priority is also associated to below yellow part. The priority of each multiplexed logic channel will be used to determine the UL grant priority, which also totally depends on the priority of the selected logic channel in the resource allocation part of LCP section.  Thus, if the additional priority determination is restricted to selected logical channel in LCP section, it has no impact to other procedure associated with the additional priority.

TS 38.321 5.4.1 UL Grant reception For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions. <skip> For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Observation 4: Restrict the additional priority determination to selected logical channel has no additional impact to UL grant priority determination.

Proposal 7: It is suggested to restrict the additional priority determination to the selected logical channel in clause 5.4.3.1.2.

The corresponding TP for Proposal 7 are attached in section 4.2.

Conclusion

According to the analysis in section 2, the following observations and proposals were made:

Proposal 1: [MAC-D2] Revise the description of logical channel priority for UL grant priority as follows:

In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority among the applied priorities of the logical channel in the LCP procedure(see clause 5.4.3.1.3), in which the logcial channle are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU .

Proposal 2: [RLC-X1] RLC layer follows the same order as in PDCP layer for SDU handling， remove the RLC calculation of the non-delay reporting data.

Proposal 3: [RLC-V01] The receiving RLC entity does not indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

Proposal 4: Keep the current Note in clause 5.3.3.3 that “NOTE:	When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.”.

Observation 1: There’re two options to further clarify the agreement on MAC-13:

- Option 1: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in current LCP procedure with adjusted priority.

- Option 2: When pdu-SetDiscard is configured, PDU Set should be treated as a whole in current and future LCP procedure that handles PDCP SDUs belonging to this PDU Set with adjusted priority.

Proposal 5: RAN2 discuss and further clarify the agreement on MAC-13 based on option 1 and option 2.

Observation 2:The LCP procedure based on current MAC specification does not support option 2.

Proposal 6: If option 2 is preferred, RAN2 fix the logical channel priority adjustment evaluation procedure in MAC:

-	Correct the procedural text in subclause 5.4.3.1.3 so that if pdu-SetDiscard is configured, consider a PDCP SDU to be priority adjustable without evaluating with the PDU Set remaining time of this PDCP SDU again in case the PDU Set of this PDCP SDU has PDCP SDU(s) which have been considered being priority adjustable before.

The corresponding TP(TS38.321) for Proposal 6 are attached in section 4.1.

Observation 3: The additional priority determination for not selected logical channel seems not necessary and increases the UE computing complexity.

Observation 4: Restrict the additional priority determination to selected logical channel has no additional impact to UL grant priority determination.

Proposal 7: It is suggested to restrict the additional priority determination to the selected logical channel in clause 5.4.3.1.2.

The corresponding TP(TS38.321) for Proposal 7 are attached in section 4.2.

Annex

Text Proposal 6

--------------------------------------- Start of the text proposal 6----------------------------------------------------

5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission:

2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard:

3>if the PDU Set of this PDCP SDU has PDCP SDU(s) which have been considered being priority adjustable before:

4>consider this PDCP SDU being priority adjustable.

3>else if the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold:

4>consider this PDCP SDU being priority adjustable.

2>	if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold:

3> consider this PDCP SDU being priority adjustable.

--------------------------------------------- End of the text proposal 6-----------------------------------------------

Text Proposal 7

--------------------------------------- Start of the text proposal 7----------------------------------------------------

5.4.3.1.3	Allocation of resources

The MAC entity shall, when a new transmission is performed:

1>	if a logical channel selected in clause 5.4.3.1.2 is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission:

2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or

2>	if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold:

3> consider this PDCP SDU being priority adjustable.

1>	allocate resources to the selected logical channels as follows:

2> if a logical channel has a priority adjustable PDCP SDU:

3>  apply additionalPriority of this logical channel;

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain:

3> if a logical channel has applied additionalPriority in the above allocation of resources and does not have any priority adjustable PDCP SDUs left:

4> apply priority of this logical channel;

3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels with equal applied priority should be served equally.

NOTE 1:	The value of Bj can be negative.

--------------------------------------------- End of the text proposal 7----------------------------------------------