3GPP TSG-RAN WG2 Meeting #131bis	R2-2506849

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Remaining issue on Redcap UE for OD-SIB1 and Paging Adaptation

Document for:	Discussion, Decision

Introduction

This paper provides our views on some remaining issues for on-demand SIB1.

Redcap for OD-SIB1

According to the agreed CR on 321:

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request or SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

In legacy, the SI request on NUL ends up with monitoring of Redcap-specific DL BWP (if configured).

Currently, the SIB1 request configuration only includes a single controlResourceSetZero-r19/ searchSpaceZero-r19 configuration, i.e., no redcap BWP specific configuration.

OD-SIB1-Config-r19  ::=       SEQUENCE {

[…]

controlResourceSetZero-r19               ControlResourceSetZero,

searchSpaceZero-r19                      SearchSpaceZero,

[…]

}

Then this paragraph hints that:

- When initialDownlinkBWP-RedCap is configured (after SIB1 acquisition), UE is supposed to rely on PDCCH configured on initialDownlinkBWP-RedCap. Then from network perspective, upon reception of a SIB1 request, it has no idea whether it is from a UE has acquired SIB1, or a UE has not. And then it is hard to decide which PDCCH to use for response, the one for initialDownlinkBWP or the one for initialDownlinkBWP-RedCap.

- Or another approach is to introduce another pair of configurations, one for SIB1 request configuration for initialUplinkBWP-Redcap, and the other for PDCCH configuration for initialDownlinkBWP-Redcap, so that network can differentiate between the two types of SIB1 request, and thus respond via corresponding BWP (normal or Redcap-specific BWP) properly.

For case-2, if they are put into the new SIBx, it can take effect before SIB1 acquisition. Or if it is configured in SIB1, it can only take effect after SIB1 acquisition, and thus the same controlResourceSetZero-r19/ searchSpaceZero-r19 configuration in OD-SIB1-Config-r19 is to carry the response to both normal UE and Redcap UE.

We slightly prefer avoiding the usage of initialUplink/DownlinkBWP-Redcap for request/response of OD-SIB1. If so, the current ASN1 design works already, but implicitly hint that the configuration of SIB1-request has to be within the bandwidth requirement of (e)Redcap UE.

initialDownlinkBWP The initial downlink BWP configuration for a serving cell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

initialUplinkBWP The initial uplink BWP configuration for a serving cell (see TS 38.213 [13], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

Or if the usage of initialUplink/DownlinkBWP-Redcap specific configuration is preferred, R2 has to discuss how to handle the ASN.1 design, covering both SIB1 request configuration for uplink, and PDCCH configuration for DL.

In either way, 321 has to be corrected

- If configuration specifically for initialUplink/DownlinkBWP-Redcap is not needed, the monitoring on initialDownlinkBWP-Redcap based on 321 should be avoided.

- If configuration specifically for initialUplink/DownlinkBWP-Redcap is needed, the 331 spec has to introduce the corresponding configuration.

R2 discuss whether to rely on initialUplink/DownlinkBWP-Redcap specific configuration for the request/response of OD-SIB1 for (e)Redcap UE. If yes, how to do that, and correct 38.331 specification accordingly. If no, remove the related part in 38.321.

Based on the agreed CR, the capability for OD-SIB1 is specified as follows

| onDemandSIB1-r19 Indicates whether the UE supports the on-demand request procedure of SIB1 as specified in TS 38.331 [9]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

It does not mention the difference due to the support of Redcap UE, when it comes to initial DL BWP.

Table 1 Comparison between non-Redcap / Redcap UE for Paging Adaptation support

|  | For the support of onDemandSIB1 |
| --- | --- |
| For non-Redcap UE | Support OD-SIB1-Config in SIBxx |
| For Redcap UE | Support OD-SIB1-Config in SIBxx (when initialDownlink/UplinkBWP-RedCap is not configured, and initialDownlink/Uplink BWP is within Redcap UE BW limitation) Support OD-SIB1-Config for Redcap-specific BWP (when initialDownlink/UplinkBWP-RedCap is configured) |

During Post-131 discussion, seems more companies tend to avoid separate capability bits for Redcap UE and non-Redcap UE, then at least the functionality difference has to be clarified in 306. E.g.,

| onDemandSIB1-r19 Indicates whether the UE supports the on-demand request procedure of SIB1 as specified in TS 38.331 [9]. UE supports xxx configured for initialDownlink/UplinkBWP-RedCap if it supports supportOfRedCap or supportOfERedCap-r18. | UE | No | No | No |
| --- | --- | --- | --- | --- |

If R2 would like to support OD-SIB1 for RedCap UE, and OD-SIB1 configuration is differentiated based on initialDownlink/UplinkBWP-RedCap, R2 clarify in 306 that the support of OD-SIB1 has different implementation for Redcap and non-Redcap UE.

If R2 decide to rely on initialUplink/DownlinkBWP-Redcap specific configuration for the request/response of OD-SIB1 for (e)Redcap UE, R2 discuss the solution to differentiate the support of onDemandSIB1-r19 for non-redcap UE and redcap UE, e.g., via the TP on 38.306 as above.

[306 OI] Redcap for Paging Adaptation

Based on the agreed CR, the capability for paging adaptation is specified as follows

| pagingAdaptation-r19 Indicates whether the UE supports paging adaption, in which the value range for parameter N and Ns as defined in TS 38.331[9] are extended to make it possible to have increased interval between Paging Frames and compensate the decrease in the number of Paging Frames. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| pagingAdaptionPEI-SupportBandList-r19 Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band for paging adaption. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs. A UE supporting this feature shall also indicate support of pagingAdaptation-r19. | UE | No | No | No |

Neither of the two mentioned the difference due to the support of Redcap UE, when it comes to initial DL BWP.

Table 1 Comparison between non-Redcap / Redcap UE for Paging Adaptation support

|  | For the support of pagingAdaptation | For the support of paingAdaptationPEI-SupportBandList |
| --- | --- | --- |
| For non-Redcap UE | Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 in PCCH-Config in DownlinkConfigCommonSIB | Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O-r19 in PDCCH-ConfigCommon in initialDownlinkBWP |
| For Redcap UE | Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 in PCCH-Config in DownlinkConfigCommonSIB (when initialDownlinkBWP-RedCap is not configured, and initialDownlinkBWP is within Redcap UE BW limitation) Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon in initialDownlinkBWP-RedCap (when initialDownlinkBWP-RedCap is configured) | Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O-r19 in PDCCH-ConfigCommon in initialDownlinkBWP (when initialDownlinkBWP-RedCap is not configured, and initialDownlinkBWP is within Redcap UE BW limitation) Support pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O-r19 in PDCCH-ConfigCommon in initialDownlinkBWP-RedCap (when initialDownlinkBWP-RedCap is configured) |

During Post-131 discussion, seems more companies tend to avoid separate capability bits for Redcap UE and non-Redcap UE, then at least the functionality difference has to be clarified in 306. E.g.,

| pagingAdaptation-r19 Indicates whether the UE supports paging adaption, in which the value range for parameter N and Ns as defined in TS 38.331[9] are extended to make it possible to have increased interval between Paging Frames and compensate the decrease in the number of Paging Frames. UE supports pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 configured in initialDownlinkBWP-RedCap if it supports supportOfRedCap or supportOfERedCap-r18. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| pagingAdaptionPEI-SupportBandList-r19 Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band for paging adaption. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs. UE supports pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O-r19 configured in initialDownlinkBWP-RedCap if it supports supportOfRedCap or supportOfERedCap-r18. A UE supporting this feature shall also indicate support of pagingAdaptation-r19. | UE | No | No | No |

R2 discuss the solution to differentiate the support of PO bundling (i.e., pagingAdaptation-r19) and PEI support for PO bundling (i.e., pagingAdaptionPEI-SupportBandList-r19) for non-(e)Redcap UE and (e)Redcap UE, e.g., via the TP on 38.306 as above.

Conclusion

We have the following proposals:

Proposal 1	R2 discuss whether to rely on initialUplink/DownlinkBWP-Redcap specific configuration for the request/response of OD-SIB1 for (e)Redcap UE. If yes, how to do that, and correct 38.331 specification accordingly. If no, remove the related part in 38.321.

Proposal 2	If R2 decide to rely on initialUplink/DownlinkBWP-Redcap specific configuration for the request/response of OD-SIB1 for (e)Redcap UE, R2 discuss the solution to differentiate the support of onDemandSIB1-r19 for non-redcap UE and redcap UE, e.g., via the TP on 38.306 as above.

Proposal 3	R2 discuss the solution to differentiate the support of PO bundling (i.e., pagingAdaptation-r19) and PEI support for PO bundling (i.e., pagingAdaptionPEI-SupportBandList-r19) for non-(e)Redcap UE and (e)Redcap UE, e.g., via the TP on 38.306 as above.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR