3GPP TSG-RAN WG2 Meeting #131bis	R2-2506852

Prague, Czech Republic, Oct. 13th - 17th, 2025	revision of R2-2505242

Agenda Item:	8.12.3

Source:	OPPO

Title:	Clarification on the coexistence between LTM or CLTM and UL-only TRP

Document for:	Discussion, Decision

# Introduction

According to the discussion in the RAN2#130 meeting [2], RAN2 made the following agreements for the UL-only TRP configuration for LTM:

- RAN2#130 meeting agreement sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

In this contribution, we provide our analysis on the coexistence issue between LTM or C-LTM (Conditional-LTM) and UL-only TRP, when PL offset is configured.

# Discussion

## Coexistence issue of LTM and UL-only TRP

Rel-19 Mobility WID [3]: - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

3GPP TS 38.321 [4]: Rel-18 LTM Cell Switch Command MAC CE:

The agreed MAC CR [5] for Rel-19 mobility WI: Rel-19 LTM Cell Switch Command MAC CE:

According to the WID of Rel-19 mobility item, compared with the Rel-18 LTM, the Rel-19 LTM is to support the extra scenarios of inter-CU mobility, for which the configuration of the asymmetric DL sTRP/UL mTRP within a candidate configuration is not impacted. According to the agreed CR for Rel-19 mobility item, compared with the Rel-18 LTM Cell Switch Command MAC CE, the Rel-19 Enhanced LTM Cell Switch Command MAC CE adds an extra “NCC value” field, which does not cause extra impacts on the configuration of UL-only TRP.

- RAN2#130 meeting agreement: sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

According to the RAN2#130 meeting agreement as quoted above, sDCI mTRP 2TA can be configured for the LTM candidate configuration, and no MAC specification change is needed.

WID of Rel-19 MIMO: - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

According to the Rel-19 MIMO WID as quoted above, the asymmetric DL sTRP/UL mTRP is only for intra-band intra-DU case, this means that during the inter-gNB or inter-DU handover, the target node is not able to provide the configuration for the asymmetric DL sTRP/UL mTRP, including the pathloss offset configuration. Then the inter-CU signalling is avoided for the configuration of asymmetric DL sTRP/UL mTRP. This means that when inter-CU LTM is configured, the inter-CU signalling is not able to add extra indication of the pathloss offset.

For the LTM procedure, we think that for the intra-DU LTM, the pathloss offset configuration can still be provided via the candidate LTM configuration to improve the uplink throughput. As the pathloss offset can be changed when the UE moves around, the UE needs to be updated with the latest pathloss offset if the UL only TRP is to be used during the cell switching procedure triggered by LTM Cell Switch Command MAC CE. The coordination (e.g. SRS reception at the target node and pathloss offset exchanging) between the source node and target node is needed. However, as the LTM procedure is used to reduce the handover latency, and the UL-only TRP is used to improve the uplink throughput, the gNB can still indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE, and the latest pathloss offset for the uplink-only TRP can be indicated after the completion of the handover. There is no need to add the pathloss offset indication in the LTM Cell Switch Command MAC CE, and there is also no need to add a restriction to prohibit the pathloss offset configuration in the LTM candidate configuration.

Observation 1: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via (enhanced) LTM Cell Switch Command MAC CE without adding the pathloss offset, and the latest pathloss offset can be provided after LTM cell switch.

Proposal 1: For both Rel-18 LTM and Rel-19 LTM, the pathloss offset can be included in the LTM candidate configuration, without adding the pathloss offset in (enhanced) LTM Cell Switch Command MAC CE.

The agreed RRC CR [5] for Rel-19 MIMO WI:

3GPP TS 38.331:

Regarding the early RACH procedure for the LTM, RAN2 needs to clarify whether the preamble for the early RACH procedural can be transmitted to the UL-only TRP. According to the agreed CR of TS 38.331 for Rel-19 MIMO, the configuration (i.e. pathlossOffsetPRACH-DCI-1-0 and prachAssociationDCI-1-0) of allowing the PDCCH-order indicating pathloss offset is included in the BWP-DownlinkDedicated, and the EarlyUL-SyncConfig has no configuration of BWP-DownlinkDedicated. This means that the PDCCH-order indicating pathloss offset cannot be used for the early RACH procedure. Regarding whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP, as the PRACH resource for early RACH is associated to SSB which is not restricted to a specific TRP type and there is no DL feedback (i.e. no Msg2) needed for the early RACH preamble, no extra specification change is needed to clarify whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP. If the target node by implementation is able to receive the preamble of early RACH via the UL-only TRP, it can indicate the TA for the UL-only TRP after LTM cell switch.

Observation 2: The RRC configuration of early RACH is not able to add the configuration for the UL-only TRP.

Proposal 2: The UL-only TRP is not used for the early RACH procedure for LTM. No extra specification change is needed.

## Coexistence issue of C-LTM and UL-only TRP

Rel-19 Mobility WID [3]: - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

The agreed MAC CR [5] for Rel-19 MIMO WI: #### 6.1.3.YY Pathloss Offset Update MAC CE The Pathloss Offset Update MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with the following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell to which the MAC CE is applied. The length of this field is 5 bits; - BWP ID: This field indicates a BWP for which the MAC CE is applied as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. If the value of unifiedTCI-StateType in the Serving Cell indicated by Serving Cell ID is joint, this field indicates a DL BWP to which the MAC CE is applied. If the value of unifiedTCI-StateType in the Serving Cell indicated by Serving Cell ID is separate, this field indicates a UL BWP to which the MAC CE is applied. The length of this field is 2 bits; - TCI state ID: This field indicates a TCI state identified by TCI-StateId or TCI-UL-State-Id as specified in TS 38.331 [5]. The length of this field is 7 bits. If the value of unifiedTCI-StateType in the Serving Cell indicated by Serving Cell ID is joint, this field indicates a TCI-StateId for a joint TCI state. If the value of unifiedTCI-StateType in the Serving Cell indicated by Serving Cell ID is separate, the most significant bit of the field is considered as the reserved bit and remainder 6 bits indicate a TCI-UL-State-Id for a UL TCI state; - Pathloss Offset: This field indicates the updated value of pathloss offset for the TCI state indicated by the preceding TCI state ID field. The range of the indicated pathloss offset is from -12 dB to 60 dB with a step size of 4 dB. The value 0 corresponds to -12 dB, the value 1 corresponds to -8 dB and so on. The values from 19 onwards are reserved. The length of this field is 5 bits; - R: Reserved bit, set to 0. Figure 6.1.3.YY: Pathloss Offset Update MAC CE

Regarding the configuration of the asymmetric DL sTRP/UL mTRP in the candidate configuration of C-LTM, according to the agreed MAC/RRC CR for the Rel-19 MIMO in [6] and [7], the UL-only TRP can be assigned with an TCI-StateId or TCI-UL-State-Id. According to the C-LTM procedure, when the UE triggers the initial access, an SSB is selected by the UE for the initial access (i.e. both RACH-based and RACH-less). According the MAC specification, the SSB index used for the initial access of C-LTM is not associated to a TCI-state configuration for the UL-only TRP. This means that the UL-only TRP is not able to be selected by the UE during C-LTM handover, and the UE uses the PRACH resource associated to SSB for C-LTM. There is no extra specification change. On the other hand, as the C-LTM is not able to indicate which tci-state is used for the initial access of the target cell, it is also difficult to extend the configuration to indicate the UL-only TRP (with pathloss offset) for C-LTM.

Observation 3: According to the current specification of both RRC and MAC, the UL-only TRP is not able to be selected for the initial access (i.e. both RACH-based and RACH-less) of C-LTM.

Thus, if the network provides the configuration of the asymmetric DL sTRP/UL mTRP in the candidate configuration of C-LTM, the UE will anyway select the symmetric UL/DL TRP. The UL-only TRP with pathloss offset can be indicated/activated after the C-LTM handover by the gNB implementation.

Proposal 3: The gNB by implementation can include the configuration (including pathloss offset) of the asymmetric DL sTRP/UL mTRP in the candidate configuration of C-LTM. No extra specification change is needed.

The agreed MAC CR [5] for Rel-19 mobility WI:

According to the Rel-19 mobility WID [3] and the agreed MAC CR in [5], the C-LTM (Conditional-LTM) is introduced by adding an LTM Candidate Timing Advance Command MAC CE for the early RACH procedure for C-LTM, in addition to the Rel-18 early RACH procedure. As discussed above, the RRC configuration of early RACH is not able to add the configuration for the UL-only TRP.

Proposal 4: The UL-only TRP is not used for the early RACH procedure for C-LTM. No extra specification change is needed.

# Conclusion

According to the analysis give above, we have the following observations and proposals:

Observation 1: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via (enhanced) LTM Cell Switch Command MAC CE without adding the pathloss offset, and the latest pathloss offset can be provided after LTM cell switch.

Observation 2: The RRC configuration of early RACH is not able to add the configuration for the UL-only TRP.

Observation 3: According to the current specification of both RRC and MAC, the UL-only TRP is not able to be selected for the initial access (i.e. both RACH-based and RACH-less) of C-LTM.

LTM

Proposal 1: For both Rel-18 LTM and Rel-19 LTM, the configuration (including pathloss offset) of the asymmetric DL sTRP/UL mTRP can be included in the LTM candidate configuration, without adding the pathloss offset in (enhanced) LTM Cell Switch Command MAC CE.

Proposal 2: The UL-only TRP is not used for the early RACH procedure of LTM. No extra specification change is needed.

C-LTM

Proposal 3: The gNB by implementation can include the configuration (including pathloss offset) of the asymmetric DL sTRP/UL mTRP in the candidate configuration of C-LTM. No extra specification change is needed.

Proposal 4: The UL-only TRP is not used for the early RACH procedure for C-LTM. No extra specification change is needed.

# Reference

[1] 3GPP TS38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification”.

[2] RAN2#130 meeting minutes

[3] RP-250339, “Revised Work Item: NR mobility enhancements Phase 4”

[4] 3GPP TS38.321, “NR; Medium Access Control (MAC) protocol specification”

[5] R2-2506229, “Introduction of NR mobility enhancements Phase 4 in MAC”

[6] R2-2506587, agreed RRC CR, “Introduction of MIMO Phase 5”

[7] R2-2506539, agreed MIMO CR, “Introduction of MIMO”