3GPP TSG-RAN WG2 Meeting #131bis	R2-2506868

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	[C003] Clarification on how UE uses the service area information in SIB and USD

Document for: 	Discussion and Decision

Introduction

SA4 supported service area types of polygon and circle in service announcement, offering a standalone/parallel alternative to the RAN2 defined SIB+MCCH-based intended service area solution. In some cases, the service area provided via these two approaches may not be completely aligned, e.g., due to the different formats or not timely updating. Considering the service area is used in several procedures, e.g., MCCH acquisition, reselection priorities handling and may be MRB establishment/release (proposed by [E014]), we should carefully define how UE determines the service area if the Target Service Area in the USD or the ISA in SIB27 are not totally aligned. We proposed a RIL [C003] for this issue during the ASN.1 review phase.

# C003 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C003; NTN; 1; clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIB27 are not totally aligned; Yes, R2-250xxxxx; CATT (Da Wang); v008; ToDo. [Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIB27 are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope. [Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIB27. NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD [Comments]:

In this contribution, the RIL [C003] is discussed with giving TP.

Discussion

The service announcement and “SIB+MCCH” approaches may provide different service areas for the same MBS broadcast service in the following cases. Company may assume that network implementation should ensure alignment between the service areas provided by the two approaches. However, it is hard to guarantee that all the time.

- Case1: The circles in service announcement use the PointUncertaintyCircle format, whereas those in the SIB27 are expressed as reference location with radius. Because these two formats have different quantisation steps, the resulting service areas may not be perfectly aligned.

- Case2: After the UE acquires service area via service announcement in RRC_CONNECTED state, it may enter RRC_IDLE state and try to receive the MBS broadcast service using the received service area before ahead. In an NTN cell, the UE could also acquire the service area by reading the SIB. When the service area of an MBS broadcast service is changed (after the UE enters RRC_IDLE state), the UE may acquire the updated service area by reading the SIB which is different from the service area stored in the USD.

In the current CRs [1][2], there is no clear definition on how the UE determines the actual service area when the SIB and USD providing inconsistent service area information. Especially for the second case as mentioned above, if the UE use invalid service area information by implementation, it may miss the updated MCCH or inappropriate processing the frequency priority.

Therefore, it is better to have a clear clarification on how UE uses the service area information in SIB and USD. Considering the SIB approach for NTN cells could provide more granular and up-to-date service area to UEs in any RRC states, it is proposed that if the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD. Considering service area is used in several procedures, including MCCH acquisition, reselection priorities handling, we can have a common illustration in section “5.2.2.4.29 Actions upon reception of SIB27”.

Proposal 1: Add a NOTE “NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.” in section 5.2.2.4.29 Actions upon reception of SIB27. Adopt the TP in Annex.

Conclusion

In this contribution, RIL [C003] is discussed with the proposal listed as follows:

Proposal 1: Add a NOTE “NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.” in section 5.2.2.4.29 Actions upon reception of SIB27. Adopt the TP in Annex.

Reference

- R2-2506522	Introduction of NTN Phase 3 enhancements.

- R2-2506547	Introduction of R19 NR NTN.

Annex

##### 5.2.2.4.29	Actions upon reception of SIB27

No UE requirements related to the contents of SIB27 apply other than the following NOTE and those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

NOTE: If the service area information is broadcast in the SIB27, the UE ignores the Target Service Area in the USD.