3GPP TSG-RAN WG2 Meeting #131bis										R2-2506895

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.3.2

Source: 	Sharp

Title:  	Initial Considerations for 6GR Control Plane

Document for: 	Discussion

# Introduction

This document discusses 6GR control plane aspects that should be considered at the early stage of discussion.

# Discussion

## RRC Modelling and Connection Management

Figure 1. UE state machine and state transitions in NR [1]

In NR, three RRC states are supported, i.e., RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE as depicted in Figure 1. From the UE perspective, there are not many differences between RRC_IDLE and RRC_INACTIVE. The main difference is the network behaviour that the core network maintains the UE contexts for fast reconnection. From RAN perspective, RRC Establishment, RRC Resume procedure and corresponding UE behaviours were specified in RAN specifications. It brought additional complexity. We see that either RRC_IDLE or RRC_INACTIVE would be sufficient, or we can consider merging those states to take beneficial parts.

Proposal 1: 6GR aims to have only two RRC states.

If the stored RRC configuration can be simply reused in the next RRC connection, it is reasonable to store the previous configuration. Even if a different RRC configuration can be used in the next RRC configuration, the stored configuration can be kept. The new RRC configuration can be re-configured after the UE becomes connected again.

Proposal 2: When UE releases the RRC connection, the UE stores the previous RRC configurations as a baseline.

For connection management, to implement RRC state transition between RRC_CONNECTED and RRC_IDLE (or RRC_INACTIVE), three-step RRC connection establishment procedure and one-step RRC connection release procedure controlled by the network have been used in LTE and NR. In our view, those have been working well, and we consider them as a baseline.

Furthermore, the concept of RLM/RLF is being used for radio link recovery. In 6G, bad coverage problem will still exist. The radio link recovery is vital. Therefore, we see that existing concept and procedure should also be kept in 6GR. With this in mind, RRC connection re-establishment procedure framework in NR can also be a reference.

Proposal 3: NR connection management framework of RRC connection establishment procedure/RRC release procedure/RRC re-establishment procedure is considered as baseline of 6GR.

## IDLE/INACTIVE Mode Operations

In NR, the IDLE/INACTIVE mode procedures for cell status and cell reservations have been expanded in a patchy manner, whenever a new feature was introduced. Since Release 15, we introduced cell barring status for ATG, 2Rx XR, eRedCap, VSAT (fixed and mobile), NES, NTN, half duplex RedCap, IAB, NCR and Mobile IAB. The consequence of this patchy approach was a need to specify complicated rules of how to treat those cell status bits (e.g., what to consider and what to ignore) based on the device capabilities, consuming a lot of time in online/offline discussion time to reach consensus. In 6GR, we should study a simple but flexible approach implemented at the first place to accommodate with future extensions.

Proposal 4: Study a simple but flexible approach for cell status/cell restrictions to accommodate with future extensions.

The cell reselection procedure in 5G may be in the similar situation. Specifically, in reselection priorities handling, we have introduced special handling rules for HSDN, V2X, mobile IAB, MBS, NCR, CAG and network slices. For the same reason as presented above for cell status/cell restrictions, in 6GR we should study a simple and future-proof design for cell reselection priority handling.

Proposal 5: Study a simple and future-proof design for cell reselection priority handling.

## Synchronization signal and system information

In NR, synchronization signals and essential system information are transmitted periodically. For initial cell selection, the UE assumes that SS/PBCH block (SSB) is transmitted with 20ms periodicity, which has implications for network power consumption.

Once a cell is detected, the UE can obtain neighbour cell/frequency information, including SMTC, enabling efficient detection and measurement of neighbour cells. This mechanism contributes to both UE energy saving and network energy saving (NES). Furthermore, On-demand SSB and On-Demand SIB1 mechanisms were introduced for NES purpose.

For 6G, considering the above mechanisms, it is important to study a framework for broadcasting synchronization signals and system information that ensures efficient initial access while improving overall energy efficiency. Although NR has multiple independent mechanisms for different purposes, 6GR study should discuss all together to prepare a comprehensive solution.

Proposal 6: Study a framework for broadcasting synchronization signals and system information that ensures efficient initial access and improved energy efficiency.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: 6GR aims to have only two RRC states.

Proposal 2: When UE releases the RRC connection, the UE stores the previous RRC configurations as a baseline.

Proposal 3: NR connection management framework of RRC connection establishment procedure/RRC release procedure/RRC re-establishment procedure is considered as baseline of 6GR.

Proposal 4: Study a simple but flexible approach for cell status/cell restrictions to accommodate with future extensions.

Proposal 5: Study a simple and future-proof design for cell reselection priority handling.

Proposal 6: Study a framework for broadcasting synchronization signals and system information that ensures efficient initial access and improved energy efficiency.

# References

[1] TS 38.331	NR RRC specification