3GPP TSG-RAN WG2 Meeting #131bis										R2-2506896

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.3.3

Source: 	Sharp

Title:  	Initial Considerations of 6GR UP/CP Common Aspects

Document for: 	Discussion

# Introduction

This document discusses 6GR security and energy saving aspects that should be considered at the early stage of the study.

# Discussion

## Security

|  | Ciphering | Integrity Protection |
| --- | --- | --- |
| User Data | Protected | Optional |
| SRB message | Protected | Protected |
| PDCP/RLC control | Not protected | Not protected |
| MAC CE | Not protected | Not protected |
| MAC header | Not protected | Not protected |

Table 1. Security protection in NR

Table 1 shows the security coverage in NR security mechanisms. In NR, only PDCP data units (data from PDCP point of view) are security protected by ciphering and integrity protection. All control messages (e.g. MAC CEs, PDCP/RLC status report, other PDCP feedback) are not security protected. It can be a security risk which is becoming a bigger potential problem in mobile networks. NR MAC specification [1] introduced a number of MAC CEs as faster lower-layer control mechanisms. Compared to LTE [2], the number of MAC CEs have been significantly increased. An evolution trend is that we rely more on lower-layer signalling which is however not security protected. Especially, LTM Cell Switch Command MAC CE which includes NCC value is used for UE mobility control whose security protection is very important. In 6G, RAN2 needs to discuss how to support the security protection for control messages. As data security protection should be mandatorily required, it would be nicer to have a common and unified security framework for both data and control message.

Proposal 1: RAN2 to study security protection (i.e. ciphering and integrity protection) of control messages (e.g. MAC CE, PDCP/RLC control PDUs).

Proposal 2: RAN2 to study a unified security framework for data and control messages.

## Energy Saving

In LTE, network energy-saving solution was not considered as macro cells in most of cases and micro cells in dense urban environments are major deployment scenarios. In NR, high-frequency usage e.g., FR2 mandates denser cell-site deployments where network energy consumption is not negligible anymore. In 6G, we see that high-frequency usage and dense cell-site deployments will not be precluded, so the network energy saving will be also very important.

In NR, various NES solutions were introduced in backward compatibility manner, so the gain had some limitations. Also, 5G NES solutions has limitations on RRC states. The initial version of 6G should support NES solutions without any backward compatibility issues. We see that NES solutions can be applicable to all RRC states.

Proposal 3: RAN2 to study 6G NES solutions commonly applicable to all RRC states. Rel-18/19 5G NES features are baselines of 6GR.

For 6G, UE power saving should be considered in the initial version. Both RRC_CONNECTED and RRC_IDLE/INACTIVE modes need to be studied. In NR, many solutions are introduced for UE power saving, including RRM measurement relaxation, RLM/BFD relaxation, DCP, LP-WUS, PEI, etc. Some solutions are for special UEs e.g., RedCap UE, UE with LR. Then, which solutions to be included or reused in the initial version of 6G should be discussed.

Proposal 4: RAN2 to study UE power saving and discuss which solutions in NR can be reused in the initial version of 6G.

Importance of network energy saving does not mean that increased UE energy consumption is acceptable. The 6G network energy saving mechanisms shall not sacrifice UE energy consumption. We see that a baseline of UE energy saving can be NR DRX. RAN2 should discuss 6G network/UE energy saving mechanism to guarantee similar UE energy saving performance with NR DRX.

Proposal 5: RAN2 to study on network/UE energy saving mechanisms to keep comparable UE energy saving performance with NR DRX.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: RAN2 to study security protection (i.e. ciphering and integrity protection) of control messages (e.g. MAC CE, PDCP/RLC control PDUs).

Proposal 2: RAN2 to study a unified security framework for data and control messages.

Proposal 3: RAN2 to study 6G NES solutions commonly applicable to all RRC states. Rel-18/19 5G NES features are baselines of 6GR.

Proposal 4: RAN2 to study UE power saving and discuss which solutions in NR can be reused in the initial version of 6G.

Proposal 5: RAN2 to study on network/UE energy saving mechanisms to keep comparable UE energy saving performance with NR DRX.

# References

[1] TS 38.321	NR MAC specification

[2] TS 36.321 LTE MAC specification