3GPP TSG-RAN WG2 Meeting #131bis										R2-2506897

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.3.3

Source: 	Sharp

Title:  	6GR AI/ML Framework

Document for: 	Discussion

# Introduction

In Release 18 and Release 19, RAN2 developed the initial frameworks and Life Cycle Management (LCM) for AI/ML features, based on use cases such as beam management, CSI prediction/compression, mobility, and positioning. For all the considered scenarios, only one-sided models (network-sided or UE-sided) were considered for development, with two-sided models (network and UE) being deprioritized. RAN2 also considered the initial data collection and data transfer procedures both from the network and UE perspectives, although standardized solutions for UE-sided data collection were later deprioritized during Release 19. Although some enhancements are already planned during Release 20 for AI/ML features, a general 6GR AI/ML framework encompassing support for future scenarios needs to be considered from the beginning.

In this contribution, we discuss the general aspects for consideration in developing a 6GR AI/ML framework from RAN2 perspective.

# Discussion

## Unified AI/ML Framework

It was noted in the 6G RAN-level SID that the 6GR shall leverage the 5G AI/ML framework and study further use cases of interest. In 5G-Advanced, the AI/ML models are one-sided, whereas two-sided models requiring coordination between the network and the UE may be developed in 6GR. To facilitate this, 6GR needs to study a unified architecture and LCM procedure, considering the potential advantages provided by one and two-sided models for future use cases. This requires that the protocol and architectures to support unified procedures are considered from the early stages of 6GR for forward-compatibility. Similar considerations can also be made for data collection and data management for 6GR AI/ML features. A limited set of new use cases, for example, energy efficiency (network/UE-sided) can be studied to develop the 6GR AI/ML architecture.

Observation 1: 6GR is expected to build on the 5G AI/ML framework while expanding the study toward additional use cases of interest.

Proposal 1: RAN2 to study a forward-compatible unified AI/ML architecture, including lifecycle management (LCM), data collection and management to support future use cases.

Proposal 2: RAN2 to develop a unified architecture for AI/ML LCM considering both one-sided and two-sided models, ensuring consistency and flexibility for diverse 6G use-cases.

## Data Collection

In NR, NW-side data collection has been supported, and UE-side data collection has been studied but more work is still needed. In 6GR with two-sided models, additional considerations must be made for data collection and data transfer procedures for efficient utilization of network and UE resources, especially considering the expected UE power consumption and signalling overhead. For 6G, both NW-side data collection and UE-side data collection should be studied, and NW-side data collection can be prioritized.

Observation 2: In 6GR, two-sided models require careful consideration of data collection and transfer procedures to balance efficient resource use with UE power consumption and signalling overhead.

Proposal 3: RAN2 to study both NW-side data collection and UE-side data collection.

Proposal 4: RAN2 to prioritize NW-side data collection over UE-side data collection.

5G NR UE may be expected to provide some data for the network-side LCM to deal with new use cases. The possibility of acquiring the data from the 5G NR UE should also be taken into account at least in the initial version of the 6GR AI/ML LCM. This provides the possibility to enhance the overall network performance in inter-RAT deployments including 5G NR and 6GR devices.

Proposal 5: RAN2 to study the feasibility of collecting 5G NR UE data and enabling its use by the 6GR AI/ML LCM within a unified data collection framework.

## AI/ML-Awareness

For 6GR, an AI-native design is envisioned, supporting AI-driven intelligent interaction between the network and devices across different layers of the protocol stack. The study phase will aim to identify promising use cases and establish a flexible AI framework covering lifecycle management (LCM) and data aspects. It needs to be ensured that the existing features remain fully operable without relying solely on AI functions. For this, 6GR must ensure efficient switching between AI/ML-based and non-AI-ML-based procedures taking into account both UE and network side operations. In line with existing 3GPP assumption, AI/ML models themselves are not standardized, the focus is on signalling control, measurements and lifecycle handling to ensure testability and consistent behaviour.

Observation 3: 6GR envisions an AI-native design that enables intelligent, AI-driven interaction between the network and devices across multiple protocol layers.

Proposal 6: RAN2 to identify and focus on a targeted set of new 6G AI/ML use cases in Rel-20 to enhance UE and network performance.

## AI/ML reporting

In NR, AI/ML and SON/MDT share a common objective: improving network performance through intelligent optimization. These functions were standardized separately, and therefore each has its own reporting format and reporting events. To avoid unnecessary reporting and complex UE procedures, we believe 6GR study should be conducted comprehensively, taking into account reporting mechanisms other than AI/ML that have been considered for 5G NR. A unified approach in 6GR would help streamline signalling procedures and reduce overhead, especially as networks become increasingly dense and heterogeneous. By harmonizing reporting frameworks, more efficient data collection and decision-making across multiple optimization layers can be enabled. This would also facilitate better interoperability between legacy and future systems, ensuring smoother transitions and more robust performance enhancements. RAN2 needs to support consistent reporting across AI/ML and SON/MDT to avoid duplication and reduce UE overhead, reusing existing triggers/formats where possible.

Observation 4: In NR, both AI/ML and SON/MDT target network performance optimization but were standardized independently, resulting in separate reporting formats and events.

Proposal 7: RAN2 to study an integrated reporting framework that consolidates reporting for network optimization purposes.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Observation 1: 6GR is expected to build on the 5G AI/ML framework while expanding the study toward additional use cases of interest.

Proposal 1: RAN2 to study a forward-compatible unified AI/ML architecture, including lifecycle management (LCM), data collection and management to support future use cases.

Proposal 2: RAN2 to develop a unified architecture for AI/ML LCM considering both one-sided and two-sided models, ensuring consistency and flexibility for diverse 6G use-cases.

Observation 2: In 6GR, two-sided models require careful consideration of data collection and transfer procedures to balance efficient resource use with UE power consumption and signalling overhead.

Proposal 3: RAN2 to study both NW-side data collection and UE-side data collection.

Proposal 4: RAN2 to prioritize NW-side data collection over UE-side data collection.

Proposal 5: RAN2 to study the feasibility of collecting 5G NR UE data and enabling its use by the 6GR AI/ML LCM within a unified data collection framework.

Observation 3: 6GR envisions an AI-native design that enables intelligent, AI-driven interaction between the network and devices across multiple protocol layers.

Proposal 6: RAN2 to identify and focus on a targeted set of new 6G AI/ML use cases in Rel-20 to enhance UE and network performance.

Observation 4: In NR, both AI/ML and SON/MDT target network performance optimization but were standardized independently, resulting in separate reporting formats and events.

Proposal 7: RAN2 to study an integrated reporting framework that consolidates reporting for network optimization purposes.