3GPP TSG RAN WG2 Meeting #131bis	R2-2506902

Prague, Czech Republic, 13th – 17th October, 2025

Title:	Issue 1-7 Security parameter in A-IoT paging

Source:	CMCC, China Unicom, China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia, Fujitsu, Interdigital, Xiaomi, vivo, LGE, Panasonic, Transsion Holdings, Quanray Electronics, ETRI

Agenda Item:	8.2.2

Document for:	Discussion

Introduction

In RAN2#131 meeting [1], one open issue on how to capture the security in A-IoT paging message remains:

1.RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not.

In this contribution, we further discuss on how to capture security parameter in A-IoT paging message.

Discussion

According to TS 33.369, authentication is mandatory capability for A-IoT device. In authentication procedure, if used by the network, the RANDAIOT_n is included in the A-IoT paging message for the device to generate the RESAIOT, which is used by the AIOTF to verify the device [2].

In this contribution, we investigate whether the 128-bit security parameter, i.e., RANDAIOT_n, is one mandatory or optional field in A-IoT paging message, from RAN2 signaling design perspective.

Future extension and forward compatibility of paging format

As other A-IoT message format discussion in R19, the future extension and forward compatibility are important aspects to consider. For example, RAN2 design of the security field in paging message should guarantee forward compatible with the potential security enhancement in R20, e.g., the different length of security parameter or different/new security parameters, since it is not exactly the same deployment scenario as in R19 topology 1.

Just in case that security parameter changes in R20, we think it is not worth to introduce a brand-new A-IoT paging message type just because of the change of this single security field. Note that there is only 2~3 R2D message type value reserved and there could be several message enhancements in R20.

Therefore, to support possible security parameter changes in R20, it is suggested to use one 1-bit presence indicator (e.g., value 0) to indicate the presence of R19 security parameter. We can use the other value (e.g., value 1) to indicate the R19 security parameter is absent and the presence of a new security parameter of future release. This seems to be one safer way to deal with potential uncertainties in future SA3 discussions from RAN2 signalling design perspective.

Observation 1:	For forward compatibility and future extension purpose, 1-bit presence indicator is needed to indicate whether the R19 security field is present, or R19 security field is absent and possible R20 security field is present in the future.

Striving for better performance

In RAN2#131 meeting, RAN2 has expressed our space pressure in A-IoT paging message in the LS out (R2-2506465). However, larger size of paging ID is still requested by SA2 in this meeting (S2-2507793).

Based on the current A-IoT MAC specification, we have about up to 776 bits MAC PDU for the paging message, i.e., 78% usage of the upper bound of R2D TBS limitation:

- About 6 bytes AS fields;

- maximum 600 bits for paging ID, considering the new LS from SA2;

  - This is unavoidable, since this is the major function of paging message, i.e., indicating which A-IoT device(s) are paged.

  - Note that this is optional for network to choose different size of paging ID, i.e., this is variable and configurable size.

- 128 bits security parameters, based on SA3 request;

  - Note that this size is approximately three times that of all RAN1 and RAN2-related fields.

The R19 device 1 is supposed to be deployed in indoor scenario. Some operators/networks may consider certain R19 indoor isolated private networks sufficiently secure, as they are not vulnerable to "Msg3 replay attacks from fake devices". In such scenarios, the network may prioritize lower overhead, for example, not always including the 128 bit RANDAIOT_n in the A-IoT paging message, and not including the 32bit RESAIOT and 128 bit RANDAIOT_d in MSG3, in order to guarantee the performance, i.e. achieving higher inventory efficiency. In such scenarios, the network may prioritize lower overhead in the inventory procedure also to achieve better coverage.

Observation 2: Due to the overhead pressure of the A-IoT paging message, some operators/networks may prefer to not always include security field in A-IoT paging message to achieve higher inventory efficiency and better coverage, in secure indoor deployments.

Apart from the overhead, the power consumption of security parameter calculation for device 1 may also impact the coverage. The peak power consumption for device 1 is ~1uW. And device 1 is capable of limited energy storage (e.g., capacitor). The calculation of security parameters, as well as storing these parameters inside the register, may occupy the power used for R2D and D2R communication, which has impact on the coverage. Some operators/networks may choose to not include the 128-bit random number in the A-IoT paging message, so as to ensure that the device's energy is fully used for communication and also longer power retention duration.

Observation 3: Considering the energy consumption restriction of device 1, some operators/networks may choose to not always include security field in A-IoT paging message to ensure that the device's energy is fully used for communication and also longer power retention duration.

Management of a specific known device

With regards to CFA case for inventorying a single device, this specific device has been known to the network, i.e., knowing its device ID and having done authentication to this device before. In such case, some operators/networks may choose not to include the 128-bit random number always, for instance, when the network has already authenticated a specific known device not long ago.

Some description in TS 33.369 is referenced as below [2]:

- 5.2 Authentication procedure - 5.2.1 General This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network.

Observation 4: In practical deployment, it is unnecessary for operator/network to always keep re-authenticating a specific known device if it has just completed the authentication.

In summary, from RAN2 signaling design perspective, it is proposed:

Proposal 1:	Add a 128-bit field in paging message to carry the security parameter, which is optionally present with 1-bit to indicate its presence.

Conclusion

Based on the discussions mentioned above, we propose that:

Observation 1:	For forward compatibility and future extension purpose, 1-bit presence indicator is needed to indicate whether the R19 security field is present or R19 security field is absent and possible R20 security field is present in the future.

Observation 2: Due to the overhead pressure of the A-IoT paging message, some operators/networks may prefer to not always include security field in A-IoT paging message to achieve higher inventory efficiency and better coverage, in secure indoor deployments.

Observation 3: Considering the energy consumption restriction of device 1, some operators/networks may choose not always to include security field in A-IoT paging message to ensure that the device's energy is fully used for communication and also longer power retention duration.

Observation 4: In practical deployment, it is unnecessary for operator/network to always keep re-authenticating a specific known device if it has just completed the authentication.

Proposal 1:	Add a 128-bit field in paging message to carry the security parameter, which is optionally present with 1-bit to indicate its presence.

Reference

- “RAN2 chair’s notes”, RAN2#131, Bengaluru, India, RAN2 Chair (InterDigital).

- 3GPP TS 33.369 V1.0.0, Sep., 2025.

Text Proposal

- 6.2.1.1	A-IoT Paging message

Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS: This field indicates the TBS of this message. The value can be {1, 2, …, 124, 125} byte(s). The length of the field is 7 bits.

-	Access Type (AT): This field indicates CBRA (when set to 1) or CFA (when set to 0). The length of the field is 1 bit.

For CBRA, the following fields are further included:

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is 6 bits.

-	Paging ID Presence Indication (PIPI): This field indicates whether Paging ID and Length of Paging ID fields are present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present. If present, the length of the field is 8 bits.

-	Paging ID: If present, this field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

-	Security Info Presence Indication (SIPI): The length of the field is 1 bit. This field indicates whether Security information field is present (when set to 1) or absent (when set to 0).

-	Security information: The length of the field is 128 bits. This field contains the security related information and the usage is in according to TS 33.369 [x].

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 18 bits.

-	K: This field indicates that the value K is 1 (when set to 0) or 4 (when set to 1) used for determining monitor window for Random ID Response message. The length of the field is 1 bit.

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

For CFA, the following fields are further included:

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

-	Security Info Presence Indication (SIPI): The length of the field is 1 bit. This field indicates whether Security information field is present (when set to 1) or absent (when set to 0).

-	Security information: The length of the field is 128 bits. This field contains the security related information and its usage is in according to TS 33.369 [x].

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Fill Bits: This field is of variable size, and can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.