3GPP TSG-RAN WG2 Meeting #131bis	R2-2506919

Prague, Czech Republic, Oct. 13th-17th

Agenda item: 		9.7.2

Source:		Lenovo

Title:		Considerations on voice support over IoT-NTN

Document for:		Discussion

- Introduction

The objectives of NTN for IoT Phase 4 (IoT_NTN_Ph4) were updated as follows after RAN#109 meeting:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected.

In this contribution we discuss potential enhancement of NB-IoT-NTN to support IMS voice call over GSO, focusing on the differences between CP option (Control Plane CIoT EPS/5GS optimization) and UP option (User Plane CIoT EPS/5GS optimization).

- Discussion

In cases without TN coverage, especially during disasters when TN infrastructure is unavailable, establishing real-time services including voice calls is essential. The NTN, in particular the GSO satellites, can provide wide, robust and sustainable coverage with sparser constellation. Meanwhile the NB-IoT devices with low complexity has much lower requirement for link budgets. As a result, support of voice over GSO for IoT NTN becomes an attractive way to facilitate global coverage of voice service especially for emergency situations.

To implement the support, the CP-based, UP-based and hybrid-based (CP+UP) options were agreed as potential solutions in SA2, while down-selection is still needed considering the limited TU budget and workload of Rel-20 5G-A. There is also a need to discuss the down-selection from RAN2 perspective as the detailed solutions over Uu are expected to be restricted by the final option selected.

The CIoT signalling reduction optimizations were introduced in LTE to fulfill the requirements of small-size data transmission and low power consumption, wherein two options including Control Plane CIoT EPS/5GS optimization (CP option) and User Plane CIoT EPS/5GS optimization (UP option) were supported. The main differences between the two options are that the CP option uses RRC container message to carry small-size user data in NAS message, while the UP solution suspends connection instead of releasing it after data transmission so that the connection can be easily and quickly resumed with stored context.

From the perspective of supporting voice over GSO for NB-IoT, the differences between CP option and UP option may impact on the following aspects:

|  | CP solution for NB-IoT | UP solution for NB-IoT |
| --- | --- | --- |
| Service differentiation | Support only 1 set of E-RAB level QoS parameters, only SRB1bis could be used for NAS message transmission. Introduction of new RB is needed to differentiate SIP signaling and voice. | Support 2 DRBs wherein one can be used for SIP signaling and the other is used for voice. Service differentiation can be implemented without enhancement. |
| RRC signaling support | RRC reconfiguration is not supported, and as a result SPS is not supported for data transmission either. Enhancements are needed to support RRC reconfiguration or other messages to indicate SPS configuration. | SPS configuration can be indicated via RRC reconfiguration. |
| User plane support | Without PDCP function. Support RLC AM only. No header compression. | With PDCP function. Support RLC AM and UM. Support header compression. |
| Security | Only NAS security | AS and NAS security |

Observation 1: To support voice over NB-IoT NTN, the CP option (User Plane CIoT EPS/5GS optimization) at least needs enhancements to support service differentiation and SPS, while other user plane enhancements are also expected.

Therefore, a preference can be expect from RAN2 perspective.

Proposal 1: From RAN2 perspective the UP option (User Plane CIoT EPS/5GS optimization) to support voice over NB-IoT NTN is preferred with less specification impact expected.

As the decision is still to be made in SA, it is expected that the detailed impacts and solutions will be discussed in RAN2 after RAN#110, i.e., when the down-selection will be done.

- Conclusion

In this contribution we discuss potential enhancement of NB-IoT-NTN to support IMS voice call over GSO. It is observed that:

Observation 1: To support voice over NB-IoT NTN, the CP option (User Plane CIoT EPS/5GS optimization) at least needs enhancements to support service differentiation and SPS, while other user plane enhancements are also expected.

And the following proposal is given:

Proposal 1: From RAN2 perspective the UP option (User Plane CIoT EPS/5GS optimization) to support voice over NB-IoT NTN is preferred with less specification impact expected.