3GPP TSG-RAN WG2#131-bis	R2-2506938

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	10.3.1

Title:	6GR User plane aspects

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objectives about the Radio interface protocol architecture and procedures for 6GR.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we would like to provide initial views on 6GR User Plane (UP) aspects in terms of possible improvements/enhancements from NR.

Discussion

Background: Learn from NR UP functions

The motivation behind developing a 6G Radio Access Technology is deeply rooted in the growing demands and technological evolution expected in the coming decade. As 5G systems mature and become widely deployed, it is becoming increasingly clear that merely evolving existing technologies will not suffice to meet the full range of future use cases, particularly those requiring extreme performance, flexibility, and scalability. 6G study represents an opportunity to revisit the foundations of radio system design, free from the constraints of backward compatibility, and thus focusing on a transformative leap in capability, efficiency, and simplicity. From RAN2 perspective, the study on user plane should emphasize practical user experience, system extensibility, and performance excellence, which are essential to support the anticipated diverse service classes ranging from immersive multimedia and massive IoT to integrated communications and sensing. These ambitions require a holistic and optimized design across all layers of the radio interface and architecture. 6G architecture should ensure alignment with evolving service demands without the compromises imposed by legacy systems.

For 6G, ITU-R has defined usage scenarios and supported services of IMT-2030 as shown in Fig.1. The requirement for services includes immersive communication, sensing, HRLLC, etc.

Fig.1 IMT-2030 scenarios

In 3GPP, new services and corresponding requirements are being studied in SA1 for 6G, where some of them have been captured in TR 22.870 [2] and RAN TSG is also studying on the scenarios and requirements for 6G Radio and the results are reported in TR 38.914 [3]. For example, the minimum requirements for user plane latency are defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios, which are 4ms for Immersive Communication and 1ms for HRLLC. For the upcoming WG 6G study on user plane aspects, it’s recommended to consider all identified services to meet the requirements defined in RAN and SA. From RAN2 perspective, the enhanced requirements like peak data rate, latency and reliability will be a key driver of 6GR UP design.

Proposal 1: For the study on user plane architecture and protocol design, consider all usage scenarios of IMT-2030.

User Plane architecture and protocol design

General requirements for 6GR UP

Most of the services which were supported in 5G NR are expected to also be supported in 6GR to avoid the negative impression from the market that 6GR underperforms 5G NR. This includes e.g. XR, FWA, URLLC, Industrial IoT, LPWA etc. Unified design is expected to be applied for the support of variable existing services/needs which require the same feature. Unified design is also expected to be able to provide better forward compatibility and facilitate the further development of 6G when new services appear particularly in industries in the near future. In the study phase of 6GR, RAN2 should consider and study the possibility to support 6GR requirements of new and existing services by using one design with a good scalability.

Since there are many new and existing services with different requirements, in design of user plane architecture and protocols, it is better to have a compact protocol stack but with flexible features/functions/toolbox adapted by the service needs. In NR, the UP protocol for supporting vertical services, like TSN (Time sensitive network), SDT (small data transmission) or Ethernet header compression (EHC) was not supported in Rel-15, therefore it has implicitly caused market fragmentation.

Therefore it is important for the day-1 design to avoid constraints when evolving technologies in the future.

Proposal 2:	6GR UP protocol should aim to support flexible functions adapted by service needs.

Taking into account the IMT-2030 technical performance requirements (TPRs) in [3], the theoretical data rates required for 6GR would be more than 50Gbps which is much higher than NR (in NR, 20Gbps for DL and 10Gbps for UL), to achieve the requirements, it obviously requires more efficient data delivery mechanism among UP sublayers for both the NW and the UE sides. In addition to the higher data rates, latency requirement should be kept as in NR (4ms for eMBB and 1ms for URLLC) to support low latency services. Given the situation, RAN2 should study 6GR UP protocol to achieve further less processing time than NR as a design target.

Proposal 3:	6GR UP protocol should aim to enable further shorter processing time than NR UP.

Also to reduce the processing time, 6GR UP protocol should target a lean protocol design. We think it would be a good starting point by reducing and/or merging duplicated functions in the existing NR UP sub-layers.

Proposal 4:	6GR UP protocol should aim to lean design than NR UP, e.g., duplicated functions among NR UP layers should be reduced/merged.

SDAP

The SDAP layer is a newly added layer by NR due to the finer user plane data processing based on QoS flow introduced by 5G core network. The main purpose of the SDAP layer is for QoS flow to DRB mapping. This function is quite simple and is most likely to be kept in 6G. If 6G core network will introduce new QoS framework which needs to be reflected in the AS layer protocol design, RAN2 can revisit the needs for enhancements on the SDAP layer. This will be triggered by the study of other WGs, such as SA2. Further, if it is determined that AI/ML or sensing related data transfer will be carried by user plane, then the SDAP layer may need to be enhanced as well.

Proposal 5:	Keep the current functions in the SDAP layer as baseline. RAN2 may consider enhancements of SDAP if triggered by other WGs (e.g., SA2), or by data transfer of AI and sensing discussions.

PDCP & RLC

The PDCP layer has the functions of the following: SN maintenance, header compression and decompression, ciphering and deciphering, integrity protection, SDU discard, duplication, split routing if configured, re-ordering, in-order delivery, and so on.

The RLC layer supports TM, UM, AM modes with basic functions like data transmission, ARQ, segmentation and reassembly, SDU discard, etc.

If we look into the detailed functions in the PDCP layer and the RLC layer, there are some similar or duplicate functions, such as SN maintenance, status report, SDU discard, data volume calculation for BSR and DSR, and so on. Besides, for enhancements to support XR, there are quite some features involving PDCP notifications to the RLC layer, such as delay-critical data indication, delay-reporting data indication for each DSR reporting threshold, trigger indication for remaining time-based retransmission and remaining time-based polling. These cross-layer signaling introduces processing delay and specification difficulties. From overhead point of view, it is also inefficient to have too many sub-headers due to too many sub-layers.

Based on the above reasons, we propose to study on integrating the PDCP layer and the RLC layer into one layer. After integration, SN maintenance, status report, SDU discard, PDU header, data volume calculation can be jointly designed, or redundancy can be removed. The cross-layer notification can be avoided as a result. The CA/DC impact on the integrated layer can be studied. Fig.2 illustrates a new data flow assuming that the RLC layer functions are integrated into the new PDCP layer. During the study, it can also be considered that some functions of the RLC layer are moved to the MAC layer, such as ARQ, which can be jointly designed with HARQ function.

Fig.2 L2 data flow when the PDCP layer integrates the RLC layer functions

Observation 1: NR UP protocol stack is inefficient in terms of header overhead and processing delay.

Observation 2: NR PDCP layer and RLC layer have some similar/duplicate functions, e.g., SN maintenance, status report, SDU discard, data volume calculation, etc.

Observation 3: Some features involve quite many PDCP notifications to RLC layers, which increase UE complexity and standard efforts.

Proposal 6:	RAN2 to study the feasibility and advantages for combining PDCP and RLC into one layer.

MAC

The MAC layer has many basic functions related to data transmission, including (de-)multiplexing, LCP, HARQ, BSR, SR, etc. In 5G NR, the resource allocation (LCP) procedure is designed to meet eMBB service requirement as baseline, targeting to support the prioritized bit rate as the first priority. It is not really friendly to delay-critical services, such as URLLC and XR. Although enhancements for supporting URLLC and XR were specified in 5G later releases, the complexity on UE implementation and uncertainty of the usefulness lead to lack of commercialization of these features.

In the new use cases for 6G, industry and verticals are typical HRLLC scenarios. Thus, it is critical to meet HRLLC service requirements on its reliability and stringent latency. The usage scenario of immersive communication extends the eMBB and covers use cases which provide a rich and interactive video (immersive) experience to users, including the interactions with machine interfaces. 6GR air interface designs for immersive communication need to ensure consistent user experience and improved network capacity, taking into account the composite requirement of high data rate and low latency. In addition, the QoS requirements for immersive communications are different among data streams. Good immersive experience lies on all streams meet their own QoS requirements. Besides, the requirements among different streams are dependent, similar to multi-modality issue discussed for XR in NR. Therefore, composite requirement of data rate, latency, reliability, capacity should be considered. To achieve this, an adaptive service-aware/QoS-aware resource allocation which can adaptively optimize resource allocation with different targets needs to be studied.

Observation 4: NR LCP-based resource allocation procedure is eMBB-centric and not friendly to XR, HRLLC and immersive communication.

Proposal 7: Study on adaptive resource allocation method to meet variable needs for different services, including services with high data rate, services with stringent latency requirement and services with composite requirement.

Conclusion

In this paper we presented some initial views on RAN2 study directions for 6GR UP aspects and have come up with the following observations and proposals:

Proposal 1: For the study on user plane architecture and protocol design, consider all usage scenarios of IMT-2030.

Proposal 2:	6GR UP protocol should aim to support flexible functions adapted by service needs.

Proposal 3:	6GR UP protocol should aim to enable further shorter processing time than NR UP.

Proposal 4:	6GR UP protocol should aim to lean design than NR UP, e.g., duplicated functions among NR UP layers should be reduced/merged.

Proposal 5:	Keep the current functions in the SDAP layer as baseline. RAN2 may consider enhancements of SDAP if triggered by other WGs (e.g., SA2), or by data transfer of AI and sensing discussions.

Observation 1: NR UP protocol stack is inefficient in terms of header overhead and processing delay.

Observation 2: NR PDCP layer and RLC layer have some similar/duplicate functions, e.g., SN maintenance, status report, SDU discard, data volume calculation, etc.

Observation 3: Some features involve quite many PDCP notifications to RLC layers, which increase UE complexity and standard efforts.

Proposal 6:	RAN2 to study the feasibility and advantages for combining PDCP and RLC into one layer.

Observation 4: NR LCP-based resource allocation procedure is eMBB-centric and not friendly to XR, HRLLC and immersive communication.

Proposal 7: Study on adaptive resource allocation method to meet variable needs for different services, including services with high data rate, services with stringent latency requirement and services with composite requirement.

References

- RP-252912, Revised SID: Study on 6G Radio

- TR 22.870, Study on 6G Use Cases and Service Requirements

- TR 38.914, Study on 6G Scenarios and requirements