3GPP TSG-RAN WG2#131-bis	R2-2506939

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	10.4

Title:	Initial consideration on 6GR Mobility

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objective about mobility and related RRM for 6GR.

(6) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we would like to provide initial views on 6GR mobility aspects in terms of possible improvements/enhancements from NR.

Discussion

General mobility principles for 6GR

In 5G, idle mode mobility and connected mode mobility exist. In 6G, at least RRC_CONNECTED and RRC_IDLE are likely to continue to be used because they are quite basic UE states, so we should study both idle mode mobility and connected mode mobility.

Usually, different mobility mechanisms are applied for different RRC states. For example, in RRC_IDLE state or in RRC_INACTIVE state introduced in 5G NR, UE based mobility, e.g. cell reselection is used while network-controlled mobility, e.g. handover is supported in RRC_CONNECTED state. In addition, radio link failure and recovery are supported in RRC_CONNECTED state. In all RRC states, measurement is required to support mobility mechanisms.

Proposal 1: In 6GR, the following mobility mechanisms and related functions are supported:

- for RRC_CONNECTED state, if supported in 6GR: network-controlled mobility, radio link failure/recovery and measurement

- for RRC_IDEL/RRC_INACTIVE state, if supported in 6GR: UE-based mobility and measurement

Connected mode mobility

Supporting type of HO in 6GR

In 5G, RAN2 had been seeking for mobility with less interruption time and robustness, and have introduced many types of connected mode mobility, which are Conditional Handover (CHO), Dual Active Protocol Stack (DAPS) and L1/L2 Triggered Mobility (LTM) in addition to baseline L3 HO. According to SID [1], 6GR should minimize the adoption of multiple options for the same functionality, focusing on practical user experience and thus RAN2 should avoid specifying so many kinds of mobility mechanism in 6GR.

As a starting point, one of 5G NR mobility mechanisms can be a baseline for connected mode mobility in 6GR. On one hand, DAPS introduced in 5G NR has not been commercialized because of its complexity. The benefit of CHO can be earned with Conditional LTM, so CHO is no longer needed if CLTM is continued to be specified in 6GR. On the other hand, LTM is beneficial for reducing interruption time, so LTM should be considered as a baseline for 6G mobility.

Proposal 2: As a starting point, LTM should be considered as a baseline for connected mode mobility in 6GR.

Unified framework

During RAN 6G workshop, some companies proposed a unified framework and signaling structure for multiple mobility mechanisms. We think that it is reasonable to apply a unified design, which will at least support variable existing services/needs and provide better forward compatibility to address the future services/needs.

In 5G NR, multiple mechanisms are introduced for network-controlled cell level mobility, as shown in Table 1.

Table 1: Summary of NR mobility mechanisms

|  | RRC involved mobility |  |  |  | Mobility without RRC involved |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | basic HO | DAPS HO | RACH-less HO | CHO | LTM | CLTM |
| preparation | One cell is provided to the UE |  |  | Multiple cells are provided to the UE |  |  |
| Pre-execution | N/A |  |  |  | - TCI state (re)activation - Early TA acquisition - Early CSI acquisition and reporting |  |
| Execution | - Explicit RRC signalling triggered; - RACH-based; | - Explicit RRC signalling triggered; - RACH-based; - DL data with SgNB; | - Explicit RRC signalling triggered; - No RACH | - Evaluation of L3 event(s) - RACH-based | - Explicit MAC CE triggered; - RACH-based or no RACH | - Evaluation of L1 or L3 event(s) - RACH-based or no RACH |
| completion | RACH completion |  | RACH completion | Completion of the first UL transmission | Completion of RACH or the first UL transmission |  |

In Rel-19, LTM was extended to inter-CU mobility, but it is limited to Xn-based and intra-AMF. For mobility which is performed via another interface e.g., N2 in 5G, L3 baseline handover can be considered. Besides, conditional execution has benefits on robustness compared to network command triggered mechanism. So, the unified framework and signalling should take these into account.

As summarized in Table 1, if the unified framework is defined, necessary information for multiple candidates will be pre-configured to the UE and HO command or UE estimated condition is used to trigger the cell switch execution. The 6G study needs to discuss whether the HO command will be carried by the RRC signalling, lower layer signalling or both, and whether candidates’ configuration and HO command can be one or different RRC messages.

Proposal 3: A unified framework is designed for network-controlled mobility in RRC_CONNECTED state at least considering that:

- Necessary information for multiple candidates should be pre-configured to the UE

- HO command/UE estimated condition is used to trigger the cell switch execution

Inter-RAT mobility

In RAN#109, RAN plenary has approved to study whether any additional migration option(s) is needed [1]. This study will be started from March 2026, and RAN will make a decision in September 2026. It means RAN2 should focus on the standalone, MRSS and inter-RAT mobility between NR-6G unless RAN plenary requests the additional migration option(s).

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

From RAN2 perspective, it may be useful to study and identify initial 6G deployment scenarios. For example, it is relevant to inter-RAT mobility functions supported in Day1.

Regarding inter-RAT mobility, cell reselection/handover between E-UTRA and 5G NR, redirection to E-UTRA and handover with SRVCC operation to UTRA are supported in 5G NR. To support inter-RAT mobility, inter-RAT measurement is needed. In 5G NR, inter-RAT event B1/B2 is used for measurement reporting.

For inter-RAT cell reselection, the following use cases are supported in 5G NR:

- Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE

- Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE

In addition, when mobility from other RAT fails, RRC connection establishment procedure is initiated for failure recovery.

In our view, network-controlled mobility and UE-based mobility should be supported between 6GR and 5G NR at the initial phase of 6G deployment. Measurement and failure recovery are supported as well.

Proposal 4: Inter-RAT measurement, cell reselection, handover and HOF recovery between 6GR and 5G NR are supported. Details are further studied after some progress is made in CP study and intra-6GR mobility.

NTN-TN mobility

In 6G, based on the moderator’s summary for NTN [2], some companies supported seamless service continuity between TN and NTN. One of the use cases for NTN is that NTN cells cover the areas which are difficult to be covered by TN cells (e.g., islands, oceans) for achieving a ubiquitous connectivity. Considering the NTN deployment models, there is a possibility that TN and NTN may overlap or be adjacent. Even in such cases, to maximise the user experience, on-going service should be continued between TN and NTN cells with minimum service interruption. Therefore, the seamless mobility between TN and NTN in connected mode should be studied in 6G.

Furthermore, if NTN and TN are of the same generation, it would be possible to construct the network architecture considering ubiquitous connectivity. Therefore, constructing intra-6GR NTN-TN mobility is more likely to achieve seamless mobility than constructing inter-RAT NTN-TN mobility.

Considering the above, mobility between 6G TN and 6G NTN (i.e., intra-RAT) should be supported in 6G Day1.

Proposal 5: Intra-6GR mobility between TN and NTN is supported in 6G Day 1.

AI for mobility

5G NR based AI/ML mobility has been studied in Rel-19 based on the high priority use cases for RRM measurement prediction and measurement event prediction. According to the study results, when future measurement results and events are well predicted by AI algorithms, handover performance can be enhanced, especially in more challenging scenarios; and when using the AI/ML based predicted result to perform handover, the degradation of handover performance is acceptable, even if partial measurement has been skipped. In Rel-20, 5G NR will work out the specification to support RRM measurement prediction and measurement event prediction for UE sided models for the use cases which have been studied in 5G NR Rel-19, e.g., temporal domain prediction case A and case B, and frequency domain prediction for co-located cases for L3 Cell level predication. L3 beam level prediction has been introduced in the scope of 5G NR Rel-20 AI/ML for mobility WID, but only for network sided model, not for UE sided model.

In 6GR, the study can be based on the study results achieved in 5G NR, while expanding the scope of mobility procedure assisted by AI, e.g., AI based mobility in 6GR should not be limited to L3 based mobility. AI models can be deployed to enhance the handover initiated by UE, like CHO, or to enhance the L1/L2 procedure for LTM, including timing advance prediction and candidate cell selection. AI/ML technology may also be used to enhance the mobility with NTN, e.g., to predict the timing for UE to re-synchronize with network to a new satellite during satellite switch. Service based mobility may be introduced in 6GR, e.g., when a mission critical service is designed to exchange data with low latency, UE may be required to HO to another cell or band due to traffic load prediction.

Since 5G NR Rel-20 AI/ML for mobility WI will start in parallel with 6G SI, waiting the work results by 5G NR WI will heavily impact the progress of 6G study. It’s not necessary for 6G to repeat the study performed in 5G, so the scope of 6GR should not overlap with 5G NR, and new use cases or scenarios should be studied in 6GR.

Considering the time limitation for 6G study, it is not possible to cover all the potential AI applicable mobility procedure, the study scenarios or use cases needs to be defined first.

Proposal 6: Outcome of 5G NR AI mobility work can be taken as baseline, and RAN2 identifies potential 6GR AI applicable mobility scenarios not planned in 5G NR as starting point.

In addition, according to 6GR SID [1], coordinated discussion needs to be ensured with related design areas, where needed, e.g., the coordination between AI mobility and AI MIMO. Both MIMO and mobility may need to perform the beam level measurement, data logging, prediction and performance monitoring. A common solution for both features can be feasible. But considering the different intentions, specific configuration for different purposes may need to be defined.

The features with potential dependency with AI Mobility study scope should be identified first to decide which part should be included in AI mobility scope, then further study can be designed for supporting all the features related to AI mobility.

Proposal 7: RAN2 to identify 6G features having potential dependency with AI Mobility.

Conclusion

In this paper we presented some initial views on RAN2 study directions for 6GR mobility aspects and have come up with the following observation and proposals:

Proposal 1: In 6GR, the following mobility mechanisms and related functions are supported:

- for RRC_CONNECTED state, if supported in 6GR: network-controlled mobility, radio link failure/recovery and measurement

- for RRC_IDEL/RRC_INACTIVE state, if supported in 6GR: UE-based mobility and measurement

Proposal 2: As a starting point, LTM should be considered as a baseline for connected mode mobility in 6GR.

Proposal 3: A unified framework is designed for network-controlled mobility in RRC_CONNECTED state at least considering that:

- Necessary information for multiple candidates should be pre-configured to the UE

- HO command/UE estimated condition is used to trigger the cell switch execution

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

Proposal 4: Inter-RAT measurement, cell reselection, handover and HOF recovery between 6GR and 5G NR are supported. Details are further studied after some progress is made in CP study and intra-6GR mobility.

Proposal 5: Intra-6GR mobility between TN and NTN is supported in 6G Day 1.

Proposal 6: Outcome of 5G NR AI mobility work can be taken as baseline, and RAN2 identifies potential 6GR AI applicable mobility scenarios not planned in 5G NR as starting point.

Proposal 7: RAN2 to identify features having potential dependency with AI Mobility.

References

- RP-252912, Revised SID: Study on 6G Radio

- RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services

- RP-252147, Architecture/migration requirements for NTN