3GPP TSG-RAN WG2 Meeting #131bis	R2-2506940

Prague, Czech Republic, 13th –17th Oct., 2025

Agenda Item:	10.3.1

Source:	Huawei, HiSilicon

Title:	Discussion on 6GR user plane functions and architecture

Document for: Discussion and Decision

1	Introduction

In the WG SID for 6G [RP-251881], the 6G user plane has been listed as one of the discussion objectives for radio interface protocol architecture and procedures

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

With the ongoing evolution towards 6G, it is essential to begin a systematic analysis of the user plane (UP) architecture and protocols beyond what is currently standardized in 5G NR. The 5G NR UP has been designed with flexibility and efficiency to accommodate diverse service categories, including eMBB, URLLC, and mMTC. However, 6G is expected to impose new performance requirements such as higher spectrum efficiency, low latency, enhanced reliability, and native AI/ML integration. These emerging demands may necessitate modifications or extensions to the NR UP design.

In this paper, we use the 5G NR UP and NR L2 functions as the foundation for analysis and offer preliminary considerations for the 6G UP design, while highlighting potential areas for further study.

2	Discussion

2.1	6G KPI and new service requirements

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107 [RP-250810]. Regarding the key performance indicators and relevant key assumptions in the scenario, the objectives of this SID mainly include:

- Investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

Based on the above guideline on the minimum set of TPRs, the key performance indicators of 6G in TR 38.914 are agreed in RAN#108, and summarized in the following table. A comparison with the 5G KPI in TR 38.913 has also been given along-side. Compared with IMT-2020, new requirements such as Positioning, Composite requirement, Sensing, and AI have been introduced in IMT-2030.

The KPIs that are relevant for user plane discussion have been highlighted.

| TR 38.913 | TR 38.914 till RAN#109 |
| --- | --- |
| - Peak data rate. - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - User plane latency - Control plane latency - Connection density - Reliability - Mobility - Mobility interruption time - Bandwidth - Energy efficiency | - Peak data rate. - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - User plane latency - Control plane latency - Connection density - Reliability - Mobility - Mobility interruption time - Bandwidth - Energy efficiency |
| NA | - Positioning - Composite requirement - Sensing - AI |

Table1, KPIs adopted in 38.913 and 38.914 so far

Furthermore, the TSG SID has also laid out the objective for studying the requirement of 3GPP for improving the existing and new services and the TR 38.914 has also listed the new services under 6G consideration. Within these services, immersive communications and AI have been listed as two services in section 5.4.2 and 5.4.5, respectively, which are related to UP enhancements.

In this section, we discuss the 6G KPI requirement and service requirements and highlight their impacts to the design of 6G user plane architecture and protocol.

2.1.1	6G Key Performance Indicators

The 6G KPIs have been currently captured in the spec TR 38.914. In the following table, we give a comparison between the 5G and 6G KPIs specified in the 5G and 6G TPR.

|  | Peak Spectral Efficiency | 5th percentile User Data Rate | User Plane Latency | Reliability |
| --- | --- | --- | --- | --- |
| IMT-2020 | DL 30 bps/Hz UL 15 bps/Hz | DL 100Mbps UL 50Mbps | eMBB 4ms URLLC 1ms | URLLC 10^-5 |
| IMT-2030 | DL: 60bps/Hz UL: 30bps/Hz Both under ICNOTE1 | DL: 300Mbps UL: 50Mbps Both under IC | IC 4ms HRLLCNOTE2 1ms | HRLLC 10^-5 |
| Gap | up to 2x | up to 3x for DL | Same KPI | Same KPI |
| NOTE1: IC refers to Immersive Communications NOTE2: HRLLC refers to Hyper Reliable Low Latency Communications |  |  |  |  |

Table2, Comparison between the IMT-2020 and IMT-2030 user plane-related KPIs

In this section, we give an overview of the 6G KPIs and discuss on their potential impacts to the user plane design.

2.1.1.1	User experienced data rate and spectral efficiency

In the current TR 38.914, the user experience data rate and the relation with user spectral efficiency has been defined as follows:

5.1.3 5th percentile user data rate The 5th percentile user data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e., the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account. In case of one frequency band and one layer of transmission reception points (TRxP), the downlink 5th percentile user data rate could be derived from the 5th percentile user spectral efficiency through the following equation. Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the 5th percentile user data rate, Ruser is given by: Ruser = W × SEuser

Based on the above definition, the user experienced data rate refers to the service that Layer 2 provides for the upper Layer 3, as the rate of service data unit (SDU) ingress the Layer 3 and egress the Layer 2. It can also be observed from the Table 2 above that there is up to 3-fold increase for DL in the 5th percentile user experienced data rate. Although uplink user experience data rate remains at 50Mbps as before, but actually it represents a more stringent requirement since the uplink user experienced data rates in IMT 2020 and IMT 2030 are calculated using different setups.

The relationship between Layer 2 design and user experienced data rate/physical layer data rate is illustrated in the Figure1 below. The SDUs are provided to the Layer 2 according to the user experienced data rate and after Layer 2 processing, the MAC PDUs are delivered to the Layer 1 at data rate supported by the physical layer.

Figure1, The relationship between L2 user plane, and physical/user experienced data rate

Hence, the design objective of the Layer 2, in terms of the user experienced data rate, should be to ensure it is not a bottleneck of the user plane processing between the Layer 3 and Layer 1: Specifically, Layer 2 should be able to fully take advantage of the data rate provided by the Layer 1; and should be able to timely process the SDUs received by the Layer 3.

Observation1-1:	From the processing complexity point of view, the design target is to ensure that the Layer2 does not become a bottleneck between the user experienced data rate it provides to Layer 3 and the uplink data rate that physical layer provides.

2.1.1.2	Latency

The user plane latency requirement has been defined as follows in TR 38.914

5.1.7 User plane latency User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. [or battery efficient (e.g. inactive)]

While from the Table2 above, the required KPI figure is the same, but the type of services is different. The IMT-2020 KPI is targeted for eMBB and URLLC services; while the IMT-2030 KPI is targeted for IC and HRLLC services.

2.1.1.3	Reliability

The reliability requirement has been defined as follows in TR 38.914

5.1.10 Reliability Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

It can be observed that except for the different type of traffic (HRLLC for IMT-2030, and URLLC for IMT-2020), the reliability requirements are the same.

2.1.1.4	Composite requirements

In R18 XR, the following KPI has been defined in the TR 38.838

7.2 KPI 7.2.1 UE satisfaction A UE is declared as a satisfied UE if all the considered streams meet their own PER and PDB requirements, i.e., more than a certain percentage of packets are successfully transmitted within a given air interface PDB. Specifically, we have followings depending on the evaluation directions considered. - In DL-only evaluation, only DL streams are considered when identifying UE satisfaction. - In UL-only evaluation, only UL streams are considered when identifying UE satisfaction.

We believe the above UE satisfaction KPI is already a type of "composite requirement". Taking the single stream downlink traffic as example, it needs to support the traffic with a certain packet size (section 5.1.1.1 of 38.838 [4]), a certain packet arrival rate (section 5.1.1.2 of 38.838) with a certain packet delay budget (section 5.1.1.3 of 38.838) and a certain packet error rate (section 5.1.1.4 of 38.838). Then, the KPI is calculated as the ratio of satisfied UEs.

Within the latest TR 38.914, the following has been defined for composite requirements

### 5.1.16 Composite requirement The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1. Table 5.1.16-1: Composite requirement [Test environment]; Data rate (Mbit/s); Latency (ms); Packet success probability; Number of users (TRxP). Dense Urban - Immersive Communication; DL = 30 Mbps/UL=10 Mbps; DL=10 ms / UL=30 ms (or DL+UL total 40 ms); 99%; 6.

The above requirement means that, under the 6G design, the number of users supported by each TRxP should be at least 6 and for these UEs, all the requirements for data rate, latency, packet success probability should be satisfied simultaneously. Furthermore, as discussed in [RP-252131], there could be different levels of requirements, and UP design shall provide efficient mechanism to support different use cases. Based on the above, we make the following observation:

Observation1-2:	The new composite requirement stipulates the number of users to be supported per TRxP while simultaneously satisfying the data rate, latency, and packet success probability.

2.1.2	6G new services

During RAN1#122, the following has been agreed for the traffic models to be used for 6G evaluations

Agreement - Study whether to introduce the following traffic models for 6G evaluations considering, e.g., - FTP-3 variant with packet delay budget requirement - Details FFS - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget) - Details FFS - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc. - Details FFS

For the usage scenarios for IMT-2030, the following Figure2 is used to explain the extensions on top of those from IMT-2020: Immersive communications, massive communications, and HRLLC are extended based on the eMBB, eMTC, and URLLC scenarios from IMT-2020. In addition, the new scenarios are added, including integrated sensing and communications, ubiquitous connectivity and integrated AI and communications.

Figure2, Usage scenarios for IMT-2030

In this section, we discuss the new services relevant for user plane in 6G and the potential impacts from these new services to 6G user plane design.

2.1.2.1	AI traffic and token communications

The AI services empowered by large AI/ML models such as large language models (LLM), vision language models (VLM), etc., are currently applied in various scenarios, such as, AI agent, chatbot, personal assistant, autonomous driving and so on [RP-252132]. For AI/ML models such as LLM, instead of transmitting raw input bit (e.g., text, images), the AI client first transforms the raw data into the machine-interpretable language for the AI server, namely tokens, for efficient processing.

A token generated and processed by the AI models can be regarded as a compact data carrying the context information. The tokenizer segments raw data sequence and then converts each input raw data segment from various modalities (e.g., text, image, audio, etc.) to tokens at the transmitter according to a specific algorithm. One example on how the token traffics can be transported by 3GPP network is given in Figure3. The tokens generated from raw data in the UE side can be transmitted directly to the receiver. Conversely, a de-tokenizer converts the received tokens at the UE back to the raw data.

Figure3, how tokens could be generated, transported by 3GPP network and received

With the booming AI services, the token communications traffic is becoming more and more significant in the wireless and wireline network. For example, by 2027, it is estimated that the daily token consumption of DOUBAO (a popular mobile AI application in China) will exceed 100 trillion per day [RP-252132], which brings operators tremendous business opportunities as well as challenges for accommodating such traffic.

Observation1-3:	It is projected that by 2027 a typical token-based application will process over 100 trillion tokens per day, making tokens traffic significant in the wireless network.

Like the other traffics transported in the network, the token communications traffic also has its own traffic characteristics, which can be summarized as follows:

- Error tolerance

One notable characteristic of token communications is error tolerance. The Figure4 below shows an example of image reconstruction with the first row of images without token prediction and the second row of images with token prediction. It can be observed that with increasing token error rate, the row of images with token prediction shows much better performances in terms of image reconstruction quality. That’s the reason why this token prediction functionality is widely used in the current AI services.

Figure4, Image reconstruction with/without token prediction

- Different importance for different tokens

Furthermore, tokens serving for different modalities have different importance. From the Figure5 below, we observe that the loss of text tokens has more significant impacts compared to the loss of image tokens. The reason is that the erroneous image tokens can be predicted and reconstructed more easily compared to the text tokens, while the later are more error-sensitive and the accuracy ratio decreases significantly once token error rate is larger than 1%.

Figure5, Accuracy ratios with different error rate for image and text token

In addition to the above example from multiple modalities, the tokens from the same modality may also exhibit different importance. As shown in the following Figure6, the loss of tokens in the latter part of the series of token transmission leads to less L2 error. Hence, token in the tail becomes less important: the error in front tokens lead to more significant degradation in reconstruction quality than that of the error in subsequent tokens.

Figure6, L2 error with different positions of lost tokens

Inspired by such characteristic, we believe that the transmission efficiency can be improved if tokens with different importance can be transmitted according to their corresponding priorities.

Observation1-4:	Token-based communications is error tolerant and different tokens have different importance depending on their modality and the location of the token in the traffic.

2.1.2.2	Immersive Communication

In 5G, the traffic characteristics of XR services in TR 38.838 [4] have been studied as a specific type of immersive communications (IC) service since R17, and the subsequent normative work has been completed in R18 and R19. With the advent of AI era, the definition of immersive services has been expanded to further include the mobile AI services.

The typical use cases of immersive services have been specified in SA1 specification TR 22.870 [7], as shown below:

- Higher resolution XR cloud gaming: Immersive gaming with 8K resolution display defined in Section 9.1

- Intelligent video conferencing on AI phones: Enhanced meeting experience defined in Section 8.3

- Contextual understanding on AI wearable: Awareness of real-time environment, such as real-time translation, navigation, blind assistance, as defined in Section 8.3

- Healthcare for aging population on embodied AI robots: Healthcare for aging population has been defined in Section 9.9

A unified service flow for IC that can be concluded based on the above-mentioned services is shown in the Figure7 below, as in [RP-252131]:

Figure7, Service flow of immersive communications services

- Step 1: IC device collects necessary data unit for AI inference or pose control information for split rendering in XR cloud.

- Step 2: IC device uploads the collected data unit or the pose control information to the cloud server via 6G network.

- Step 3: Cloud server performs AI inference based on received data unit or split rendering based on the received pose control information.

- Step 4: Cloud server sends the inference/rendered data to the IC device via 6G network.

The IC services bring new requirements on the 6GR system with ultra-high data rate within stringent latency requirement, which consequently also imposes challenges on the system capacity of the 6GR system. We define the End-to-End (E2E) latency for the immersive interactive services in Figure7 as the time from the UE uploading the collected data to receiving the inference results (TUE+PDBair+TTrans+Tinf). The End-to-End (E2E) latency is important for guaranteeing smooth user experience or even the safety. For example, it is important for an AI robot when executing actions such as obstacle avoidance, especially in high mobility scenario.

It is worth noting that QoE provided by such interactive services highly relies on the guaranteed end-to-end (E2E) latency, which usually varies even for the same application. For example, at different stages when the AI robot is executing a task like “wash the dirty mug”, the E2E latency requirement varies. The E2E latency requirement of the sub-task routing planning to the sink can be more relaxed than that of mug washing. In the latter sub-task, the environment is more dynamic (e.g., running water, grabbing) and the AI robot needs to react to the environmental changes quickly, otherwise the sub-task may fail.

The varied E2E latency requirement requires the system to allocate delay budget flexibly. To be more specific, if the network schedules the data with the fixed E2E latency, the delay budget over air interface must be always configured according to the most demanding latency requirement. Consequently, the performance of the system, e.g., capacity, will decline as data transmission of other traffics or UEs is deprioritized.

Observation1-5:	The E2E latency is critical for ensuring good user experience in IC services; However the E2E latency requirement can be time-varying even within a single IC application.

2.1.3	Summary for 6G new KPI and services

The discussion above on the new KPIs and new services presents the motivations to design a new release of Layer 2 user plane in 6G for satisfying the new requirements without worrying about the backward-compatibility issues. Hence, we propose the following:

Proposal1:	RAN2 user plane design shall support the targeted 6G KPI requirements (e.g., user experienced data rate, latency, reliability, composite requirement) and new service requirements (e.g., AI traffic and immersive communications).

2.2	Analysis of overall user plane functions

During the evolution of LTE and NR, user plane has been consistently enhanced. Most of the enhancements were introduced to satisfy higher requirements of different KPIs and traffic characteristics from different services. A simple summary is provided in Table3 below for the discussion in NR.

|  | 5th User experienced data rate | Latency | Reliability | Traffic characteristics for new services | Processing complexity reduction |
| --- | --- | --- | --- | --- | --- |
| R15 |  |  | PDCP duplication |  | - Removal of RLC concatenation; - Reordering functionality from RLC to PDCP |
| R16 | PDCP EHC | Intra-UE prioritization | Enhance PDCP duplication to up to 4 legs | PDCP EHC |  |
| R17 | PDCP UDC | MO-SDT |  |  |  |
| R18 | Refined BSR table | - PDCP SN gap report; - Single entry delay status report; - MT-SDT |  | - PDU Set-based discard - PSI-based discard |  |
| R19 | RLC unnecessary retransmission avoidance | - Remaining time based RLC retransmission and polling; - Multiple entry delay status report; - Delay-aware LCP |  |  |  |

Table3, Features introduced in each release of NR for different objectives

Observation2-1:	During evolution of NR, Layer 2 enhancements were primarily focused on how to support more stringent KPI requirements (including latency, reliability, etc.) and to enable new services (including URLLC, IIoT and XR, etc.) as add-on features.

In NR, due to such an add-on design, some features have multiple options, or some functions may have high complexity to support different features. This may lead to difficulty in real commercialization. For the study of 6G, RAN2 should consider the Day-1 design to support the below KPI and new service requirements, with potential complexity reduction where applicable:

- 6G has new KPIs, as analyzed in sections 2.1.1, To provide the higher rate with the stringent latency requirement of 4ms, RAN2 may need to investigate L2 functions enhancements from perspective of latency reduction and user experienced spectral efficiency improvement. Some detailed analyses on 6G are provided in Section 2.3.2 and 2.3.3 respectively.

- 6G new services also have new traffic characteristics, as analyzed in section 2.1.2. Analysis on how latency reduction can be achieved for token communications has been given in section 2.3.2.

- Besides, there were also some enhancements introduced to reduce the processing complexity during NR evolution, e.g., RLC concatenation was removed to enable pre-processing. RAN2 can also take implementation complexity into account when designing 6G L2 protocols. More details about these aspects on 6G can also be found in section 2.3.1.

Based on the discussion above, we propose the following:

Proposal2:	RAN2 should study and analyze which NR functionalities require enhancement or new design to meet 6G KPI and service requirement and reduce processing complexity.

2.3	Aspects to be studied for user plane

Within the WG SID for 6G [RP-251881], the objective for user plane discussion is quite generic and covered by a single bullet. In this section, we intend to analyze in details the potential directions that we can work on for the 6G user plane.

2.3.1	User plane function simplification

During the study phase discussions of L2 protocol for NR, it was studied whether L2 functionalities in LTE can be removed, merged from different layers, and reformulated/replaced across different layers with a focus on the reordering, retransmission, concatenation and segmentation. Some user plane functions in NR have been simplified and restructured compared to LTE, as can be seen in the analysis in section 2.2.

For 6G, we can consider the following aspects for how the Layer 2 user plane can be simplified:

2.3.1.1	SN assignments

In the legacy NR, SN assignments are performed under both RLC and PDCP layer in L2 protocol. For the AM DRB, when the complete RLC SDU is transmitted, the SN assigned by the RLC will be used for RLC status report/ARQ retransmission. For both UM and AM DRB, when the segmented RLC SDU is transmitted, the SN assigned by the RLC will be used for reassembly. Due to the removal of concatenation in the RLC layer, there is a one-to-one correspondence between the RLC and PDCP SDU.

This may cause some inefficiency to the system. On one hand, the transmitting/receiving device needs to manage the packets separately at PDCP and RLC, and two separate Tx/Rx windows are needed at PDCP and RLC respectively, which may cause some implementation complexity. On the other hand, having both PDCP SN and RLC SN will lead to more Uu interface overhead.

One straightforward solution for this is to have shared SN between PDCP and RLC. In the standalone architecture, the RLC SN and PDCP SN are the same for one RLC PDU for all these cases, so only one SN transmitted over the Uu interface is feasible. However, if DC is supported, how to work with only one SN for split bearer need further study.

Observation3-1:	Sharing single SN between RLC and PDCP layers and single Tx/Rx window can reduce the implementation complexity. Further study would be needed for its feasibility in DC.

2.3.1.2	Retransmissions

In NR, the retransmissions mechanism is supported in RLC layer and PDCP layer for selective retransmission: These two retransmission mechanisms serve different purposes.

- For ARQ, retransmissions are performed at the level of RLC SDU or RLC SDU segments such that the residual error of the HARQ retransmission can be eliminated for RLC AM.

- For PDCP retransmission, only PDCP SDU retransmission is supported and is performed when PDCP and lower layer is re-established, during handover, or PDCP data recovery.

Currently, we don't see justification to remove either one of the retransmission schemes. One possible study direction is whether to have retransmission in one layer to cover all the purposes. This may save one SN overhead and reduce the complexity of UP protocol. However, additional aspects also need to be considered, such as the placement of the segmentation function.

Observation3-2:	Unified mechanism for RLC and PDCP retransmission may simplify the L2 protocol, while it needs to be further studied where segmentation is placed in the protocol stack.

Based on the above discussions, we proposed that:

Proposal3:	RAN2 to study potential directions for L2 protocol simplification/reformulation from the perspective of, e.g., shared SN between RLC and PDCP, unified retransmission mechanism in L2, also considering RANP decision on DC.

2.3.2	Latency reduction

Latency is another important requirement for the new 6G KPI. Next, we discuss several potential contributing factors in the current Layer 2.

2.3.2.1	Scheduling request latency

In NR, after Layer 2 of UE obtains data from upper layer, UE initiates the UL resource request procedure by sending SR and BSR/DSR to obtain a dynamic grant (DG). The acquisition delay of DG is relatively large. To reduce the latency caused by this SR-BSR-UL grant cycle, both NR and LTE support configured grant (CG), at the cost of higher resource consumption. In 6G, we need to consider new methods for acquiring UL resources that can simultaneously achieve low latency and low resource consumption.

2.3.2.2	Delay caused by waiting for complete TB delivery

Some services can tolerate errors at the APP layer of receiver, meaning that the APP layer of receiver does not need to correctly receive all data to recover the original information, for example, PDU Sets encoded with AL-FEC for immersive communications and token communication as described in Section 2.1.2.1.

However, the technical benefits of AL-FEC, AI traffic or token communication cannot be realized in practice, because even though the APP layer of receiver can tolerate errors, AS layer (i.e., PHY/MAC layer) of receiver strives for 100% reliability for delivery and decoding of one TB. Specifically, the PHY layer of receiver supports CB level error handling. However, only TB-level delivery to the MAC layer is supported, where all CBs in the TB must be successfully received before they can be delivered to the MAC layer for decoding.

Figure8, Example of current frame delivery

For 6G, due to the error tolerant characteristics of the applications, APP layer may not require all the data delivered by the lower layer to be fully correct. Hence, partially correctly received data within a TB can still be delivered to the APP with the possibility that it may be correctly recovered. An example is given in Figure8 where an application layer data unit with 20% error tolerance is transmitted via 2 TBs, where TB1 succeeds and the decoding of some CBs in TB2 fails. If the successfully decoded CBs can be delivered to the application, it may already be enough for the application to recover the data, since 90% of the data is already correctly received. With the error tolerance capability from AI codec and token, the erroneous AI data received at the APP layer also contributes to the AI prediction and hence the better recovering capability.

Observation4-1:	The current approach of decoding and delivering of a transport block only when it is fully received is unsuitable for error tolerant services and cause delay at the receiver.

2.3.2.3	Delay caused by MAC HARQ and ARQ retransmission

In NR, HARQ retransmission and ARQ retransmission are specified under MAC and RLC layers respectively, and these two mechanisms are decoupled. To prevent simultaneous HARQ retransmissions and ARQ retransmissions for a single RLC SDU, the triggering of RLC status reports (e.g., triggered by t-Reassembly expiry) occurs after the completion of HARQ reordering, and TX performs ARQ retransmissions based on the RLC status report. This may mean that ARQ retransmissions only occur after t-Reassembly expiry, which could be long after HARQ retransmissions have been completed. Hence, this could potentially lead to a large delay in ARQ retransmissions (approximately 40ms for typical 80ms t-Reassembly configuration).

In 6G, to reduce this delay, we may need to consider how to avoid the case when the RLC ARQ mechanism needs to wait for the expiry of the t-Reassembly to trigger retransmission.

Observation4-2:	Currently, ARQ transmissions might be performed long after HARQ reordering is completed, which causes delay at receiver side.

In summary of the discussions above, RAN2 should study potential directions for latency reduction though the following proposal.

Proposal4:	RAN2 to study the potential direction for latency reduction from the perspective of , e.g., quick scheduling request mechanism, enhancement for error tolerant services, and quick retransmissions.

2.3.3	Spectral efficiency

In RAN#108, the KPIs for 6G are agreed as in TR 38.914 [RP-251873], where spectral efficiency is one of the KPIs as discussed in section 2.1.1.1. Higher spectral efficiency means higher user experienced data rate, i.e., the number of bits contained in the SDUs delivered to Layer 2/3.

If we look at the data flow through the User Plane Layer 2 protocols for NR shown below in Figure9, we can observe that current NR user plane stack operates on per IP-packet basis. On one hand, the headers are added at each of L2 layers including SDAP, PDCP, RLC, and MAC. On the other hand, security-algorithm invocations, including the Integrity protection of every IP packet is performed for each packet at PDCP layer.

Figure9, Packet construction in Layer 2

With respect to L2 header overhead, taking the voice packet and IPv4 ACK as examples, the L2 header overhead ratio can be up to 19.4% for a voice packet with size of 25 bytes, and the L2 header overhead ratio can be up to 13% for an IPv4 ACK with size of 40 bytes. Hence, for these types of traffics, the L2 header overhead constitutes a significant amount of air interface resources.

The current L2 header overhead can be summarized as below:

| Layer | Overhead |
| --- | --- |
| SDAP | 0 bytes, or 1 byte carrying QFI |
| PDCP | 2 or 3 bytes with 12 bits or 18 bits SN |
| RLC | Up to 5 bytes for different transmission modes with segment or not |
| MAC | 1 byte to 5 bytes containing mainly the F field, L filed and LCID (eLCID) field |

Table4, Summary of the header overhead in layer2

It can be observed that the SN assignment for each packet is currently performed both by PDCP and RLC layer. Applying both PDCP SN and RLC SN for a packet at the same time will lead to more Uu interface overhead. In addition, for small packets, e.g., IP packets, assigning PDCP SN and RLC SN regardless of the size of the IP-packet can further reduce spectral efficiency.

Furthermore, the integrity protection for DRB has been introduced in NR. Specifically, the MAC-I field with 32 bits length is present for DRB configured with integrity protection. There is no doubt that integrity protection for data can decrease the chances of inter-play attack, but introducing MAC-I field with 32 bits length for each packet will consume more resources, especially for the case where the packet size is small. In this case techniques such as PDCP concatenation can help to reduce the overhead.

As a result, to improve the spectral efficiency, RAN2 should study aspects related to Layer 2 overhead reduction, e.g., header size reduction and PDCP concatenation.

Proposal5:	RAN2 to study the potential directions for increased spectral efficiency from the perspective of, e.g., L2 header size reduction, PDCP concatenation.

2.3.4	Reduction of processing complexity

Processing complexity is another issue that needs to be considered for the 6G user plane design, which has already been considered as one important factor in the R15 discussion. As has been reviewed in section 2.2, in the R15 the functionality of RLC concatenation has been removed from the RLC layer, with the main intention to allow for pre-processing in the RLC layer and PDCP layer: when SDU is received in the PDCP layer, the SDU can be processed straight down to the RLC PDU, without waiting for the advent of UL grant. This can reduce the processing complexity at the time of MAC PDU assembly when the UL grant arrives.

From our point of view, the following aspects can be investigated in the 6G L2 user plane study under the direction of processing complexity reduction

- PDCP concatenation, including integrity protection and ciphering, may need to occupy intensive computing resources for the UE under both UL and DL. The processing complexity can be further reduced if multiple PDCP SDUs are concatenated and then processed by a single ciphering/ integrity protection procedure.

- PDCP/RLC transmit and reception operation: Currently, both PDCP and RLC receivers need to maintain some state variables, timers and windows. However, the reordering and reassembly operations could have been performed under a single receiving window, as in LTE RLC. On the transmitter side, for each PDCP SDU received from upper layer, it needs to be stored twice in the memory of PDCP and RLC, which occupies additional memory and adds to the processing complexity.

- MAC PDU format impacts the real-time processing complexity because it is generated only at the time of UL grant arrival and cannot be pre-generated based on the current NR spec. For the current MAC PDU format, the L field can be dynamic, which is dependent on the LCP procedure for the last MAC subPDU for a certain logical channel and whether it is variable-sized MAC CE. For fixed-sized MAC CE, the L field is not carried. Because the length of the fixed-size MAC CE could only be deduced by the LCID corresponding to the MAC CE, this introduces more processing complexity.

Based on the discussions above, we propose the following

Proposal6:	RAN2 to study the potential directions of processing complexity reduction from the perspective of e.g., PDCP concatenation, PDCP/RLC Tx/Rx operation, and MAC PDU formats.

2.4	QoS

2.4.1	Dynamic QoS adaptation

User experience as a KPI itself is not a new concept and lots of efforts have been made before, like QoE, first introduced in Release-15 LTE. In Release-17 NR, QoE metrics are transmitted to core network and are mainly used by OAM. Later in Release-18, RAN visible QoE was introduced for RAN scheduling improvement. Although QoE is standard ready, the main obstacle for wide commercialization is the dependence on the ecosystem: the whole QoE feature relies on 3rd party to provide the experience data.

SA2 FS_6G_ARC had some related discussion on achieving an independence from 3rd party and closed-loop support within 3GPP. With the advancement of AI in UE, UE device can perform service awareness, infer the QoE, and deduce QoS demand based on real-time channel status, environment information, etc., then indicate the QoS demand to the network. According to the latest WT#1.2 [S2-2508030] of FS_6G_ARC, dynamic QoS has been listed as one of the potential directions for QoS control mechanism

- Whether and how to enhance QoS control mechanisms to - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network. - to improve QoS to applications with dynamic requirements.

The existing UE/NW triggered QoS adjustment mechanism is based on NAS signaling. The NAS-based procedure needs to pass through lots of network functions, and takes long time, normally 200ms~.

Facing with new 6G use cases, like mobile AI or immersive communications, the traffic request is naturally varying burst by burst, the QoS requirements may change frequently. Therefore, efficient QoS adjustment mechanism is needed in 6G. This efficient QoS adjustment is especially important if the QoS adjustment demand is small, like a small PDB change from 20ms to 18ms, which can be satisfied via RAN scheduling rather than relying on the CN mechanism.

Observation7-1:	The current QoS adaption framework based on NAS signaling is insufficient for latency sensitive services.

2.4.2	E2E latency

In addition, the QoS framework has to consider the deterministic requirements from the IC services, as discussed in section 2.1.2.2. A smooth user experience relies on whether the end-to-end (E2E) latency requirement can be fulfilled or not. In 5G, 3GPP has supported the N3 delay measurement based on the time stamp contained in the GTP-U header in the DL, and the delay status report (DSR) in the UL, which help to guarantee the delay requirements of packets.

Dynamic QoS framework combined with the N3 delay measurement method should guarantee the deterministic delay requirement for the downlink streams. For example, as shown in Figure10, during the E2E coordination of the latency between the UE’s APP and the remote server, the latency within the 3GPP network could be dynamically adjusted: when the N3 latency is long, the Uu latency should be adjusted so that it still meets the total latency requirements.

Figure10, Delay budget collaboration among different regions

Therefore, the 6G protocol stack should be designed to schedule the data with flexible E2E latency requirement to improve the system capacity while maintaining the QoE of the users, for example by jointly adjusting the delay budgets between different interfaces, e.g., Uu and N3. To achieve this, 6G protocol stack should support more timely latency measurement and QoS coordination between different interfaces in order to accommodate the varying latency requirement of the IC data.

Observation7-2:	The QoS framework needs to consider the number of users in the composite requirement while guaranteeing the E2E QoS requirements.

Proposal7:	RAN2 to study enhancements on QoS mechanism, e.g., dynamic QoS adaptation, enhancements for E2E latency requirements, in coordination with SA2 progress.

3	Conclusion

6G KPI and new services

Observation1-1:	From the processing complexity point of view, the design target is to ensure that the Layer2 does not become a bottleneck between the user experienced data rate it provides to Layer 3 and the uplink data rate that physical layer provides.

Observation1-2:	The new composite requirement stipulates the number of users to be supported per TRxP while simultaneously satisfying the data rate, latency, and packet success probability.

Observation1-3:	It is projected that, by 2027, a typical token-based application will process over 100 trillion tokens per day, making tokens traffic significant in the wireless network.

Observation1-4:	Token-based communications is error tolerant and different tokens have different importance depending on their modalities and the location of the token in the traffic.

Observation1-5:	The E2E latency is critical for ensuring good user experience in IC services; However, the E2E latency requirement can be time-varying even within a single IC application.

Proposal1:	RAN2 user plane design shall support the targeted 6G KPI requirements (e.g., user experienced data rate, latency, reliability, composite requirement) and new service requirements (e.g., AI traffic and immersive communications).

Analysis of overall user plane functions

Observation2-1:	During evolution of NR, Layer 2 enhancements were primarily focused on how to support more stringent KPI requirements (including latency, reliability, etc.) and to enable new services (including URLLC, IIoT and XR, etc.) as add-on features.

Proposal2:	RAN2 should study and analyze which NR functionalities require enhancement or new design to meet 6G KPI and service requirement and reduce processing complexity.

Discussion directions for 6G user plane

Observation3-1:	Sharing single SN between RLC and PDCP layers and single Tx/Rx window can reduce the implementation complexity. Further study would be needed for its feasibility in DC.

Observation3-2:	Unified mechanism for RLC and PDCP retransmission may simplify the L2 protocol, while it needs to be further studied where segmentation is placed in the protocol stack.

Proposal3:	RAN2 to study potential directions for L2 protocol simplification/reformulation from the perspective of, e.g., shared SN between RLC and PDCP, unified retransmission mechanism in L2, also considering RANP decision on DC.

Observation4-1:	The current approach of decoding and delivering of a transport block only when it is fully received is unsuitable for error tolerant services and cause delay at the receiver.

Observation4-2:	Currently, ARQ transmissions might be performed long after HARQ reordering is completed, which causes delay at receiver side.

Proposal4:	RAN2 to study the potential direction for latency reduction from the perspective of , e.g., quick scheduling request mechanism, enhancement for error tolerance service, and quick  retransmission.

Proposal5:	RAN2 to study the potential directions for increased spectral efficiency from the perspective of, e.g., L2 header size reduction, PDCP concatenation.

Proposal6:	RAN2 to study the potential directions of processing complexity reduction from the perspective of e.g., PDCP concatenation, PDCP/RLC TxRx operation, and MAC PDU formats.

Observation7-1:	The current QoS adaption framework based on NAS signaling is insufficient for latency sensitive services.

Observation7-2:	The QoS framework needs to consider the number of users in the composite requirement while guaranteeing the E2E QoS requirements.

Proposal7:	RAN2 to study enhancements on QoS mechanism, e.g., dynamic QoS adaptation, enhancements for E2E latency requirements, in coordination with SA2 progress.

4	References

- RP-252912, Study on 6G Radio, 3GPP RAN#109, September, 2025

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP RAN#107, March, 2025.

- TR 38.914 v0.2.0, Study on 6G Scenarios and Requirements, 3GPP

- TR 38.838, Study on XR (Extended Reality) evaluations for NR

- RP-252132, Consideration on supporting new AI services for 6G, Huawei, HiSilicon, CAICT, CMCC, China Unicom

- RP-252131, Consideration on supporting immersive communication services for 6G, Huawei, HiSilicon

- TR 22.870, Study on 6G Use Cases and Service Requirements

- S2-2508030, [WT#1.2.x, QoS] QoS Framework, Ericsson, Xiaomi, OPPO, Nokia, InterDigital, Google