3GPP TSG-RAN WG2 Meeting #131bis	                                                                              R2-2506941

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Discussion on remaining MAC issues

Agenda Item:    8.12.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining MAC issues on NR MIMO [1]. More specially, the following issues are covered,

- PUCCH notification upon the BWP configured with CG PUSCH is deactivated

- UL multi-panel transmission with sDCI mTRP two-TA under SDM mode

- Unnecessary power consumption for mode-A PDCCH monitoring

Our proposals are summarized in Section 3. The conclusion of this paper is given in Section 4.

Discussion

PUCCH notification upon the BWP configured with CG PUSCH is deactivated [Issue-2]

Mode-A

For UE initiated beam reporting with mode-A, there is a case when UE triggers the CSI reporting, the BWP that configures the PUCCH resource for the first channel is not active.

Generally, there are two candidate options to solve this problem,

Option 1 – UE performs the BWP switch to the BWP configured with the PUCCH resource for the first channel;

Option 2 – UE gives up the CSI reporting due to failure of sending the notification to the NW;

From our view, it is possible to have this case like TAT expiry case. We do not make much optimization on the TAT expiry case [1],

For both mode-A and mode-B UEI beam report, - When UEIBR is initiated but TAT expires, UE does not initiate RACH. - If TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

That means, if the serving cell providing the PUCCH resource is associated with an expired TAT, the PUCCH resource for the first channel will be cleared and thus the NW will not schedule the second PUSCH for the mode-A CSI reporting. If the NW cares about the CSI information initiated by UE, it can trigger UE to re-sync to the NW and re-configure UE with the PUCCH resource.

When considering the BWP case, we also do not see much urgent to optimize this case, i.e., it is also under NW control. If the NW really cares about the CSI report triggered by UE, it can ensure the active BWP to be the one configured with the PUCCH resource.

Proposal 1 [Issue-2]: For mode-A UEIBR, it’s up to NW implementation to ensure the BWP configured with the PUCCH resource to be activated if it cares about the CSI report triggered by UE. No MAC spec impact.

Mode-B

For mode-B UE initiated beam reporting, it sees more complicated than mode-A because the PUCCH for the notification to the NW and the CG PUSCH for piggybacking the UCI can be configured within different BWPs/serving cells.

Some companies thought that we can follow the similar handling with DRX. In DRX, UE will check whether CG PUSCH is in DRX active time upon sending the PUCCH. This is because the UE cannot predict whether CG PUSCH is under active time in early due to the unpredicted UL data which may trigger SR.

- UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time, and if the PUCCH has been transmitted, UE does not transmit the PUSCH if type-1 CG occasion is outside DRX Active time. Inform RAN1 about this conclusion.

However, the difference is the BWP switch can be under NW control. Upon triggering the UEIBR, if the PUCCH is in active BWP and has been transmitted by UE, then the NW is aware of this mode-B UEIBR notification and can further activate the UL BWP configured with CG PUSCH in early or upon arrival of the CG PUSCH.

Proposal 2a [Issue-2]: For mode-B UEIBR, it’s up to NW implementation to ensure the BWP configured with CG PUSCH is active in early or upon arrival of the CG PUSCH if it has received the PUCCH notification for UEIBR. No MAC spec impact.

Another case is the PUCCH is not in active BWP upon triggering the UEIBR. Similar as the mode-A UEIBR, if the NW really cares about the CSI report triggered by UE, it can ensure the active BWP to be the one configured with the PUCCH resource.

Proposal 2b [Issue-2]: For mode-B UEIBR, it’s up to NW implementation to ensure the BWP configured with the PUCCH resource to be activated if it cares about the CSI report triggered by UE. No MAC spec impact.

UL multi-panel transmission with sDCI mTRP two-TA under SDM mode [Issue-5]

One controversial issue in the post email discussion is related to the sDCI mTRP two-TA with UL multi-panel transmission under SDM mode.

In UL multi-panel transmission with SDM mode, some of the MIMO layers for a TB are transmitted over the DCI indicated UL TCI state_1 and the others are transmitted over the DCI indicated UL TCI state_2. The gNB needs to receive all layers to decode the TB successfully. Therefore, if the TAT associated a certain one TCI state is expired, for the CG resource which is configured to be applied for both TCI states, how to handle this scenario needs further clarification.

One way is to clarify UE will not transmit the CG PUSCH over any indicated TCI state under SDM mode, if one of the indicated TCI states is associated with the expired TAT. The benefit of this way is the UE can reuse these CG resources upon re-sync to the NW. Another way is to follow the similar handling as legacy, i.e., clear the CG resource if at least one TCI state is associated with expired TAT.

From our view, we want to follow legacy behavior. If the NW triggers UE to perform UL re-sync to the expired TCIs, it can re-configure UE with the CG resource applied for these TCI states under SDM mode. Therefore, it is suggested that,

Proposal 3 [Issue-5]: If UE is configured with sDCI mTRP two TA and UL multi-panel transmission with SDM mode, UE clears the CG resource if at least one TCI state indicated by the DCI for the CG resource is associated with the expired TAT.

Unnecessary power consumption for mode-A PDCCH monitoring [Issue-10]

During the post email discussion, one company pointed out that there is a case where the UE may monitor PDCCH scheduling in vain for mode-A UCI reporting due to the fact that the gNB may fail to receive the PUCCH notification, which causes the huge power consumption by the UE.

There are two candidate solutions on the table:

Option 1: until the next PUCCH resource for the UEIRI;

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until reception of a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH; or - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until the next PUCCH resource for transmitting UE Initiated Report Indication in case a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is not received.

Option 2: Introduce a maximum PDCCH monitoring time period;

RRC controls DRX operation by configuring the following parameters: - modeAPDCCHschedulingwindow: the maximum duration until a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is received; When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until reception of a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH; or - during the period of modeAPDCCHschedulingwindow in case a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is not received.

From our view, the NW configures the periodic PUCCH resource for notification to the NW for mode-A PUSCH scheduling. Based on the current MAC spec definition,

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]).

If UE does not receive the PDCCH scheduling until the next PUCCH resource arrival, UE will send the notification for mode-A CSI reporting again, and then keep the active time on until receiving the PDCCH scheduling. That means the current MAC spec is aligned with option 1. Our understanding is it is a corner case that the gNB misses the notification from UE or UE misses the PDCCH scheduling for UCI reporting. So we prefer to keep the current spec wording on this topic.

Proposal 4 [Issue-10]: For mode-A UEIBR, keep the current wording in the MAC spec that UE considers the DRX as active time while a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

PUCCH notification upon the BWP configured with CG PUSCH is deactivated [Issue-2]

Proposal 1 [Issue-2]: For mode-A UEIBR, it’s up to NW implementation to ensure the BWP configured with the PUCCH resource to be activated if it cares about the CSI report triggered by UE. No MAC spec impact.

Proposal 2a [Issue-2]: For mode-B UEIBR, it’s up to NW implementation to ensure the BWP configured with CG PUSCH is active in early or upon arrival of the CG PUSCH if it has received the PUCCH notification for UEIBR. No MAC spec impact.

Proposal 2b [Issue-2]: For mode-B UEIBR, it’s up to NW implementation to ensure the BWP configured with the PUCCH resource to be activated if it cares about the CSI report triggered by UE. No MAC spec impact.

UL multi-panel transmission with sDCI mTRP two-TA under SDM mode [Issue-5]

Proposal 3 [Issue-5]: If UE is configured with sDCI mTRP two TA and UL multi-panel transmission with SDM mode, UE clears the CG resource if at least one TCI state indicated by the DCI for the CG resource is associated with the expired TAT.

Unnecessary power consumption for mode-A PDCCH monitoring [Issue-10]

Proposal 4 [Issue-10]: For mode-A UEIBR, keep the current wording in the MAC spec that UE considers the DRX as active time while a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH.

Reference

[1] R2-2506202	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	Vice Chairman (CATT)	Report